| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0119 |
| --- | --- |

| Title: | EE2-1.20m: Combination test of Test 1.17 and Test 1.18 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Kai Zhang, Li Zhang (Bytedance) Karam Naser, Philippe Bordes, Franck Galpin, Kevin Reuze, Antoine Robert (InterDigital) | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com karam.naser@interdigital.com |
| Source: | Bytedance Inc., InterDigital |  |  |

_____________________________

- Abstract

This contribution reports test results for EE2 Test 1.20m, which is the combination of Test 1.17 and Test 1.18. In Test 1.17, an intra template matching prediction (IntraTMP) fusion with intra prediction is proposed. In Test 1.18, an extension to CIIP to replace the inter-prediction part by IntraTMP is proposed. On top of ECM-8.0, simulation results of Test 1.20m are reported as below:

Test 1.20m test #1: Test 1.17 + Test 1.18a:

AI: {%, %, %; %, %};

RA: {%, %, %; %, %}.

Test 1.20m test #2: Test 1.17 + Test 1.18b:

AI: {%, %, %; %, %};

RA: {%, %, %; %, %}.

Introduction

Test 1.17 description

IntraTMP fusion with intra prediction was proposed in JVET-AC0170 [1] and established as Test 1.17 in JVET-AC EE2 [2]. In IntraTMP fusion with intra prediction, the prediction block is the weighted sum of two prediction signals generated by IntraTMP and intra prediction derived by TIMD. A CU flag is signalled to indicate whether the proposed method is used.

Test 1.18 description

CIIP extension with intra template matching was proposed in JVET-AC0201 [3] and established as Test 1.18 in JVET-AC EE2. In Test 1.18a, it is proposed to signal a new mode, named spatial CIIP, inside the intra prediction syntax. This mode uses the same functionality as CIIP, except that the inter part is replaced by IntraTMP prediction. That is, the intra part remains unchanged, as well as the computation of weights. In Test 1.18b, the modified search range as in EE2-1.14a and EE2-1.14b is applied on top of Test 1.18a.

Test 1.20m test #1 = Test 1.17 + Test 1.18a

Test 1.20m test #2 = Test 1.17 + Test 1.18b

Simulation results

The proposed combination is implemented on top of ECM-8.0 [4]. Simulations are performed following the common test conditions of EE2 [5]. The simulation results of Test 1.20m test #1 and Test 1.20m test #2 compared to ECM-8.0 are shown in Table 1 and Table 2.

Table 1: Test results of Test 1.17 + Test 1.18a

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.23% | -0.20% | -0.16% | 105% | 119% | -0.10% | -0.20% | 0.00% | 101% | 99% |
| Class E | -0.85% | -0.84% | -0.75% | 105% | 104% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.41% | -0.68% | -0.51% | 106% | 102% | -0.04% | -0.10% | -0.36% | 102% | 100% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 2: Test results of Test 1.17 + Test 1.18b

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.11% | -0.12% | -0.14% | 108% | 107% | -0.05% | -0.04% | 0.21% | 100% | 95% |
| Class E | -0.30% | -0.22% | -0.18% | 102% | 102% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.03% | -0.15% | -0.13% | 104% | 101% | -0.01% | 0.25% | -0.40% | 101% | 98% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, test results of Test 1.20m which is the combination of Test 1.17 and Test 1.18 are reported. It is recommended to adopt the combination method into ECM.

Reference

- Y. Wang, K. Zhang, and L. Zhang, “Non-EE2: Fuse intra template matching prediction with intra prediction,” Document of Joint Video Experts Team, JVET-AC0170, by teleconference, January 2023.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AC2024, by teleconference, January 2023.

- K. Naser, P. Bordes, F. Galpin, K. Reuze, and A. Robert, “AHG12: CIIP extension with Intra Template Matching,” Document of Joint Video Experts Team, JVET-AC0201, by teleconference, January 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).