| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0120-v1 |
| --- | --- |

| Title: | EE2-1.3: Local-Boosting Cross-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2-1.3 on Local-Boosting Cross-Component Prediction. Four subtests are investigated according to EE2 description:

Test 1.3a: MM-CCLM/MM-CCCM filtering with neighbouring samples.

Test 1.3b: Cross-component prediction usage is determined by a template cost.

Test 1.3c: Cross-component prediction signalling modification.

Test 1.3d: Test 1.3a + Test 1.3b + Test 1.3c

Simulation results based on ECM-8.0 are reported as below:

Test 1.3a: AI {-0.07%, -0.23%, -0.13%, 101%, 99%}; RA {-0.04%, -0.12%, 0.00%, 101%, 99%}

Test 1.3b: AI {-0.05%, -0.14%, -0.06%,	 101%, 98%}; RA {-0.04%, -0.15%, -0.07%, 99%, 99%}

Test 1.3c: AI {0.00%, 0.00%, -0.03%, 100%, 100%}; RA {-0.02%, -0.09%, 0.03%, 100%, 98%}

Test 1.3d: AI {-0.11%, -0.44%, -0.31%, 101%, 100%}; RA { -0.05%, -0.16%, 0.04%, 101%, 100%}

For further information, we tested a combination of Test 1.3a and Test 1.3b, noted as Test 1.3e, and simulation results of Test 1.3e are listed as follows:

Test 1.3e: AI {-0.11%, -0.39%, -0.27%, 101%, 100%}; RA {-0.04%, -0.13%, -0.10%, 101%, 100%}

# Introduction

Cross-component prediction (CCP) including CCLM, CCCM and their variants were adopted in ECM to exploit the cross-component correlation. Local neighbouring information can be utilized with CCP in the intra chroma fusion mode, where CCP and an angular prediction mode are weighted summed to generate the final prediction. In JVET-AC0175 [1], a Local-Boosting Cross-Component Prediction (LB-CCP) was proposed to utilize more local information to further improve the performance of CCP. The LB-CCP was included into EE2 for further investigation.

# Proposed methods

Methods of LB-CCP are proposed to boost CCP by taking more advantage from local information, including neighbouring samples and prediction samples. Three aspects are involved in LB-CCP.

Aspect #1: Prediction samples of MM-CCLM/MM-CCCM can be filtered with neighbouring samples. As shown in Fig.1, a 3×3 low-pass filter is applied to filter prediction samples generated by MM-CCLM/MM-CCCM. For a sample at a top/left boundary, the filtering window may involve neighbouring reconstructed samples. For inner samples, the filtering window only involves prediction samples, which may be padded. A flag is signaled to indicate whether filtering is applied or not for a block coded with MM-CCLM/MM-CCCM.

Aspect #2: Template costs are calculated to implicitly determine the usage of CCP. The template cost is derived by applying the candidate CCP on the template, as illustrated in Fig.2, and calculating the SAD between the prediction samples and reconstruction samples in the template. For CCCM/CCCM-L/CCCM-T modes, two template costs are calculated by using 6 lines or 2 lines of neighbouring samples as training samples, respectively. For MM-CCCM/MM-CCCM-L/MM-CCCM-T modes, two template costs are calculated by using neighbouring luma samples or luma samples collocated to the current chroma block to derive the mean value as the threshold to separate the two models, respectively. For intra chroma fusion mode, two template costs are calculated by fusing the angular chroma prediction with MM-CCLM or MM-CCCM, respectively, and the weighting values depend on the minimum template cost. In all the cases, the option with a lower template cost is selected as the final CCP method on the current block.

Aspect #3: The binarization of CCP modes is modified. Instead of signaling CCLM-L/CCLM-T/MM-CCLM-L/MM-CCLM-T (or CCCM-L/CCCM-T/MM-CCCM-L/MM-CCCM-T) with a truncated unary code, two syntax elements are coded. multi_mode_flag indicates whether multiple or single model is applied, and ccp_dir_flag indicates whether left or above direction is applied.

According to EE2 description, each of the three aspects is investigated individually. Specifically, Test 1.3a is for Aspect #1, Test 1.3b is for Aspect #2, and Test 1.3c is for Aspect #3. A combination test of all three aspects is listed as Test 1.3d. Moreover, a combination test of Test 1.3a and Test 1.3b is listed as Test 1.3e.

Fig. 1 The filter on samples of MM-CCLM/MM-CCCM.

Fig. 2 The template of a block.

# Experimental results

The proposed methods are implemented on top of ECM-8.0 [1] and tested following the common test conditions [2]. The simulation results of Test 1.3 a-e are summarized in Table 1-5.

Table 1: Coding performance of Test 1.3a

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.16% | 0.01% | 100% | 100% | -0.08% | -0.08% | 0.20% | 101% | 100% |
| Class A2 | -0.06% | -0.16% | -0.04% | 100% | 99% | -0.04% | -0.12% | 0.17% | 101% | 98% |
| Class B | -0.06% | -0.18% | -0.01% | 101% | 100% | 0.00% | -0.22% | -0.31% | 101% | 97% |
| Class C | -0.08% | -0.10% | -0.14% | 101% | 96% | -0.06% | -0.02% | 0.10% | 101% | 100% |
| Class E | -0.08% | -0.59% | -0.56% | 102% | 103% |  |  |  |  |  |
| Overall | -0.07% | -0.23% | -0.13% | 101% | 99% | -0.04% | -0.12% | 0.00% | 101% | 99% |
| Class D | -0.08% | -0.28% | -0.20% | 101% | 97% | -0.02% | 0.00% | 0.01% | 101% | 100% |
| Class F | -0.05% | -0.03% | 0.04% | 102% | 101% | 0.01% | -0.29% | -0.24% | 101% | 100% |
| Class TGM | -0.04% | -0.06% | 0.01% | 100% | 97% | 0.02% | 0.01% | 0.05% | 101% | 99% |

Table 2: Coding performance of Test 1.3b

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.10% | -0.12% | 100% | 97% | -0.06% | -0.03% | -0.15% | 99% | 100% |
| Class A2 | -0.04% | -0.04% | 0.00% | 100% | 98% | -0.03% | -0.07% | 0.03% | 99% | 98% |
| Class B | -0.05% | -0.17% | -0.04% | 102% | 100% | -0.03% | -0.35% | -0.18% | 99% | 97% |
| Class C | -0.04% | -0.05% | -0.09% | 101% | 98% | -0.04% | -0.06% | 0.04% | 100% | 100% |
| Class E | -0.03% | -0.34% | -0.06% | 101% | 100% |  |  |  |  |  |
| Overall | -0.05% | -0.14% | -0.06% | 101% | 98% | -0.04% | -0.15% | -0.07% | 99% | 99% |
| Class D | -0.05% | -0.27% | -0.26% | 100% | 98% | -0.01% | -0.03% | 0.08% | 100% | 99% |
| Class F | -0.03% | -0.17% | -0.16% | 101% | 103% | -0.04% | -0.21% | -0.13% | 100% | 100% |
| Class TGM | -0.02% | -0.09% | -0.04% | 100% | 101% | 0.00% | 0.01% | 0.04% | 100% | 100% |

Table 3: Coding performance of Test 1.3c

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.06% | 101% | 99% | -0.01% | 0.03% | -0.06% | 100% | 100% |
| Class A2 | -0.02% | 0.00% | -0.01% | 99% | 99% | -0.01% | -0.06% | 0.18% | 100% | 98% |
| Class B | 0.01% | -0.05% | -0.02% | 100% | 101% | -0.02% | -0.03% | 0.01% | 100% | 99% |
| Class C | -0.01% | 0.01% | 0.00% | 101% | 99% | -0.04% | -0.26% | 0.01% | 100% | 95% |
| Class E | 0.02% | 0.06% | -0.04% | 101% | 102% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.03% | 100% | 100% | -0.02% | -0.09% | 0.03% | 100% | 98% |
| Class D | -0.01% | 0.03% | 0.13% | 99% | 104% | -0.02% | -0.03% | -0.28% | 99% | 98% |
| Class F | -0.02% | -0.06% | 0.10% | 101% | 98% | -0.03% | -0.23% | -0.07% | 101% | 99% |
| Class TGM | -0.02% | 0.00% | 0.03% | 100% | 99% | 0.03% | 0.02% | 0.01% | 99% | 100% |

Table 4: Coding performance of Test 1.3d (combination of 1.3a, 1.3b, and 1.3c)

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.36% | -0.20% | 102% | 101% | -0.09% | -0.02% | 0.15% | 101% | 101% |
| Class A2 | -0.11% | -0.28% | -0.16% | 101% | 100% | -0.06% | -0.19% | 0.08% | 101% | 98% |
| Class B | -0.09% | -0.50% | -0.24% | 102% | 101% | 0.00% | -0.27% | -0.04% | 100% | 98% |
| Class C | -0.10% | -0.25% | -0.30% | 101% | 100% | -0.06% | -0.10% | 0.04% | 101% | 102% |
| Class E | -0.09% | -0.86% | -0.71% | 101% | 98% |  |  |  |  |  |
| Overall | -0.11% | -0.44% | -0.31% | 101% | 100% | -0.05% | -0.16% | 0.04% | 101% | 100% |
| Class D | -0.09% | -0.30% | -0.27% | 101% | 99% | -0.05% | -0.13% | -0.32% | 101% | 99% |
| Class F | -0.03% | -0.10% | 0.03% | 102% | 103% | -0.02% | -0.50% | -0.39% | 101% | 102% |
| Class TGM | -0.06% | -0.10% | -0.03% | 101% | 101% | -0.02% | -0.04% | -0.06% | 101% | 102% |

Table 5: Coding performance of Test 1.3e (combination of 1.3a and 1.3b)

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.32% | -0.20% | 101% | 100% | -0.10% | -0.09% | 0.16% | 100% | 101% |
| Class A2 | -0.10% | -0.20% | -0.14% | 103% | 101% | -0.02% | -0.11% | -0.17% | 100% | 99% |
| Class B | -0.09% | -0.41% | -0.19% | 101% | 101% | -0.01% | -0.12% | -0.25% | 102% | 100% |
| Class C | -0.11% | -0.19% | -0.20% | 102% | 100% | -0.06% | -0.16% | -0.06% | 100% | 101% |
| Class E | -0.12% | -0.90% | -0.70% | 100% | 100% |  |  |  |  |  |
| Overall | -0.11% | -0.39% | -0.27% | 101% | 100% | -0.04% | -0.13% | -0.10% | 101% | 100% |
| Class D | -0.11% | -0.20% | -0.19% | 103% | 98% | -0.08% | -0.02% | -0.02% | 100% | 99% |
| Class F | -0.10% | -0.21% | 0.04% | 101% | 102% | -0.05% | -0.24% | -0.20% | 100% | 99% |
| Class TGM | -0.06% | -0.14% | -0.08% | 101% | 100% | -0.01% | -0.09% | -0.13% | 100% | 100% |

# Conclusions

This contribution reports the simulation results of EE2-1.3 on LB-CCP. It is asserted that the proposed method contributes promising coding gains with negligible computational complexity. It is proposed to adopt the proposed method to the ECM.

# References

- Kai Zhang, Li Zhang, Zhipin Deng, “Non-EE2: Local-Boosting Cross-Component Prediction”, JVET-AC0175, Jan. 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).