| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0121-v1 |
| --- | --- |

| Title: | AHG9: Attenuation Map Information SEI for reducing energy consumption of displays |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Olivier LE MEUR, Claire-Hélène DEMARTY, Franck AUMONT, Edouard FRANCOIS, Laurent BLONDE, Erik REINHARD | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Following up on JVET-AC0122 [JVET-AC0122], this contribution reports a comparison of the proposal to use an Attenuation Map to reduce the energy consumption of images while displayed, with a linear scaling and an implementation of a clipping as described in informative annex B of Green metadata specification. Two different techniques to build attenuation maps are reported and compared. The comparison is reported in terms of objective quality metrics. It is asserted that the use of attenuation maps as built by the two compared techniques provides higher objective quality metrics than the approach described in Green metadata specification, and comparable PSNR values and higher SSIM values than linear scaling.

This contribution also states that the use of Attenuation Maps to reduce the energy consumption of images while displayed is consistent across different screen types and options.

Following the recommendations from JVET 29th meeting, this contribution also provides a technical solution to carry the attenuation map by reusing auxiliary pictures of type AUX_ALPHA. It is reported that auxiliary pictures of type AUX_ALPHA with only luma component are currently supported, limiting the use of Attenuation Map to one single component.

Finally, it is reported that the new metadata for Attenuation Maps will be proposed to be defined in an amendment of the Green metadata specification.

# Introduction

Climate change is a fact and every industry should contribute to mitigating its effects by moving to clean sources of energy, and by reducing the amount of energy used. This includes the entertainment and media industry. Each step, from production, encoding, transmission, to decoding and rendering on television screens contributes to the overall energy consumption, with televisions taking as much as 50% of the total energy use [Kamiya, 2020; Carnstone, 2022]. The reduction of energy consumption of television screens is therefore a priority.

This contribution is a follow-up of JVET-AC0122 [JVET-AC0122], which argued for the signaling of metadata for assisting with attenuating of the decoded video at the receiver side and enabling the use case by proposing a new type of auxiliary pictures and associated Attenuation Map Information (AMI) SEI message.

During JVET 29th meeting, the group recommended to:

- Compare a pixel-based method as proposed in the contribution with the approach in Green metadata display adaptation.

- Assess the influence of different screens and screen options on the method.

- Study whether the existing auxiliary picture dedicated to alpha blending may be reused for the proposed use case.

Following this recommendation, this contribution provides the outcome of these three points:

- A comparison of two pixel-wise methods with a linear scaling and an implementation similar to the approach described in Annex B of the Green metadata specification shows that the pixel-wise methods provide higher objective quality scores than the method in Green metadata specification and comparable PSNR values and higher SSIM values than linear scaling.

- A comparison of different screen settings states that these settings do not influence the pixel-wise methods and are not influenced by them.

- The use of alpha plane auxiliary pictures to carry Attenuation Maps is possible within the specific case alpha_channel_use_idc equal to 3, without syntax and semantics modifications in the ACI SEI message, provided that the proposed metadata dedicated to Attenuation Maps are also sent independently. However, a drawback was identified in the limitation of the Alpha Maps to having information in their luma component only. This implies a limitation in the framework proposed in [JVET-AC0122]; the Attenuation Map should be limited to one single component. The impact over the semantics of the related metadata proposed in [JVET-AC0122] is presented.

# Motivation

As explained in the introduction, one goal of the contribution is to further assess the benefit of the proposal in JVET-AC0122 by comparison with other techniques and with different TV settings. This will be done in sections 3 and 4.

Beforehand, the general motivation for proposing a new framework in order to leverage a pixel-wise information of energy reduction, called Attenuation Map is reminded. Such map would be computed by energy-aware image algorithms.

An illustration of such a framework in the global video chain is proposed in Figure 1. Pixel-wise energy-aware image algorithms generating pixel-based information of energy reduction would therefore be mainly positioned prior to the encoding in the analysis step. They would also benefit from several well-suited characteristics: by nature, they are more flexible as changes applied on content can be spatially localized; perceptual properties of the human vision can be exploited to control the content modification at the pixel level; this will allow for a finer management of the trade-off between energy reduction and quality of experience in the resulting modified images. This is especially interesting when one need to respect the content creator intent.

The proposed framework differs with the one in the Green metadata specification in the sense that it proposes the signaling of new metadata for enabling the attenuation of each sample value (of each color component) of a decoded video at the receiver side.

To this end, this contribution proposes the transmission of related metadata as well as an attenuation map, carried through auxiliary pictures. The attenuation map could be the output of the pre-encoding analysis step (as shown in Figure 1). It will provide a pixel-wise information of the modification to apply to any pixel of the corresponding input image.

The accompanying metadata will help define how the attenuation map should be applied on the image to be displayed, the simplest way being a pixel-wise subtraction of the attenuation map. The metadata also convey other useful information, i.e., on which components (luminance, R, G, B, etc.) the attenuation map is to be applied, indication of any rescaling algorithm to apply to the attenuation map before usage, etc. Together, these enable an intelligent spatially varying reduction of pixel values so as to reduce the energy consumption, while maintaining or controlling the quality of experience for the end user. Furthermore, it is also possible to convey information in the metadata on how to use the attenuation map to derive a backlit scaling that could serve to reduce the energy consumption of displays.

Figure 1: Global framework for pixel-wise content-adaptive energy reduction.

The Attenuation Maps are proposed to be transmitted as auxiliary pictures of type AUX_ALPHA, as recommended during JVET 29th meeting. Although feasible, this implies a limitation of the proposed framework which will be detailed in section 5.

# Comparison between existing methods

In this section, the performance of the proposed framework is assessed when considering different methods for reducing the energy consumption of displays and compared to existing methods. Those methods are grouped in two categories. The first category groups two global methods (without considering the local content) whereas the second category groups two methods which generate pixel-wise attenuation map as illustrated in Figure 1.

The four methods are listed below and briefly described in the following subsections:

- Global methods:

  - Linear scaling of the luminance channel

  - Clipping the highlights to simulate the current Green Metadata solution, called Green MPEG Like in the following

- Pixel-wise attenuation map methods:

  - Third party implementation of the R-ACE network [Nugroho, 2022]

  - Light weight deep network inspired from R-ACE network, called DeepPVR [Le Meur et al., 2023a]

## Linear scaling of the luminance component

A first global method to reduce the amount of emitted light consists in scaling down the luminance channel in a linear manner. Such a linear scaling does not depend on the content nor on the spatial neighborhood. This is illustrated by Figure 2 (left hand side).

Figure 2: Linear scaling method (left) and Green Metadata like method (right).

## Display adaptation based on Green metadata standard

The second global method used in the comparison corresponds to an implementation of the behavior of the Green metadata proposition. As illustrated in Figure 2 (right hand side), it consists in a clipping of the highlights as proposed in [Fernandes et al., 2018] (see the Figure 3 of the paper for more details). As for the linear scaling, this method does not consider the local content.

## R-ACE network

In 2019, Shin et al. [Shin, 2019] propose a method in which the luminance of the input image is first uniformly reduced by a scaling factor deduced from the power saving rate R. As this kind of scaling degrades the image quality, authors introduce a deep network, called ACE (Adaptive Contrast Enhancement), designed to improve the image quality while keeping the desired power saving rate. An improvement has been brought by [Nugroho, 2022] introducing the concept of a dimming map. As illustrated by Figure 3, the network computes a dimming map which is added back to the luminance channel. The backbone of the proposed network is similar to the one proposed in [Shin, 2019].

Figure 3: Architecture of the R-ACE network as proposed in [Nugroho, 2022].

In this contribution, a proprietary implementation of [Nugroho, 2022] was used and retrained from scratch. More details are given in [Le Meur et al., 2023b].

## Proprietary method

The R-ACE network has a significant complexity with more than 40.000 of trainable parameters. This method has been recently improved in [Le Meur et al., 2023a] by revisiting the architecture of the network. The most important differences are related to the number of input channels, to the CAN (Context Aggregation Network) that has been replaced by ATrous spatial pyramid pooling and by the use of a channel and spatial attention layers. The total number of trainable parameters is 4,832, which is much less than the 41,147 trainable parameters of [Nugroho, 2022].

## Results

This section reports the comparison between the different methods described above, in terms of PSNR and SSIM, as functions of power consumption.

### Test protocol

The set of sequences used for the test is listed in Table 1. For each sequence, 5 frames were evenly extracted.

The power consumption of each original and processed image is obtained thanks to a power meter. The processed images are obtained by considering 4 different rates of power reduction: 10%, 20%, 40% and 60%. Their corresponding objective metric values are computed with the PSNR and the SSIM.

Sequences

A_PeopleOnStreet_2560x1600_30_crop

A_Traffic_2560x1600_30_crop

B_BasketballDrive_1920x1080_50

B_BQTerrace_1920x1080_60

B_Cactus_1920x1080_50

B_Kimono1_1920x1080_24

B_ParkScene_1920x1080_24

B_Tennis_1920x1080_24

C_BQMall_832x480_60

C_Flowervase_832x480_30

C_Keiba_832x480_30

C_Mobisode2_832x480_30

C_PartyScene_832x480_50

C_RaceHorses_832x480_30

D_BasketballPass_416x240_50

E_Johnny_1280x720_60

E_Vidyo4_1280x720_60

Table 1: List of sequences

### PSNR and SSIM scores

Figure 4 and Figure 5 present the PSNR and SSIM scores in function of the power consumption. It is stated that the two pixel-wise methods and linear scaling reach similar PSNR values for all 4 reduction rates, whereas the pixel-wise methods reach significantly higher SSIM values than global methods. In all cases, the approach described in the Green metadata specification is outperformed by pixel-wise methods.

Figure 4: Power consumption vs PSNR. The control points represent the rates of power reduction: 10%, 20%, 40% and 60%. For Green MPEG-like method, the control points represent the value used for thresholding the image.

Figure 5: Power consumption vs SSIM. The control points represent the rates of power reduction: 10%, 20%, 40% and 60%. For Green MPEG-like method, the control points represent the value used for thresholding the image.

### Visual comparison

Figure 6 presents a visual comparison between three methods: Linear scaling, Green metadata like and DeepPVR method.

The following observations are made:

- The Green Metadata like method has the lowest visual quality. The visual quality quickly drops down even for small power reduction;

- The DeepPVR method provides sharper and less blurry results. Figure 7 and Figure 8 illustrate the visual quality difference between DeepPVR method and the linear scaling method, on a small part of frame content. The linear scaling method is visually less sharp, blurry and has less fine details.

Figure 6: Visual quality of the different methods. The power reduction as well as PSNR and SSIM values are provided.

Figure 7: Visual comparison between DeepPVR and Linear Scaling (Power=103W).

Figure 8: Visual comparison between DeepPVR and Linear Scaling (Power=99W).

# Influence of external parameters (TV settings)

This section reports the behavior of the use of an Attenuation Map as computed by using the RACE method described in section 3.3  for different sets of screen options and for two different screens, as listed in Table 2.

| Mode | Description |
| --- | --- |
| nominal | SONY KD-55AF9 (55’’, HD), all possible options set to OFF, picture mode STANDARD, maximal brightness (=50) |
| brightness30 | SONY KD-55AF9 (55’’, HD), all possible options set to OFF, picture mode STANDARD, brightness=30 |
| brightness15 | SONY KD-55AF9 (55’’, HD), all possible options set to OFF, picture mode STANDARD, brightness=15 |
| cinema | SONY KD-55AF9 (55’’, HD), all possible options set to OFF, picture mode CINEMA, maximal brightness (=50) |
| game | SONY KD-55AF9 (55’’, HD), all possible options set to OFF, picture mode GAME, maximal brightness (=50) |
| LG42C2 | LG 42C2 (42’’, 4K), all possible options set to OFF, picture mode STANDARD, maximal brightness, no ambient light adaptation |
| LG42C2-ambientlight | LG 42C2 (42’’, 4K), all possible options set to OFF, picture mode STANDARD, maximal brightness, ambient light adaptation |

Table 2: Modes used for the comparison with different sets of screen options

Again, average power consumptions and their corresponding metric values were computed over the set of sequences as listed in Table 1 and for 4 different rates of power reduction: 10%, 20%, 40% and 60%. For each sequence, 5 frames were evenly extracted to compute the average power consumption and metrics.

Figure 9: Power consumption vs PSNR. The control points represent the rates of power reduction: 10%, 20%, 40% and 60%.

Figure 10: Power consumption vs SSIM. The control points represent the rates of power reduction: 10%, 20%, 40% and 60%.

From the above figures, it is stated that the application of energy reduction process thanks to the use of an Attenuation Map as computed through the RACE method described in section 3.3, does not influence, nor is influenced by the set of chosen options, or screens. The amount of reduction is consistent whatever the TV setting and for both TV screens.

Some general observations:

- Increasing the brightness of the TV increases, as expected, the energy consumption;

- The most consuming mode is the cinema mode (more than 100W in average).

In conclusion, the use of Attenuation Maps is relevant for reducing the energy consumption of displays and independent of the TV settings and TV screens.

# Attenuation map transmission through auxiliary picture of type AUX_ALPHA

## Approach

In order to leverage the auxiliary picture of type AUX_ALPHA to carry the Attenuation Map, it is proposed to use the ACI SEI message without any change in syntax and semantics, but to send it with predefined default values, in particular with alpha_channel_use_idc equal to 3.

Because auxiliary pictures of type AUX_ALPHA are currently limited to luma component (as stated in section 5.2), the current proposal implies that the Attenuation Map will also contains only one component. This is a limitation to the original proposed framework in JVET-AC0122. The originally proposed metadata related to the use of the Attenuation Map need to be modified accordingly.

In this proposal, in addition, it is still proposed to prepare and send the metadata related to the use of the Attenuation Map in a dedicated manner. This will be proposed to be defined in an amendment of the Green metadata specification.

## Limitation of the use of auxiliary picture of type AUX_ALPHA

In document ITU-T H.265 (Annex F, section F.7.4.3.1.1 Video parameter set extension semantics), it is stated that:

It is a requirement of bitstream conformance that when AuxId[ lId ] is equal to AUX_ALPHA or AUX_DEPTH, either of the following applies:

- chroma_format_idc is equal to 0 in the active SPS for the layer with nuh_layer_id equal to lId.

- The value of all decoded chroma samples is equal to 1 << ( BitDepthC − 1 ) in all pictures that have nuh_layer_id equal to lId and for which this VPS RBSP is the active VPS RBSP.

In any case, auxiliary pictures of type AUX_ALPHA carry information on their luma component only. This limits the use of Attenuation Maps with more than one channel component, i.e., the cases where different attenuations stored as channel components of the Attenuation Maps are applied individually on the channel components of the decoded primary picture.

From this statement, two options are available:

- Use auxiliary pictures of type AUX_ALPHA to carry Attenuation Maps as proposed in the current contribution, and discard some of the use cases of the Attenuation Maps. The modified semantics corresponding to the limited use of Attenuation Maps is described in section 5.4.

- Define and use a new type of auxiliary pictures with three channel components to support all proposed use cases for the use of the Attenuation Maps as originally proposed in JVET-AC0122.

## ACI SEI message default values

In documents ISO/IEC 23002-3, ITU-T H.265 and ITU-T H.274, it is stated that:

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture should be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture should not be multiplied by the interpretation sample values of the decoded auxiliary picture in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the auxiliary picture is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

For this reason, it is proposed to set alpha_channel_use_idc to 3, to save the decoders to decode and interpret any other metadata in the ACI SEI message. Other metadata values are proposed to be set to the following default values (although such values should only serve as an example implementation):

- alpha_channel_bit_depth_minus8 = 0

- alpha_transparent_value = alpha_opaque_value = 0

- alpha_channel_incr_flag = 0

- alpha_channel_clip_flag = 0

## Modification of the AMI metadata semantics

Due to the limitation of the auxiliary picture for alpha blending to luma components, it is necessary to modify the metadata as originally proposed in JVET-AC0122. These will be further proposed to be defined in an amendment of the Green metadata specification.

Modifications in the metadata semantics are highlighted in yellow in the following.

Note: In a preferred mode, the Attenuation Map shall be applied on the sample values of the decoded primary picture(s), i.e., just after the decoding process.

The Attenuation Map Information (AMI) SEI message provides information about the interpretation of the Attenuation Map sample values coded in auxiliary pictures of type AUX_ALPHA (in the following picAMI) and the post-processing intended to be applied to the one or more associated primary pictures of the CVS.

Note: the association of auxiliary pictures to primary pictures are specified in the SDI SEI message (see ISO/IEC 23002-7).

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i, no picture in the CVS shall be associated with a AMI SEI message.

When an access unit (AU) contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and a AMI SEI message, the SDI SEI message shall precede the AMI SEI message in decoding order.

When an AU contains a picAMI in a layer, with nuh_layer_id equal to nuhLayerIdAMI, that is indicated as an Alpha Map auxiliary layer by an SDI SEI message, the Attenuation Map sample values of picAMI persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdAMI is output.

–	A CLVS containing the auxiliary picture picAMI ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdAMI ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the AMI SEI message applies.

ami_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous Attenuation Map Information SEI message in output order. ami_cancel_flag equal to 0 indicates that Attenuation Map Information parameters follow.

ami_map_number specifies the number of auxiliary pictures of type AUX_ALPHA in the CVS.

ami_display_model is a bit field mask which indicates the display models on which the Attenuation Map sample values of the auxiliary picture picAMI may be used.

Table 3 - Interpretation of the bits of ami_display_model

| Bit number | Display model |
| --- | --- |
| 0 | Backlit pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

For example, ami_display_model=11 means the Attenuation Map Information can be used for both “Backlit” and “Emissive” display models.

ami_map_approximation_model specifies the model used to extrapolate a set of received Attenuation Map sample values(s) from a set of decoded auxiliary picture(s) with individual energy reduction rate(s) to another set of Attenuation Map sample values with a different energy reduction rate.

ami_map_approximation_model equal to 0 specifies that a linear scaling of the Attenuation Map sample values of the provided auxiliary picture given its respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate. In case several auxiliary pictures picAMI are provided, the auxiliary picture with the lowest ami_energy_reduction_rate is used for the linear scaling. This is the preferred type.

ami_map_approximation_model equal to 1 specifies that a bilinear interpolation between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 2 specifies that an interpolation of type Lanczos between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 3 specifies that an interpolation of type bicubic between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 4 specifies that a proprietary user defined process should be used to infer corresponding Attenuation Map sample values for another energy reduction rate from the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate.

Table 4 - Interpretation of ami_map_approximation_model

| ami_map_approximation_model | Attenuation Map interpolation process |
| --- | --- |
| 0 | Linear scaling |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | Bicubic interpolation |
| 4 | User defined |
| 5..15 | Reserved for future uses |

ami_global_flag equal to 0 indicates that ami_attenuation_use_idc[ i ], ami_attenuation_comp_idc[ i ], ami_preprocessing_flag[ i ], ami_preprocessing_type_idc[ i ], ami_preprocessing_scale_idc[ i ], ami_backlight_scaling_idc[ i ] for i=0.. ami_map_number, shall be present. ami_global_flag equal to 1 indicates that only ami_attenuation_use_idc[ 0 ], ami_attenuation_comp_idc[ 0 ], ami_preprocessing_flag[ 0 ], ami_preprocessing_type_idc[ 0 ], ami_preprocessing_scale_idc[ 0 ], ami_backlight_scaling_idc[ 0 ] shall be present.

ami_attenuation_use_idc[ i ] specifies the use of the Attenuation Map sample values of the decoded auxiliary picture of index i.

ami_attenuation_use_idc[ i ] equal to 0 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be subtracted from one or more associated primary picture decoded sample(s) before displayed on screen. This is the preferred type.

ami_attenuation_use_idc[ i ] equal to 1 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be multiplied by one or more associated primary picture decoded sample(s) before displayed on screen.

ami_attenuation_use_idc[ i ] equal to 2 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be used according to a proprietary user defined process to modify the one or more associated primary picture decoded sample(s) before displayed on screen.

Table 5 - Interpretation of ami_attenuation_use_idc[ i ]

| ami_attenuation_use_idc[ i ] | Process to apply on associated primary picture decoded samples |
| --- | --- |
| 0 | Subtraction |
| 1 | Multiplication |
| 2 | User defined |
| 3..15 | Reserved for future uses |

ami_attenuation_comp_idc[ i ] specifies on which colour component(s) of the associated primary picture(s) decoded samples the decoded auxiliary picture of type AUX_ALPHA of index i should be applied using the process defined by ami_attenuation_use_idc[ i ]. It also specifies how many components the decoded auxiliary picture of index i should contain.

ami_attenuation_comp_idc[ i ] equal to 0 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the luma component of the associated primary picture(s) decoded samples. This is the preferred type.

ami_attenuation_comp_idc[ i ] equal to 1 specifies that the decoded auxiliary picture of type AUX_ALPHA of index i contains two components and that the first component should be applied to the luma component of the associated primary picture(s) decoded samples and the second component should be applied to both chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 1 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the luma component and the chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 2 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the RGB components (after YUV to RGB conversion) of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 4 specifies that the decoded auxiliary picture of type AUX_ALPHA of index i contains three components and that these components should be applied respectively to the luma and chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 5 specifies that the decoded auxiliary picture of type AUX_ALPHA of index i contains three components and that these components should be applied respectively to the RGB components (after YUV to RGB conversion) of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 3 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the first component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 4 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the second component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 5 specifies that the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i should be applied to the third component of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 6 specifies that the mapping between the luma component of the decoded auxiliary picture of type AUX_ALPHA of index i and the components of which to apply the decoded auxiliary picture of type AUX_ALPHA of index i corresponds to some proprietary user-defined process.

Table 6 - Interpretation of ami_attenuation_comp_idc[ i ]

| ami_attenuation_comp_idc[ i ] | Mapping between components of the Attenuation Map and primary picture components on which to apply the Attenuation Map |
| --- | --- |
| 0 | Luma component in the picAMI applied to the luma component (Y=>Y) of the associated primary picture |
| 1 | Two components in the picAMI applied as: - First component to luma component - Second component to the chroma components of the associated primary picture |
| 1 | Luma component in the picAMI applied to the luma and chroma components of the associated primary picture |
| 2 | Luma component in the picAMI applied to the three RGB components of the associated primary picture |
| 3 | Luma component in the picAMI applied to the first component of the associated primary picture |
| 4 | Luma component in the picAMI applied to the second component of the associated primary picture |
| 5 | Luma component in the picAMI applied to the third component of the associated primary picture |
| 4 | Three components in the picAMI applied to resp. luma and chroma components of the associated primary picture |
| 5 | Three components in the picAMI applied to resp. the RGB components of the associated primary picture |
| 6 | User defined |
| 7..15 | Reserved for future uses |

ami_backlight_scaling_idc[ i ] specifies the process to compute the scaling factor of the backlight of transmissive pixel displays, derived from the Attenuation Map sample values of the decoded auxiliary picture of index i.

ami_backlight_scaling_idc[ i ] equal to 0 specifies that the scaling to apply to the backlight of the display is computed as the ratio between the maximal values of the associated primary picture decoded samples after and before applying the Attenuation Map sample values of the decoded auxiliary picture of index i. The associated primary picture decoded sample(s) on which the Attenuation Map sample values of the decoded auxiliary picture of index I are applied are further rescaled to their maximal value before the application of the Attenuation Map sample values of the decoded auxiliary picture of index i. This is the preferred type.

ami_backlight_scaling_idc[ i ] equal to 1 specifies that the scaling to apply to the backlight of the display is determined according to a proprietary user defined process derived from the Attenuation Map sample values of the decoded auxiliary picture of index i.

Table 7 - Interpretation of ami_backlight_scaling_idc[ i ]

| ami_backlight_scaling_idc[ i ] | Backlight scaling processing type |
| --- | --- |
| 0 | Scaling by the ratio between maximum values before and after the use of the Attenuation Map |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_preprocessing_flag[ i ] specifies whether some pre-upsampling is to be used on the Attenuation Map sample values of the decoded auxiliary picture of index i. In that case it is supposed that the auxiliary coded picture(s) and the primary coded picture have different sizes.

ami_preprocessing_type_idc[ i ] specifies the recommended type of the interpolation (e.g., bicubic) used to resample the Attenuation Map sample values of the decoded auxiliary picture of index i at the same resolution as the associated decoded picture.

ami_preprocessing_type_idc[ i ] equal to 0 specifies that an interpolation of type bicubic between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_type_idc[ i ] equal to 1 specifies that a bilinear interpolation between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 2 specifies that an interpolation of type Lanczos between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 3 specifies that a proprietary user defined process should be used to pre-upsample the Attenuation Map sample values of the provided auxiliary picture of index i to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

Table 8 - Interpretation of ami_preprocessing_type_idc[ i ]

| ami_preprocessing_type_idc[ i ] | Attenuation Map preprocessing type |
| --- | --- |
| 0 | Bicubic interpolation |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | User defined |
| 4..15 | Reserved for future uses |

ami_preprocessing_scale_idc[ i ] specifies which scaling should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture.

ami_preprocessing_scale_idc[ i ] equal 0 specifies that a scaling of 1255 should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_scale_idc[ i ] equal 1 specifies that a proprietary user defined scaling should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture.

Table 9 - Interpretation of ami_preprocessing_scale_idc[ i ]

| ami_preprocessing_scale_idc[ i ] | Attenuation Map scaling preprocess |
| --- | --- |
| 0 | Scaling of 1255 |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_layer_id[ i ] specifies the identifier of the decoded layer for the Attenuation Map of index i.

ami_ols_number[ i ] specifies the number of Output Layer Sets to which the Attenuation Map of index i belongs.

ami_ols_id[ i ][ j ] specifies the identifier of the Output Layer Set of index j for the Attenuation Map of index i. This identifier shall be used to select the OLS to output both the primary decoded picture and the Attenuation Map of index i.

ami_energy_reduction_rate[ i ] indicates the expected energy saving rate when the video is displayed after applying the Attenuation Map sample values of the decoded auxiliary picture of index i on the sample values of the decoded picture.

# Conclusion

This contribution reports that the use of pixel-wise Attenuation Maps to reduce the consumption of images while displayed on screen produces higher quality metrics than the Green Metadata like implementation.  It also provides comparable PSNR values and higher SSIM values than linear scaling.

This contribution also reports that the use of Attenuation Maps is not influenced and does not influence the set of screen options and is consistent over two different screens.

A technical solution is also proposed to carry the Attenuation Maps through alpha planes as provided by auxiliary pictures of type AUX_ALPHA. It is reported however that this solution limits the proposed framework to Attenuation Maps with one luma component only.

The new metadata related to the use of the Attenuation Maps will be proposed to be specified in a new amendment of the Green metadata specification.

# Ackowledgements

This work has been achieved in the context of the project 3EMS-2 funded by the “Region Bretagne”, Rennes Métropole, and “Images et Réseaux”.

# References

[Carnstone, 2022] Carbon Emissions of Streaming and Digital Terrestrial Television, https://www.ofcom.org.uk/research-and-data/technology/general/carbon-emissions-of-streaming-and-digital-terrestrial-television

[JVET-AC0122] AHG9: Attenuation Map Information SEI for reducing energy consumption of displays https://jvet-experts.org/doc_end_user/current_document.php?id=12324

[Fernandes et al., 2018] Fernandes, Felix C., et al. "Mobile display power reduction for video using standardized metadata." IEEE Transactions on Mobile Computing 18.1 (2018): 165-178.

[Kamiya, 2020] Kamiya, George. "The carbon footprint of streaming video: fact-checking the headlines." (2020).

[Le Meur et al., 2023a] Le Meur et al., Deep-learning-based energy-aware images, submitted to ICIP’2023.

[Le Meur et al., 2023b] Le Meur et al., Energy-aware images: Quality of Experience vs Energy Reduction, MHV, 2023.

[Nugroho, 2022] Nugroho, Kuntoro Adi, and Shanq-Jang Ruan. "R-ACE Network for OLED Image Power Saving." 2022 IEEE 4th Global Conference on Life Sciences and Technologies (LifeTech). IEEE, 2022.

[Shin, 2019] Shin, Yong-Goo, et al. "Unsupervised deep contrast enhancement with power constraint for oled displays." IEEE Transactions on Image Processing 29 (2019): 2834-2844.

[Yu, 2015] Yu, Fisher, and Vladlen Koltun. "Multi-scale context aggregation by dilated convolutions." arXiv preprint arXiv:1511.07122 (2015).

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).