| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0122-v1 |
| --- | --- |

| Title: | [AHG8] Configuration changes for random access and low delay |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Rickard Sjöberg, Jacob Ström | Email: | christopher.hollmann@ericsson.com rickard.sjoberg@ericsson.com jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution results for modified configurations for random access and low delay for the area of optimizations for machine analysis of coded video content are described. The results state that increasing the QP offset by 5 for certain temporal layers can give a BD-rate result of −7.3%/−7.0% (SFU-HW/TVD) for random access and −9.6%/−13.3% for low delay. It is proposed to include the detailed description in the draft for the technical report.

# Introduction

At its 29th meeting in January 2023 JVET continued its study into the area of optimizations for encoders and receiving systems for machine analysis of coded video content and produced a set of common test conditions (CTC) [1] and a first draft for a technical report [2]. Compared to the CTC used in the Call for Proposals for Video Coding for Machines [3], the new CTC in JVET includes new configurations such as low delay and all intra, and a new pre-processing for the Tencent Video Dataset (TVD) [4].

This contribution is a continuation of JVET-AB0275 [5], in which we provided results including a modified encoder configuration for random access, where the QP offset for several temporal layers was increased. An overview of how pictures are placed in temporal layers and the respective temporal layer IDs (TID) in the default random access configuration is shown in Figure 1.

Figure 1. Hierarchical structure of pictures in the random access configuration

Similarly, the default low delay configuration also applies a hierarchical picture structure, as is visualized in Figure 2.

Figure 2. Hierarchical structure of pictures in the low delay configuration

# Method and results

As we already showed in JVET-AB0275, it is beneficial to increase the QP offset for pictures on higher temporal layers in random access. In this contribution we are showing that adding a delta of 5 to the QP offsets for temporal layers 1 and 2 for the low delay configuration gives a similar positive effect on the BD-rate results. The offsets we are using compared to the offsets used in the CTC are listed in Table 1.

Table 1. QP offsets used for default configuration and the proposed method

| Temporal layer | RA CTC | RA our test (JVET-AB0275) | LD CTC | LD our test (new) |
| --- | --- | --- | --- | --- |
| 0 | −1 | −1 | 1 | 1 |
| 1 | 0 | 0 | 4 | 9 |
| 2 | 0 | 5 | 5 | 10 |
| 3 | 3 | 8 | -- | -- |
| 4 | 5 | 10 | -- | -- |
| 5 | 6 | 11 | -- | -- |

Compared to using the CTC configuration, the results are listed in Table 2. For some sequences the RD-curve is non-monotonic, i.e., the performance at a lower bitrate is higher than at a higher bitrate. In these cases, the Pareto-mAP (for SFU-HW) or Pareto-MOTA (for TVD) is reported. Detailed results can be found in the Excel document that is attached to this contribution.

The VVC Test Model (VTM) version 12.0 [6] was used both for the anchor and for our test without any changes.

Table 2. BD-rate results for changing the QP offsets in the configuration

| Dataset | Task | Random access | Low delay |
| --- | --- | --- | --- |
| SFU-HW | Object detection | −7.34% | −9.57% |
| TVD | Object tracking | −6.96% | −13.32% |
| Average | -- | −7.15% | −11.45% |

# Conclusion

In this contribution we have reported BD-rate results for changing QP offsets for temporal layers compared to the default CTC configuration. For the low delay configuration, increasing the QP offsets by 5 for temporal layers 1 and higher grants a BD-rate result of −11.5%. For the random access configuration, increasing the QP offsets by 5 for temporal layers 2 and higher reportedly gives a BD-rate of −7.2%.

It is proposed to refine the description in the draft for the technical report in [2] regarding the QP changes on temporal layers in section 7.2 as detailed in the attached version of the technical report.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | S. Liu and C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," JVET-AC2031, Teleconference, January 2023. |
| --- | --- |
| [2] | J. Chen, C. Hollmann and S. Liu, "Optimization of encoders and receiving systems for machine analysis of coded video content (draft 1)," JVET-AC2030, Teleconference, Janaury 2023. |
| [3] | ISO/IEC JTC 1/SC 29/WG 2, "Call for Proposals for Video Coding for Machines," N00220, Online, July 2022. |
| [4] | X. Xu, S. Liu, R. Yao, L. Wang, S. Tian, D. Wu, Y. Hu, J. Li, J. Xia, W. Qi, J. Zhang and J. Wen, "A video dataset for training in neural network based video coding," JVET-U0116, Online, January 2021. |
| [5] | C. Hollmann, J. Ström, P. Wennersten, L. Litwic, R. Sjöberg and M. Damghanian, "Information about Ericsson’s response to the CfP for Video Coding for Machines," JVET-AB0275, Mainz, DE, October 2022. |
| [6] | ISO/IEC JTC 1/SC 29/WG 5, "VVC Reference Model (VTM)," https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git. |