| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0125-v1 |
| --- | --- |

| Title: | EE2-1.12: Intra TMP with sub-pel precision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2 Test 1.12. In Test 1.12, sub-pel precision is supported in Intra TMP. Specifically, three precisions (1/2-pel, 1/4-pel and 3/4-pel) in eight directions around the integer-pel position obtained by template matching are further enabled to Intra TMP. The eight directions are further sorted by template cost for each CU and only the first four direction can be used. The selected precision and direction are signaled in CU level. Four-tap DCT-IF interpolation filters are used for sub-pel interpolation. It is reported that on top of ECM-8.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are {-0.19%, -0.23%, -0.22%, 100%, 101%} in AI and {-0.08%, -0.07%, -0.04%, 100%, 100%} in RA.

Introduction

Intra template matching prediction (Intra TMP) is a newly added intra prediction mode in ECM [1]. Intra TMP generates the predicted values of the current block by copying the reconstructed values of the matching block in the current frame, whose position is determined by template matching at both the encoder side and the decoder side. The template matching is performed based on the L-shaped template of the current block and SAD is used as a cost function. However, Intra TMP only supports integer-pel precision.

In order to improve the coding efficiency of Intra TMP, it is proposed to support sub-pel precision in Intra TMP in the last JVET meeting [2], and studied in EE2 Test 1.12 [3]. This contribution reports the simulation results of EE2 Test 1.12.

Proposed methods

In Test 1.12, it is proposed to enable sub-pel precision in Intra TMP. Three sub-pel precisions, including half-pel, quarter-pel and three quarter-pel, with eight directions around the integer-pel position are supported in Test 1.12 as shown in Figure 1. Furthermore, the eight directions are sorted by template cost with half-pel precision for each CU, and only the first four directions can be used for each sub-pel precision.

If Intra TMP mode is selected for the current block, a precision index is signaled to indicate which of the integer-pel and three sub-pel precisions is used as shown in Table 1. If one of the three sub-pel precisions is used, a direction index is signaled to indicate which of the four directions is used as shown in Table 2.

Four-tap DCT-IF interpolation filters in ECM are used for sub-pel interpolation in Intra TMP.

Figure 1. The adjacent half-pel positions in 8 directions.

Table 1. Binarization of the precision idx

| Precision idx | 0 (int-pel) | 1 (1/2-pel) | 2 (1/4-pel) | 3 (3/4-pel) |
| --- | --- | --- | --- | --- |
| Binarization | 0 | 10 | 110 | 111 |

Table 2. Binarization of the direction idx

| Direction idx | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Binarization | 00 | 01 | 10 | 11 |

Simulation results

The proposed method is implemented on top of ECM-8.0 and is tested under CTC for enhanced compression tool testing [4] with all intra and random access configurations. The simulation result of Test 1.12 is shown in Table 3.

Table 3. Test 1.12: Intra TMP with sub-pel precision

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.06% | 100% | 100% |
| Class A2 | -0.26% | -0.27% | -0.28% | 100% | 101% |
| Class B | -0.20% | -0.18% | -0.22% | 101% | 102% |
| Class C | -0.06% | -0.11% | -0.13% | 100% | 100% |
| Class E | -0.48% | -0.54% | -0.46% | 100% | 99% |
| Overall | -0.19% | -0.23% | -0.22% | 100% | 101% |
| Class D | -0.04% | -0.24% | -0.04% | 100% | 99% |
| Class F | 0.02% | -0.14% | 0.02% | 100% | 101% |
| Class TGM | -0.08% | -0.04% | -0.04% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.06% | -0.03% | 101% | 101% |
| Class A2 | -0.16% | -0.13% | -0.04% | 102% | 102% |
| Class B | -0.09% | -0.14% | -0.07% | 98% | 99% |
| Class C | -0.03% | -0.05% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.07% | -0.04% | 100% | 100% |
| Class D | -0.06% | -0.15% | 0.04% | 99% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

References

- K. Naser, T. Poirier, F. Le Léannec and G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “Non-EE2: Intra TMP with half-pel precision”, JVET-AC0087, Jan. 2023.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AC2024, Jan. 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).