| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0126-v1 |
| --- | --- |

| Title: | EE2-1.20k: Combination of Test 1.12 and Test 1.14c |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li, Ru-Ling Liao, Jie Chen, Yan Ye (Alibaba group) Yanzhuo Ma, Junyan Huo, Hongqing Du, Hongli Zhang, Zhenyao Zhang, Wenhan Qiao, Fuzheng Yang (Xidian University) Fan Wang, Lai Zhang, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | sid.lxw@alibaba-inc.com yzma@mail.xidian.edu.cn wangfan6@oppo.com |
| Source: | Alibaba group, Xidian University, OPPO |  |  |

_____________________________

- Abstract

This contribution reports results of EE2 Test 1.20k, which is a combination of Test 1.12 (Intra TMP with sub-pel precision) and Test 1.14c (combination of Test 1.14a which extend the adjacent search area of Intra TMP and Test 1.14b which enlarge the search range for small blocks of Intra TMP). It is reported that on top of ECM-8.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are {-0.51%, -0.55%, -0.52%, 100%, 101%} in AI and {-0.18%, -0.16%, -0.13%, 100%, 100%} in RA.

Introduction

This contribution presents the simulation results of EE2 Test 1.20k which studies the joint test of Test 1.12 [1] and Test 1.14c [2].

In Test 1.12, sub-pel precision is supported in Intra TMP. Specifically, three precisions (1/2-pel, 1/4-pel and 3/4-pel) in eight directions around the integer-pel position obtained by template matching are further enabled to Intra TMP. The eight directions are further sorted by template cost for each CU with 1/2-pel precision and only the first four direction can be used. The selected precision and direction are signaled in CU level. Four-tap DCT-IF interpolation filters are used for sub-pel interpolation.

Test 1.14c is a combination of Test 1.14a and Test 1.14c. In Test 1.14a, the search range of Intra TMP is modified so that the adjacent neighboring region of the coding block can be searched. In Test 1.14b, the search range of Intra TMP is enlarged for small blocks.

Simulation results

The proposed method is implemented on top of ECM-8.0 and is tested under CTC for enhanced compression tool testing [4] with all intra and random access configurations. The simulation result of Test 1.20k is shown in Table 3.

Table 3. Test 1.20k: Combination of Test 1.12 and Test 1.14c

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.01% | 99% | 98% |
| Class A2 | -0.81% | -0.95% | -1.02% | 99% | 100% |
| Class B | -0.51% | -0.55% | -0.52% | 101% | 102% |
| Class C | -0.20% | -0.21% | -0.20% | 100% | 100% |
| Class E | -1.07% | -1.14% | -0.96% | 100% | 101% |
| Overall | -0.51% | -0.55% | -0.52% | 100% | 101% |
| Class D | -0.42% | -0.77% | -0.45% | 101% | 100% |
| Class F | -0.06% | -0.26% | -0.14% | 100% | 101% |
| Class TGM | -0.33% | -0.28% | -0.24% | 100% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.08% | -0.04% | 100% | 100% |
| Class A2 | -0.42% | -0.44% | -0.24% | 100% | 100% |
| Class B | -0.22% | -0.22% | -0.10% | 99% | 100% |
| Class C | -0.08% | -0.05% | -0.14% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.16% | -0.13% | 100% | 100% |
| Class D | -0.03% | -0.21% | -0.24% | 100% | 99% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

References

- X. Li, R.-L. Liao, J. Chen, Y. Ye, “EE2-1.12: Intra TMP with sub-pel precision”, JVET-AD0125, Apr. 2023

- Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang, F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.14c combination test of EE2-1.14a and EE2-1.14b”, JVET-AD0075, Apr. 2023

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).