| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0127-v2 |
| --- | --- |

| Title: | Non-EE2: Harmonization between IBC HMVP and IBC-LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is inherited. To harmonize IBC HMVP and IBC-LIC, it is proposed that the IBC-LIC flag can be inherited from an IBC HMVP candidate.

On top of the ECM8.0, simulation results of the proposed method are reported as below:

AI: Class F -0.31%,99%	,101%; Class TGM -0.05%,101%,101%

RA: Class F -0.26%	,99%,99%; Class TGM -0.02%,100%,99%

# Introduction

Intra block copy with local illumination compensation (IBC-LIC) is a coding tool which compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear equation. In the last MPEG meeting, IBC-LIC is adopted.

In ECM, the inter LIC flag can be inherited from a HMVP candidate. However, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is inherited.

# Proposed methods

In this contribution, it is proposed that the IBC-LIC flag can be inherited from an IBC HMVP candidate to harmonize IBC HMVP and IBC-LIC similar to the inter LIC case.

# Experimental Results

The proposed method has been implemented on top of the ECM-8.0 [1]. Testing results of the proposed method and the combination of the proposed method and EE2-3.7 [2] under AI/RA configurations are summarized in Table 1 and Table 2~Table7. The coding time in Table 2~Table7 are not reliable. The simulations were performed following the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.31% | -0.30% | -0.21% | 99% | 101% |
| Class TGM | -0.05% | 0.00% | 0.00% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.48% | -0.24% | 99% | 99% |
| Class TGM | -0.02% | 0.01% | 0.00% | 100% | 99% |

Table 2: Coding performance of the combination of the proposed method and EE2-3.7a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.72% | -0.75% | -0.71% | 102% | 102% |
| Class TGM | -0.79% | -0.75% | -0.72% | 101% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.63% | -0.78% | -0.54% | 101% | 100% |
| Class TGM | -0.91% | -0.81% | -0.84% | 101% | 99% |

Table 3: Coding performance of the combination of the proposed method and EE2-3.7b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.72% | -0.67% | -0.83% | 103% | 105% |
| Class TGM | -0.97% | -0.92% | -0.86% | 102% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.87% | -0.85% | 102% | 99% |
| Class TGM | -1.11% | -0.96% | -0.97% | 101% | 100% |

Table 4: Coding performance of the combination of the proposed method and CfgA1 of EE2-3.7

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.53% | -0.47% | -0.61% | 102% | 99% |
| Class TGM | -0.64% | -0.61% | -0.59% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.55% | -0.73% | -0.29% | 100% | 99% |
| Class TGM | -0.86% | -0.70% | -0.79% | 100% | 99% |

Table 5: Coding performance of the combination of the proposed method and CfgA3 of EE2-3.7

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.77% | -0.82% | -0.90% | 102% | 105% |
| Class TGM | -0.93% | -0.85% | -0.85% | 104% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.73% | -1.00% | -0.76% | 102% | 100% |
| Class TGM | -1.07% | -0.89% | -0.98% | 102% | 100% |

Table 6: Coding performance of the combination of the proposed method and CfgB1 of EE2-3.7

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.66% | -0.79% | 102% | 103% |
| Class TGM | -0.83% | -0.82% | -0.79% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.85% | -0.87% | 101% | 99% |
| Class TGM | -1.01% | -0.85% | -0.93% | 102% | 100% |

Table 7: Coding performance of the combination of the proposed method and CfgB3 of EE2-3.7

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.88% | -0.99% | -0.83% | 107% | 108% |
| Class TGM | -1.17% | -1.10% | -1.08% | 107% | 117% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.72% | -1.06% | -0.70% | 103% | 99% |
| Class TGM | -1.23% | -1.01% | -1.13% | 101% | 101% |

- Conclusions

In this contribution, to further improve the efficiency of IBC prediction, it is proposed to harmonize IBC HMVP and IBC-LIC. The proposed method can bring some coding gain with almost no coding time change. It also has almost no coding performance overlap with EE2-3.7 and it is direct. It is recommended to adopt this method into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

[2] N. Zhang, K. Zhang, L. Zhang, Z. Zhang, P. Nikitin, H. Huang, V. Seregin, M. Karczewicz, “EE2-3.7: Combination tests of EE2-3.2+EE2-3.4b,” JVET-AD0097, Apr. 2023.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).