| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0128-v1 |
| --- | --- |

| Title: | AHG12: modified CTC proposal for low-delay configurations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec F. Galpin K. Naser T. Poirier E. François | Tel: Email: | Fabrice.Leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to modify EE2 common test conditions, to significantly reduce the worst-case encoding runtime for LDB/LDP, together with slightly improved overall BD-rate results.

The CTC proposal consists in modifying encoder configuration as follows. The maximum MTT hierarchy depth is set to 4 in the I slice. For QP 22 of HD sequences, it is decreased from 3 to 2 for slices with temporal depth higher or equal to 2.

In LDB, the proposed modified CTC reportedly leads to ~66% encoding time for the worst case (BQTerrace QP 22), 97% average encoding time relative to ECM-8.0 CTC, with -0.22% luma coding gain.

In LDP, the proposed modified CTC reportedly leads to ~70% encoding time for the worst case, 100% average encoding time relative to ECM-8.0 CTC, with -0.17% luma coding gain.

# Introduction

ECM-8.0 encoding time is becoming problematic for investigating new coding tools. In particular, the worst-case encoding time for low-delay B HD sequences makes it very long to wait for complete results.

In the current Ahg12 CTC, the worst cases correspond to the LDB QP 22 encoding of class B and class F “Arena of Valor” sequences.

Depending on the processing performance and configuration used, around 15 days of computation can be needed to generate full results (case of BQTerrace QP 22).

Therefore, this contribution proposes to reduce the encoding worst case, in a similar way to what was adopted for random class A QP 22 cases ([1]).

The encoding parameters used in the proposed modified CTCs are:

- MaxBTLumaISlice=128

- MaxBTChromaISlice=128

- MaxTTLumaISlice=128

- MaxTTChromaISlice=128

- MaxMTTHierarchyDepthISliceL=4

- MaxMTTHierarchyDepthISliceC=4

In addition, for class B and class F HD sequence, following parameter is used:

- MaxMTTHierarchyDepthByTidOverrideByQP: 22 332222

# Impact on performances and runtime

The Table 1 below shows the performance changes of ECM with proposed coding parameters, in Ahg12 test conditions [2]. -0.22% and -0.17% luma coding gains are observed with average encoding times of 97% and 100%, respectively for LDB and LDP.

Table 1: performance results of proposed LDB and LDP encoder settings

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | -0,79% | -0,76% | 91% | 100% |
| Class C | -0,14% | -0,12% | -0,52% | 100% | 100% |
| Class E | -0,64% | -0,74% | -2,39% | 102% | 99% |
| Overall | -0,22% | -0,55% | -1,09% | 97% | 100% |
| Class D | -0,10% | -0,93% | 0,04% | 100% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | -0,44% | -0,71% | 95% | 97% |
| Class C | -0,11% | -0,14% | -0,45% | 105% | 101% |
| Class E | -0,52% | -2,04% | -2,77% | 105% | 102% |
| Overall | -0,17% | -0,74% | -1,14% | 100% | 100% |
| Class D | -0,08% | -0,52% | 0,79% | 103% | 102% |
| Class F | -0,21% | -0,65% | -0,25% | 110% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

The Table 2 below depicts the reduction of worst-case encoding times brought by the proposed modified CTCs. As can be seen, the longest simulation time is reduced by 4.75 days for LDB and 2.25 days for LDP. This corresponds to coding times reduced by 34% and 30%. Several days are also saved for all QP 22 cases in LDB and 1 to 2 days in LDP.

It is stated these modified encoder settings make the ECM more practical to use in low-delay configurations.

Table 2: impact of proposed CTCs on worst case encoding times

|  | ECM-8.0, low-delay B, QP=22 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reference |  | Test |  |  |  |
| Sequence | Enc T [h] | Enc T [d] | Enc T [h] | Enc T [d] | ΔEnc T [d] | EncR |
| MarketPlace | 259,20 | 10,80 | 173,61 | 7,23 | -3,57 | 0,67 |
| RitualDance | 171,69 | 7,15 | 118,90 | 4,95 | -2,20 | 0,69 |
| Cactus | 213,74 | 8,91 | 139,82 | 5,83 | -3,08 | 0,65 |
| BasketballDrive | 267,10 | 11,13 | 171,89 | 7,16 | -3,97 | 0,64 |
| BQTerrace | 331,04 | 13,79 | 216,93 | 9,04 | -4,75 | 0,66 |
| ArenaOfValor | 223,05 | 9,29 | 0,00 | 0,00 |  | 0,00 |

|  | ECM-8.0, low-delay P, QP=22 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reference |  | Test |  |  |  |
| Sequence | Enc T [h] | Enc T [d] | Enc T [h] | Enc T [d] | ΔEnc T [d] | EncR |
| MarketPlace | 182,26 | 7,59 | 136,98 | 5,71 | -1,89 | 0,75 |
| RitualDance | 114,60 | 4,77 | 83,03 | 3,46 | -1,32 | 0,72 |
| Cactus | 137,81 | 5,74 | 98,26 | 4,09 | -1,65 | 0,71 |
| BasketballDrive | 180,35 | 7,51 | 126,26 | 5,26 | -2,25 | 0,70 |
| BQTerrace | 225,59 | 9,40 |  |  |  |  |
| ArenaOfValor | 165,32 | 6,89 | 129,60 | 5,40 | -1,49 | 0,78 |

# Conclusion

A modification of EE2 CTCs is proposed to significantly reduce the worst-case encoding time of ECM in low delay configurations.

It is shown the overall performance in EE2 CTCs would be slightly improved, while the ECM test set is made more convenient to use in practice.

Furthermore, some visual checks were performed, which confirmed no visual quality decrease was noticed with the proposal.

It is proposed to adopt one of the two proposed CTC modifications for the next JVET meetings.

# References

- JVET-AA0098, “Ahg 12: encoder configuration proposal to reduce worst case encoding time”, F. Le Léannec, P. Andrivon, M. Radosavljević, Xiaomi, July 2022

- JVET-Y2017, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, 	M. Karczewicz, Y. Ye, January 2022

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).