| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0130-v1 |
| --- | --- |

| Title: | AhG12: On ECM partitioning prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno | Tel: Email: | 33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a temporal prediction method of the partitioning parameters. The partitioning splits allowance and partitioning depths are derived for each block according to the partitioning parameters of the current frame and parameters obtained from a temporal area. It is reported that the proposed modifications give an average coding gain for both RA and LDB configurations and an average encoding time decrease.

Compared to ECM-8.0, the average BDR gains and runtimes reported in this contribution are as follows:

-0.09%	-0.07%	-0.07%  -  98%, 101% for the RA configuration,

-0.05%	0.23%	0.17%  -  99%, 102% for the Low Delay B configuration.

# Introduction

The partitioning parameters of QT TT BT seem to have temporal relationships. In the current ECM, the subblock temporal exploits the temporal correlations as well as the CABAC initialization from previous inter slice (JVET-Y0181) [2]. In this contribution, we propose to go further by predictions of the partitioning parameters for each block based on temporal parameters obtained from several temporal blocks.

# Proposed method

For each block, the allowances of splits partitioning are predicted according to the minimum QT split and the average QT split obtained from a temporal area. So, when the current QT depth is inferior to the temporal minimum QT depth minus 1, only the QT split is allowed and when the current QT depth is inferior to the temporal average QT depth minus 1, No split, QT Split and TT split are allowed and BT split is allowed if TT is selected in a parent node.

In the same way, the maximum multi-tree depth is predicted for each block. If the temporal maximum multi-tree depth is inferior to the maximum multi-tree depth of the current block, it is decreased except if the current QP is superior or equal to the QP of the reference frame. If the temporal maximum multi-tree depth is superior to the maximum multi-tree depth of the current block and if the current depth is equal to the current QT depth the maximum multi-tree depth is incremented.

# Results

The proposed modifications are implemented on top of the ECM-8.0 [1] and tested according to the common test conditions (CTC) for ECM-8.0. The result details per classes of this implementation are reported in the following table.

Table 2 Coding performances of the proposed modifications.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0_ANCHOR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.23% | -0.23% | 102% | 101% |
| Class A2 | -0.12% | -0.11% | -0.18% | 99% | 101% |
| Class B | -0.09% | -0.08% | -0.03% | 97% | 101% |
| Class C | -0.03% | 0.09% | 0.09% | 94% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.07% | -0.07% | 98% | 101% |
| Class D | -0.02% | 0.07% | 0.00% | 96% | 97% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-8.0_ANCHOR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.37% | 0.17% | 98% | 102% |
| Class C | -0.05% | 0.54% | 0.06% | 97% | 102% |
| Class E | -0.12% | 0.80% | 0.32% | 99% | 102% |
| Overall | -0.05% | 0.23% | 0.17% | 98% | 102% |
| Class D | -0.01% | -0.75% | 0.25% | 99% | 102% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

A gain is obtained for both configurations as well as an encoding time decrease.

# Conclusion

In this contribution, some modifications of the QT BT TT partitioning are proposed. The method predicts some allowances of splits and depths according to the parameters obtained from a temporal area in a reference frame.

An average BD-rate gain is obtained for both RA and LDB configurations with a systematic gain per class and with a decrease of encoding time.

We suggest considering the proposed changes for investigation in the next round of EE2 in the ECM-9.0.

# References

- ECM-8.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

- Vadim Seregin, Jie Dong, Nan Hu, Marta Karczewicz, “AHG12: CABAC initialization from previous inter slice,” JVET-Y0181, Jan. 2022.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).