| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, by Antalya, 21–28 April 2023 | Document: JVET-AD0139-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AD0096 (EE2-3.4a: Copy-Padding for IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Heeji Han and Haechul Choi | Tel: Email: | +82428211149 sarhakor@naver.com choihc@hanbat.ac.kr |
| Source: | Hanbat National University |  |  |

_____________________________

- Abstract

In JVET-AC0193, IBC copy-padding method allowing reference to invalid regions was proposed. Compared to the current determination, the copy-padding method allows the use of block vectors that are closer by enabling reference to invalid regions, and as a result, effective reduction of bit rate was achieved while maintaining objective visual quality. This document verifies the performance numbers in JVET-AD0096.

Introduction

The proposal [1][2] is related to the IBC BV search process, in which the overlapped area between the current block and the reference block is padded. Cross-check of the source code and simulation results have been conducted. Inspected source code follows the description in the proposal. Cross-check is run on AI and RA on top of ECM-8.0[3].

Results

The following tables show our experimental results for the cross-check.

The encoding and decoding times under AI configuration are reliable, as they were measured on a computer with the same performance level. On the other hand, the results under the RA configuration may be less precise due to the use of multiple computers with varying specifications.

- Crosscheck results on EE2-3.4a

| All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Over ECM-8.0 |  |  |  |  |  |  |
| Sequences |  | Y | U | V | EncT | DecT |
| classF | BasketballDrillText | -0.02 | 0.15 | 0.07 | 95.95% |  |
|  | ArenaOfVlavor | 0.01 | -0.10 | 0.02 | 93.30% |  |
|  | SlideEditing | -0.02 | -0.02 | 0.07 | 94.12% |  |
|  | SlideShow | 0.02 | -0.38 | 0.18 | 97.23% |  |
|  | Average | 0.00 | -0.09 | 0.09 | 94.98% |  |
| classTGM | FlyingGraphic | -0.24 | -0.11 | -0.04 | 90.36% |  |
|  | Desktop | -0.32 | -0.26 | -0.25 | 91.00% |  |
|  | Console | -0.24 | -0.32 | -0.24 | 87.55% |  |
|  | ChineseEditing | -0.07 | -0.14 | -0.09 | 91.45% |  |
|  | Average | -0.22 | -0.21 | -0.16 | 89.87% |  |

| Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Over ECM-8.0 |  |  |  |  |  |  |
| Sequences |  | Y | U | V | EncT | DecT |
| classF | BasketballDrillText | - | - | - | - | - |
|  | ArenaOfVlavor | - | - | - | - | - |
|  | SlideEditing | 0.00 | -0.39 | -0.26 | 91.63% | - |
|  | SlideShow | -0.32 | -1.90 | -1.38 | 91.03% | - |
|  | Average | - | - | - | - | - |
| classTGM | FlyingGraphic | - | - | - | - | - |
|  | Desktop | -0.40 | -0.26 | -0.31 | 73.82% | - |
|  | Console | - | - | - | - | - |
|  | ChineseEditing | -0.09 | 0.04 | -0.11 | 73.97% | - |
|  | Average | - | - | - | - | - |

- Difference between crosscheck result and EE2-3.4a(JVET-AD0096) result

| All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Over EE2-3.4a (OVERLAPIBC on / TBVP off) |  |  |  |  |  |  |
| Sequences |  | Y | U | V | EncT | DecT |
| classF | BasketballDrillText | 0.00 | 0.00 | 0.00 | 101.62% |  |
|  | ArenaOfVlavor | 0.00 | 0.00 | 0.00 | 101.83% |  |
|  | SlideEditing | 0.00 | 0.00 | 0.00 | 98.54% |  |
|  | SlideShow | 0.00 | 0.00 | 0.00 | 102.09% |  |
|  | Average | 0.00 | 0.00 | 0.00 | 101.00% |  |
| classTGM | FlyingGraphic | 0.00 | 0.00 | 0.00 | 98.90% |  |
|  | Desktop | 0.00 | 0.00 | 0.00 | 96.53% |  |
|  | Console | 0.00 | 0.00 | 0.00 | 96.84% |  |
|  | ChineseEditing | 0.00 | 0.00 | 0.00 | 97.54% |  |
|  | Average | 0.00 | 0.00 | 0.00 | 97.44% |  |

| Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Over EE2-3.4a (OVERLAPIBC on / TBVP off) |  |  |  |  |  |  |
| Sequences |  | Y | U | V | EncT | DecT |
| classF | BasketballDrillText | - | - | - | - | - |
|  | ArenaOfVlavor | - | - | - | - | - |
|  | SlideEditing | 0.00 | 0.00 | 0.00 | 94.51% | - |
|  | SlideShow | 0.00 | 0.00 | 0.00 | 96.65% | - |
|  | Average | - | - | - | - | - |
| classTGM | FlyingGraphic | - | - | - | - | - |
|  | Desktop | 0.00 | 0.00 | 0.00 | 80.17% | - |
|  | Console | - | - | - | - | - |
|  | ChineseEditing | 0.00 | 0.00 | 0.00 | 81.75% | - |
|  | Average | - | - | - | - | - |

Conclusions

Test EE2-3.4a was cross-checked and the BD-rates match with the result provided by the proponent in JVET-AD0096. Inspection of the code shows consistency between the proposition and the implementation.

References

[1] Na. Zhang, Kai Zjamg, and Li Zhang, “Non-EE2: Copy-Padding for IBC”, JVET-AC0193, online, January 2023

[2] Na. Zhang, Kai Zjamg, and Li Zhang,  “EE2-3.4: Copy-Padding for IBC”, JVET-AD0096, Antalya, April 2023

[3] ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[4] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.