| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0143-v1 |
| --- | --- |

| Title: | AHG9: On picture size for input and output pictures of NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Hyeongmoon Jang Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contributions asserted that modifications related to picture size related to input pictures and output pictures of NNPFC SEI messages are needed.

For input picture size, a constraint is proposed to clarify such that when an NNPF is activated by an NNPFA SEI message and the NNPF takes / uses multiple input pictures (i.e., as specified by NNPFC SEI message), it is constrained that all input pictures of the NNPF have the same size as the current picture (i.e., the picture in the same access unit that contains the NNPFA SEI message).

For output picture size, it is proposed that the signalling design accommodates expression of output picture size that include signalling a fixed output picture size and signalling output picture size based on ratio of the input picture size.

# Problem Descriptions

## Size of input pictures

In the current design of neural-network post-filter characteristics (NNPFC) SEI message, size (width and height) of input picture(s) is not signalled but is given / assumed available. It is worth to note that while only one picture size is given, an NNPF may be designed to take multiple input pictures. Furthermore, it is possible that the input bitstream is coded such that multiple picture resolutions may be present within coded video sequence of the bitstream. For example, VVC support reference picture resampling (RPR) feature.

Although, one may assume that since only one input picture size is given, it is already implicitly constrained that input pictures for the NNPF defined in an NNPFC SEI message shall all have the same size, others may interpret it in creative way that allows pictures with different picture size to be used as input for the NNPF. For example, one may assume that if an input picture has size larger than the given input picture size, it is assumed that subset (e.g., the area from the top-left point to the given width and height) of the picture can still be used as input. It would be desirable if this issue is clarified.

## Size of output pictures

For handling of output pictures’ size, the following design is currently used:

- If the purpose(s) of the NNPF does not include spatial up-sampling, the size of output picture is inferred to be the same as the size of the input picture.

- Otherwise, the size of output picture(s) is signalled.

Considering that an NNPF that is carried in an NNPFC SEI message may be activated by one or more neural-network post-filter activation (NNPFA) SEI messages at different pictures, it is possible that the size of input picture(s) may be different at depending on the time of activation and yet currently when the NNPF includes spatial up-sampling purpose, the size of output picture is fixed to be the same.

In our assertion, both of the following scenarios are possible and desirable:

- The bitstream has multiple picture sizes and an NNPF is used to up-sample the resolution of all pictures to be the same size. In this case, the current design of NNPFC SEI message already accommodate this.

- The bitstream has multiple picture sizes and an NNPF is used to up-sample the resolution of all pictures while keeping the ratio of the picture resolution of the input bitstream.

For the second scenario, one may argue that multiple NNPFC SEI messages with fixed output picture size can be present and activated as needed. However, if the difference among these NNPFC SEI messages is only their output picture size, it would be good if there is a way to handle this with only one NNPFC SEI message. We recommend the JVET experts to consider the above two scenarios that involve NNPF for spatial up-sampling and find a solution to accommodate them.

# Proposals and text implementation

## Proposal 1: regarding size of input pictures

We propose to add a constraint such that when an NNPF is activated by an NNPFA SEI message and the NNPF takes / uses multiple input pictures (i.e., as specified by NNPFC SEI message), it is constrained that all input pictures of the NNPF have the same size as the current picture (i.e., the picture in the same access unit that contains the NNPFA SEI message).

The text implementation of the proposed constraint is as follows:

    - Neural-network post-filter activation SEI message semantics

The neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter (NNPF), identified by nnpfa_target_id, for post-processing filtering of a set of pictures. For a particular picture for which the NNPF is activated, the following applies:

–	Tthe target NNPF is the NNPF specified by the last NNPFC SEI message with nnpfc_id equal to nnpfa_target_id, that precedes the first VCL NAL unit of the current picture in decoding order that is not a repetition of the NNPFC SEI message that contains the base NNPF.

NOTE 1 – There can be several NNPFA SEI messages present for the same picture, for example, when the NNPFs are meant for different purposes or for filtering of different colour components.

–	If the target NNPF takes multiple input pictures, all input pictures shall have the same resolution as the resolution of the current picture.

…

## Proposal 2: regarding size of output pictures

We propose to accommodate expression of output picture size that include both mechanisms (i.e., fixed output picture size and output picture size based on ratio of the input picture size). The needed changes to support this are as follows:

- For signalling output picture size, add a flag (i.e., nnpfc_output_pic_size_flag) to specify whether the output picture size is a fixed size (i.e., given width and height) or a ratio relative to the input picture size.

  - nnpfc_output_pic_size_flag equal to 0 means width and height in luma samples for output picture size are present.

  - nnpfc_output_pic_size_flag equal to 1 means ratio of output picture relative to the input picture size is present.

- When the value of nnpfc_output_pic_size_flag is equal to 1, the following information is signalled:

  - nnpfc_pic_width_ratio_num and nnpfc_pic_width_ratio_denum specify the numerator and denumerator for the ratio of output picture width relative to the input picture width.

  - nnpfc_pic_height_ratio_num and nnpfc_pic_height_ratio_denum specify the numerator and denumerator for the ratio of output picture height relative to the input picture height.

The text implementation of the proposed solution is as follows:

    - Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| … |  |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_output_pic_size_flag | u(1) |
| if( nnpfc_output_pic_size_flag = = 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } else { |  |
| nnpfc_pic_width_ratio_num | ue(v) |
| nnpfc_pic_width_ratio_denum | ue(v) |
| nnpfc_pic_height_ratio_num | ue(v) |
| nnpfc_pic_height_ratio_denum | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_output_pic_size_flag equal 0 specifies that nnpfc_pic_width_in_luma_samples and nnpfc_pic_width_in_luma_samples are present. nnpfc_output_pic_size_flag equal 1 specifies that nnpfc_pic_width_ratio_num, nnpfc_pic_width_ratio_denum, nnpfc_pic_height_ratio_num,  and nnpfc_pic_height_ratio_denum are present.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the NNPF identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_pic_width_ratio_num and nnpfc_pic_width_ratio_denum specify the numerator and denumerator for the ratio of output picture width relative to CroppedWidth. The value of nnpfc_pic_width_ratio_num / nnpfc_pic_width_ratio_denum shall be in the range of 1 to 16 − 1, inclusive.

The variable PicWidthInLumaSamples if derived as follows:

- If nnpfc_output_pic_size_flag is not present, the value of PicWidthInLumaSamples is set to be equal to CroppedWidth.

- Else if nnpfc_output_pic_size_flag is equal to 0, the value of PicWidthInLumaSamples is set to be equal to nnpfc_pic_width_in_luma_samples.

- Otherwise (i.e., nnpfc_output_pic_size_flag is equal to 1), the value of PicWidthInLumaSamples is set to be equal to ( CroppedWidth * nnpfc_pic_width_ratio_num ) / nnpfc_pic_width_ratio_denum.

nnpfc_pic_height_ratio_num and nnpfc_pic_height_ratio_denum specify the numerator and denumerator for the ratio of output picture height relative to CroppedHeight. The value of nnpfc_pic_height_ratio_num / nnpfc_pic_height_ratio_denum shall be in the range of 1 to 16 − 1, inclusive.

The variable PicHeightInLumaSamples if derived as follows:

- If nnpfc_output_pic_size_flag is not present, the value of PicHeightInLumaSamples is set to be equal to CroppedHeight.

- Else if nnpfc_output_pic_size_flag is equal to 0, the value of PicHeightInLumaSamples is set to be equal to nnpfc_pic_height_in_luma_samples.

- Otherwise (i.e., nnpfc_output_pic_size_flag is equal to 1), the value of PicHeightInLumaSamples is set to be equal to ( CroppedHeight * nnpfc_pic_height_ratio_num ) / nnpfc_pic_height_ratio_denum.

…

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples  &&  xY < PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(85)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples  && xY < PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

…

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, and outPatchCHeight are derived as follows:

outPatchWidth = (PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth	(86)

outPatchHeight = (PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight	(87)

horCScaling = SubWidthC / outSubWidthC		(88)

verCScaling = SubHeightC / outSubHeightC		(89)

outPatchCWidth = outPatchWidth * horCScaling		(90)

outPatchCHeight = outPatchHeight * verCScaling		(91)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to PicWidthInLumaSamples nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to PicHeightInLumaSamples nnpfc_pic_height_in_luma_samples * inpPatchHeight.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).