| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0146-v1 |
| --- | --- |

| Title: | EE2-releated: EE2-1.15a improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Xue Zhang, Cheng Fang, Dong Jiang，Jucai Lin, Kai Fu, Peng Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

In EE2-1.15a, a linear filter method for the prediction of Intra TMP (FTMP) is proposed to improve compression efficiency of ECM, where the filter input consists of 5 spatial samples and a bias term. In this contribution, a gradient and location based filter method is proposed. The proposed method is implemented on top of EE2-1.15a, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

Results on EE2-1.15a anchor:

AI: -0.04%, -0.05%, 0.00%, 95% (EncT), 94% (DecT)

Results on ECM8.0 anchor:

AI: -0.16%, -0.19%, -0.17%, 97% (EncT), 93% (DecT)

Introduction

In EE2-1.15a [1], a 6-tap filter consist of a 5-tap plus sign shape spatial component and a bias term is proposed [2]. The input to the spatial 5-tap component of the filter consists of a center (C) sample in the reference block which is at corresponding locations with the sample in the current block to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Spatial part of the filter

The bias term B represents a scalar offset between the input and output, and is set to middle luma value (512 for 10-bit content).

The output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5B

where ci are the filter coefficients.

In GL-CCCM mode of ECM8.0 [3], gradient and location are used to improve the cross-component filter. Similarly, it can be used to improve the FTMP filter.

Proposed method

In this contribution, a gradient and location based linear filter model for the prediction of Intra TMP (GL-FTMP) is proposed.

The output of the GL-FTMP filter is calculated as follows:

predLumaVal = c0C + c1Gx + c2Gy + c3X + c4Y + c5B

Where Gy and Gx are the vertical and horizontal gradients, respectively, and are calculated as:

Gy = (2N + NW + NE) – (2S + SW + SE)

Gx = (2W + NW + SW) – (2E + NE + SE)

Moreover, the Y and X parameters are the vertical and horizontal locations of the center luma sample.

The parameter calculation process of the proposed method is the same as EE2-1.15a.

Spatial samples used for calculating Gy and Gx

The GL-FTMP mode is signalled with a CU level flag. Specifically, GL-FTMP is considered a sub-mode of FTMP. That is, GL-FTMP flag is only signalled if FTMP flag is true.

Experimental results

The proposed method is implemented on top of EE2-1.15a software under the common test conditions[4]. It is noted that the encoding and decoding runtime reported in the results is unreliable, as the runtime is obtained under non-aligned computing cluster conditions of anchor and test.

Results on EE2-1.15a anchor:

Results of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.15a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.05% | 0.11% | 98% | 99% |
| Class A2 | -0.02% | -0.09% | -0.01% | 99% | 101% |
| Class B | -0.03% | -0.09% | -0.05% | 98% | 101% |
| Class C | -0.02% | -0.05% | -0.02% | 90% | 81% |
| Class E | -0.13% | 0.03% | -0.02% | 90% | 92% |
| Overall | -0.04% | -0.05% | 0.00% | 95% | 94% |
| Class D | -0.02% | -0.18% | 0.18% | 98% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Results on ECM8.0 anchor:

The proposed method + EE2-1.15a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.01% | 0.04% | 97% | 104% |
| Class A2 | -0.23% | -0.25% | -0.19% | 101% | 97% |
| Class B | -0.19% | -0.27% | -0.20% | 103% | 97% |
| Class C | -0.06% | -0.12% | -0.16% | 93% | 83% |
| Class E | -0.33% | -0.25% | -0.31% | 93% | 85% |
| Overall | -0.16% | -0.19% | -0.17% | 97% | 93% |
| Class D | -0.03% | -0.22% | 0.06% | 100% | 112% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Additional results, EE2-1.15a vs. ECM8.0.

EE2-1.15a vs. ECM8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 99% | 105% |
| Class A2 | -0.20% | -0.16% | -0.19% | 102% | 96% |
| Class B | -0.16% | -0.18% | -0.15% | 104% | 96% |
| Class C | -0.04% | -0.07% | -0.13% | 103% | 102% |
| Class E | -0.20% | -0.29% | -0.29% | 103% | 92% |
| Overall | -0.12% | -0.13% | -0.16% | 102% | 98% |
| Class D | -0.02% | -0.04% | -0.12% | 103% | 111% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This contribution proposes a gradient and location based filter extension method for EE2-1.15a. The results show that the proposed method can bring obvious coding gains. It is suggested to study the proposed method in the next EE round.

References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Wiken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, Jan. 2023.

- J. Huo, W. Qiao, X. Hao, Z. Zhang, H. Du, Y. Ma, F. Yang, “Non-EE2: Intra template matching (Intra TMP) based on linear filter model,” JVET-AC0109, Jan. 2023.

- M. Coban, F. L. Léannec, R-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).