| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0148-v1 |
| --- | --- |

| Title: | Non-EE2: SGPM combined with multiple IntraTMP predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Shuang Peng, Dong Jiang，Jucai Lin, Xue Zhang, Heng Jin, Xiameng Shi, Fang Ye | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution introduces multiple IntraTMP predictors to the SGPM design. In this method, the two geometric blocks can not only use intra prediction modes, but also use IntraTMP mode. Besides, it is proposed to use DIMD to derive an intra prediction mode based on the SGPM prediction samples, then the MTS transform set or the LFNST transform set is respectively determined by the derived intra mode. The proposed method was implemented on top of ECM-8.0, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

Test1:

AI: -0.07%, -0.10%, -0.05%, 102% (EncT), 108% (DecT)

Test2:

AI: -0.02%, -0.05%, 0.00%, 100% (EncT), 101% (DecT)

Introduction

In ECM-7.0[1], the Spatial GPM method was adopted to intra prediction. SGPM consists of one partition mode and two associated intra prediction modes for two geometric blocks. The length of the candidate list of combinations is set equal to 16.

SGPM uses a candidate list to express the necessary partition and prediction combinations, and these combinations are ranked in ascending order of their SATD between the prediction and reconstruction of the template. The length of the candidate is set equal to 16.

In the last meeting, JVET-AC0097[2] proposed to introduce one IntraTMP poredictor to the geometric block which has more corresponding template pixels.

In addition, JVET-AC0115[3] Test-4.1a was adopted to use DIMD to derive an intra prediction mode based on the IntraTMP prediction samples, then the MTS transform set or the LFNST transform set is respectively determined by the derived intra mode.

Proposed method

In this contribution, we introduce 3 IntraTMP predictors to the SGPM design. By using the IntraTMP searching and SAD ranking process, the best 3 BV are selected as predictors. Both of the geometric blocks can use the 3 IntraTMP predictors to do the prediction, an example is shown in Fig.1. However, if a geometric block whose corresponding template pixel number is smaller than the one quarter of the full template pixel number, it is forbidden to use the IntraTMP predictor.

SGPM combined with IntraTMP

The IntraTMP predictors only join the SGPM template rank process, so the candidate list length is still 16, and no additional syntax elements are required.

Besides, similar to JVET-AC0115, it is proposed to use DIMD to derive an intra prediction mode based on the SGPM prediction samples, then the MTS transform set or the LFNST transform set is respectively determined by the derived intra mode.

Experimental results

We implement two tests: Test1 is the whole proposed method, Test2 only test the DIMD derived intra mode based transform in SGPM.

Both tests were implemented on top of ECM-8.0 and tested for AI configurations for class A to class F. The following experimental results were obtained. The encoding and decoding running times may have some fluctuation.

Results of the Test1 on top of ECM-8.0 under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.01% | 103% | 107% |
| Class A2 | -0.08% | -0.04% | -0.03% | 99% | 108% |
| Class B | -0.06% | -0.12% | -0.06% | 102% | 108% |
| Class C | -0.06% | -0.09% | -0.13% | 104% | 107% |
| Class E | -0.14% | -0.20% | 0.00% | 102% | 108% |
| Overall | -0.07% | -0.10% | -0.05% | 102% | 108% |
| Class D | -0.03% | -0.05% | -0.05% | 101% | 109% |
| Class F | 0.06% | -0.03% | 0.15% | 104% | 113% |

Results of the Test2 on top of ECM-8.0 under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.08% | 101% | 101% |
| Class A2 | -0.01% | -0.09% | 0.00% | 99% | 99% |
| Class B | -0.01% | -0.02% | 0.06% | 100% | 100% |
| Class C | -0.03% | -0.05% | -0.03% | 101% | 101% |
| Class E | -0.02% | -0.08% | 0.02% | 100% | 102% |
| Overall | -0.02% | -0.05% | 0.00% | 100% | 101% |
| Class D | -0.04% | -0.20% | -0.01% | 103% | 102% |
| Class F | 0.03% | -0.02% | 0.01% | 101% | 103% |

Conclusions

This contribution introduces multiple IntraTMP predictors to the SGPM design. The results show that the proposed method can bring more coding gains.

References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- Dongcheol Kim, Kyungyong Kim, Ju-Hyung Son, Jin Sam Kwak, K. Naser, T. Poirier, F. Galpin, A. Robert, “EE2-4.1: Experimental results of EE2-4.1a, EE2-4.1b, and EE2-4.1c”, JVET-AC0115, Jan. 2023.

- Cheng Fang, Shuang Peng, Dong Jiang, Jucai Lin, Xue Zhang, “Non-EE2: SGPM combined with IntraTMP”, JVET-AC0097, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).