| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0150-v1 |
| --- | --- |

| Title: | Non-EE2: combination of JVET-AD0146 and JVET-AD0148 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Shuang Peng, Xue Zhang, Dong Jiang，Jucai Lin, Heng Jin, Xiameng Shi, Kai Fu, Peng Zhang, Fang Ye | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

In JVET-AD0148, an IntraTMP combined SGPM method is proposed. In JVET-AD0146, a gradient and location based FTMP method is proposed. This contribution implements a combination test of  these two methods, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

Results on EE2-1.15a anchor:

AI: -0.09%, -0.06%, 0.02%, 101% (EncT),110% (DecT)

Results on ECM8.0 anchor:

AI: -0.21%, -0.19%, -0.15%, 103% (EncT), 108% (DecT)

Introduction

In JVET-AD0148 [1], an IntraTMP combined SGPM method is proposed. In the proposed method, we introduce multiple IntraTMP predictors to the SGPM design, and both geometric blocks can use IntraTMP predictors to do the prediction. Besides, it is proposed to use DIMD to derive an intra prediction mode based on the SGPM prediction samples, then the MTS transform set or the LFNST transform set is respectively determined by the derived intra mode

In JVET-AD0146 [2], a gradient and location based FTMP method is proposed. In this contribution, a gradient and location based linear filter model for the prediction of Intra TMP (GL-FTMP) is proposed.

The output of the GL-FTMP filter is calculated as follows:

predLumaVal = c0C + c1Gx + c2Gy + c3X + c4Y + c5B

Where Gy and Gx are the vertical and horizontal gradients, and the Y and X parameters are the vertical and horizontal locations of the center luma sample.

Proposed method

JVET-AD0146 and JVET-AD0148 are both IntraTMP related contributions, so we implement a combination test to see whether the gain can added.

Experimental results

The proposed method is implemented on top of EE2-1.15a software under the common test conditions[3]. The encoding and decoding running times may have some fluctuation, as the runtime is obtained under different computing clusters.

Results on EE2-1.15a anchor:

Results of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.15a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.00% | 0.12% | 102% | 105% |
| Class A2 | -0.06% | -0.10% | -0.02% | 103% | 114% |
| Class B | -0.08% | -0.08% | -0.01% | 97% | 114% |
| Class C | -0.08% | -0.04% | 0.01% | 103% | 102% |
| Class E | -0.21% | -0.07% | 0.00% | 100% | 117% |
| Overall | -0.09% | -0.06% | 0.02% | 101% | 110% |
| Class D | -0.04% | -0.07% | 0.01% | 100% | 99% |
| Class F | -0.10% | -0.32% | -0.16% | 100% | 105% |

Results on ECM8.0 anchor:

The proposed method + EE2-1.15a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.05% | 0.05% | 101% | 110% |
| Class A2 | -0.26% | -0.26% | -0.21% | 105% | 109% |
| Class B | -0.24% | -0.26% | -0.16% | 101% | 109% |
| Class C | -0.12% | -0.11% | -0.12% | 106% | 104% |
| Class E | -0.41% | -0.36% | -0.29% | 103% | 108% |
| Overall | -0.21% | -0.19% | -0.15% | 103% | 108% |
| Class D | -0.06% | -0.11% | -0.10% | 102% | 109% |
| Class F | -0.06% | -0.29% | -0.08% | 104% | 110% |

Additional results, EE2-1.15a vs. ECM8.0.

EE2-1.15a vs. ECM8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 99% | 105% |
| Class A2 | -0.20% | -0.16% | -0.19% | 102% | 96% |
| Class B | -0.16% | -0.18% | -0.15% | 104% | 96% |
| Class C | -0.04% | -0.07% | -0.13% | 103% | 102% |
| Class E | -0.20% | -0.29% | -0.29% | 103% | 92% |
| Overall | -0.12% | -0.13% | -0.16% | 102% | 98% |
| Class D | -0.02% | -0.04% | -0.12% | 103% | 111% |
| Class F | 0.04% | 0.03% | 0.07% | 104% | 105% |

Conclusions

The results show that the gain of the two methods can be added. It is suggested to study the proposed method in the next EE round.

References

- Cheng Fang, Shuang Peng, Dong Jiang，Jucai Lin, Xue Zhang, Heng Jin, Xiameng Shi, Fang Ye, “Non-EE2: SGPM combined with multiple IntraTMP predictors”, JVET-AD0148, Apr. 2023.

- Shuang Peng, Xue Zhang, Cheng Fang, Dong Jiang，Jucai Lin, Kai Fu, Peng Zhang, “EE2-releated: EE2-1.15a improvement”, JVET-AD0148, Apr. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).