| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0151-v1 |
| --- | --- |

| Title: | AHG9: On output picture design in NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Hyeongmoon Jang Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

It is asserted that the current text related to input picture and corresponding output picture (i.e., related to syntax element nnpfc_input_pic_output_flag[ i ]) is not clear, particularly when the value of that syntax element is equal to 0. Furthermore, it is also asserted that the current design of NNPFC SEI message related to signalling of output picture should be either modified to make it more flexible such that it is possible for an NNPF to have more than one output picture when it takes more than one input picture, regardless its purpose or be described clearer regarding which input picture corresponds to the output picture when the purpose of the NNPF does not include picture rate upsampling.

This contribution has the following proposals:

Proposal 1: add a note to describe / clarify the notion of output picture and corresponding input picture as follows:

- Input picture that has a corresponding picture outputted from NNPF filtering process may be replaced by the output picture in the picture bitstream after the filtering process.

- Input picture that has no corresponding picture outputted from NNPF filtering process is present in the picture stream after them filtering process.

Proposal 2: do one of the following options:

Option 1:

- Modify signalling whether input picture is outputted (i.e., filtered / modified) such that it is signalled without condition whether the purpose of the NNPF include picture rate upsampling.

- Add a constraint such that when the purpose of an NNPF does not include picture rate upsampling, there shall be at least one output picture corresponds to an input picture.

Option 2:

- Clarify that when the purpose of the NNPF does not include picture rate upsampling, the value of nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1 and the value of nnpfc_input_pic_output_flag[ i ] for i in the range from 1 to nnpfc_num_input_pics_minus1, inclusive, is inferred to be equal to 0.

# Problems description

In the current design of neural-network post-filter characteristics (NNPFC) SEI message, it is asserted that the design and text related to output picture(s) are not clear and may be modified to make it better.

## On the semantics of nnpfc_input_pic_output_flag[ i ]

When the purposes of the NNPF include picture rate upsampling, in addition to the signalling of flags nnpfc_interpolated_pics[ i ] that specifies the number of interpolated picture(s) that are generated between the i-th and the (i + 1)-th input picture, NNPFC SEI message also includes signalling of nnpfc_input_pic_output_flag[ i ] that specifies whether the i-th input picture is outputted. The semantics of nnpfc_input_pic_output_flag[ i ] is asserted to be unclear and may cause different interpretations, particularly related to how the bitstream after the filtering processing, as described in the example below.

Consider the example shown in Figure 1 below. Two NNPFC SEI messages with nnpfc_id 0 and 1 are present and they are activated. The two NNPFs are actually the same filters but with slight difference in input and output pictures signalling signalling. Assume that the (i − 1)-th and (i − 3)-th are associated nnpfc_input_pic_output_flag[ i ] equal to 0, it is asserted that there can be two possible of interpretations:

- Possibility 1: When an input picture is associated with value 0 for its nnpfc_input_pic_output_flag[ i ], it simply means that that picture is not modified / filtered by the NNPF filtering process and it is still part of the final bitstream (i.e,. the bitstream after the filtering process). As illustrated in Figure 1, with this interpretation, the bitstream after the filtering process consists of zero or more pictures that are not modified by the filtering process, one or more pictures that are modified by the filtering process, and one or more new pictures (i.e., interpolated picture(s)) that are generated by the filtering process.

- Possibility 2: When an input picture is associated with value 0 for its nnpfc_input_pic_output_flag[ i ], it means that that picture is not outputted by the NNPF filtering process and therefore it is not part of the final bitstream. In other words, it is removed after the filtering process. As illustrated in Figure 1, with this interpretation, the bitstream after the filtering process consists only one or more pictures that are modified by the filtering process, and one or more new pictures (i.e., interpolated picture(s)) that are generated by the filtering process.

In our assertion, possibility 1 is the intent and the current text is simply not clear about that. This issue can be solved by simply clarifying the semantics of nnpfc_input_pic_output_flag[ i ].

Figure 1 – Illustration of possibilities of interpreting the meaning of output pictures.

## On the multiple output pictures regardless the purposes of an NNPF

Assuming that output picture in NNPFC SEI message means picture that is a modified / filtered version of the input picture and it is intended for replacing that particular input picture in the final bitstream, then one may argue that if there are more than one input pictures, it is possible that there can be more than one filtered pictures, regardless the purpose of the NNPF.

The possibility of having multiple filtered pictures when the NNPF takes more than one input pictures is not currently supported when the purpose of the NNPF does not include picture rate upsampling. Such design is asserted to be limited for no good reason so we recommend the JVET experts to consider allowing a more flexible design on this.

## On output picture when NNPF’s purpose does not include picture rate upsampling

When the purpose of an NNPF does not include picture rate upsampling, currently there is no signalling about output pictures while the number of output picture is set to be equal to 1 (i.e., as per equation (76). This design becomes unclear when the NNPF takes more than one input picture. One may assume that in such case, the output picture corresponds to the first input picture, which is the picture in the same access unit with the NNPFA SEI message that activates the NNPF; however, without clear description, it is difficult to confirm if that is the intent.

In our assertion, if no modification is made to accommodate / solve the issue described above (i.e., problem 1.2), we recommend the JVET group to clarify the one output picture and its input picture correspondence when the purpose of the NNPF does not include picture rate upsampling.

# Proposals

## Proposal 1

To overcome the problem described in 1.1, we propose to add a note to describe / clarify the notion of output picture and corresponding input picture as follows:

- Input picture that has a corresponding picture outputted from NNPF filtering process may be replaced by the output picture in the picture bitstream after the filtering process.

- Input picture that has no corresponding picture outputted from NNPF filtering process is present in the picture stream after them filtering process.

## Proposal 2

To overcome the problem described in 1.2 and 1.3, we propose the following options:

Option 1:

- Modify signalling whether input picture is outputted (i.e., filtered / modified) such that it is signalled without condition whether the purpose of the NNPF include picture rate upsampling.

- Add a constraint such that when the purpose of an NNPF does not include picture rate upsampling, there shall be at least one output picture corresponds to an input picture.

Option 2:

- Clarify that when the purpose of the NNPF does not include picture rate upsampling, the value of nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1 and the value of nnpfc_input_pic_output_flag[ i ] for i in the range from 1 to nnpfc_num_input_pics_minus1, inclusive, is inferred to be equal to 0.

# Text implementations

## For proposal 1

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture.

NOTE x – Input picture that has a corresponding output picture may be replaced by that output picture in the picture stream after the filtering process. Input picture that has no corresponding output picture is present in the picture stream after the filtering process.

…

## For proposal 2 option 1

    - Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| … |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| nnpfc_base_flag | u(1) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ( nnpfc_purpose & 0x20 ) != 0 ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ( nnpfc_purpose & 0x04 ) != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( ( nnpfc_purpose & 0x08 ) != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the NNPF. The value of nnpfc_num_input_pics_minus1 shall be in the range of 0 to 63, inclusive. When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture. When nnpfc_purpose & 0x08 is equal to 0, the value of nnpfc_input_pic_output_flag[ i ] shall be equal to 1 for at least one i in the range of 0 to nnpfc_num_input_pics_minus1, inclusive.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the NNPF between the i-th and the ( i + 1 )-th picture used as input for the NNPF. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 63, inclusive. The value of nnpfc_interpolated_pics[ i ] shall be greater than 0 for at least one i in the range of 0 to nnpfc_num_input_pics_minus1 − 1, inclusive.

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture.

The variables numInputPics, specifying the number of pictures used as input for the NNPF, and numOutputPics, specifying the total number of pictures resulting from the NNPF, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_output_flag[ i ] )
		numOutputPics++
if( ( nnpfc_purpose & 0x08 )  !=  0 ) {
	for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
		if( nnpfc_input_pic_output_flag[ i ] )
			numOutputPics++
	for( i = 0; i  <=  numInputPics − 2; i++ )		(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else 
	numOutputPics = 1

…

## For proposal 2 option 2

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture. When nnpfc_purpose & 0x08 is equal to 0, the value of nnpfc_input_pic_output_flag[ 0 ] is inferred to be equal to 1 and the value of nnpfc_input_pic_output_flag[ i ] for i in the range from 1 to nnpfc_num_input_pics_minus1, inclusive, is inferred to be equal to 0.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).