| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0155 |
| --- | --- |

| Title: | Non-EE2: High-Accuracy template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents a method of high-accuracy template matching with two aspects. In aspect #1, an additional refinement is applied to TM for bi-prediction. In aspect #2, the diamond search pattern used in TM is modified from 8 points to 16 points. On top of ECM-8.0, simulation results of the proposed method are reported as below:

RA: {-0.06%, -0.09%, -0.07%; 100%, 100%};

LB: {-0.14%, -0.08%, -0.03%; 100%, 100%}.

Introduction

In ECM, template matching (TM) is a decoder-side MV derivation method to refine the motion information of the current CU by finding the closest match between a template (i.e., top and/or left neighbouring blocks of the current CU) in the current picture and a block (i.e., same size to the template) in a reference picture.

Figure 1. Template matching performs on a search area around initial MV.

When TM is used for bi-prediction, the following steps are applied:

- The initial motion vector of list 0 (MV0) is refined to derive a refined MV (MV’0) and a TM cost C0 is calculated;

- The initial motion vector of list 1 (MV1) is refined to derive a refined MV (MV’1) and a TM cost C1 is calculated;

- When C0 is larger than C1, MV’1 is used to derive a further refined MV (MV’’0) by refining MV’0; Otherwise, MV’0 is used to derive a further refined MV (MV’’1) by refining MV’1 .

The coding performance of TM for bi-prediction could be improved using an additional refinement.

Proposed method

A method of high-accuracy template matching with two aspects is proposed to improve template matching. In aspect #1, an additional refinement is applied to TM for bi-prediction. In aspect #2, the diamond search pattern used in TM is modified from 8-point to 16-point.

Aspect #1:

An additional refinement step is applied after step 3) as below:

- When MV’0 is refined in step 3), MV’’0 is used to derive MV’’1 by refining MV’1; Otherwise, MV’’1 is used to derive MV’’0 by refining MV’0.

Aspect #2:

The 8-point diamond search pattern used in TM is modified to 16-point diamond search pattern, as shown in Figure 2.

Figure 2. 16-point diamond search pattern for TM.

In addition, TM for bi-prediction is also enabled when DMVR condition is satisfied.

Simulation results

The proposed method is implemented on top of ECM-8.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results compared to ECM-8.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-8.0

|  | Random access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.19% | -0.17% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.07% | -0.19% | 0.04% | 99% | 100% |  |  |  |  |  |
| Class B | -0.05% | -0.01% | -0.06% | 100% | 100% | -0.14% | -0.51% | 0.07% | 100% | 100% |
| Class C | -0.07% | -0.07% | -0.08% | 101% | 101% | -0.17% | 0.14% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  | -0.11% | 0.36% | 0.05% | 101% | 100% |
| Overall | -0.06% | -0.09% | -0.07% | 100% | 100% | -0.14% | -0.08% | -0.03% | 100% | 100% |
| Class D | -0.07% | -0.06% | -0.17% | 101% | 101% | -0.06% | -0.13% | -0.07% | 103% | 100% |
| Class F | -0.07% | -0.11% | -0.09% | 100% | 100% | 0.15% | -0.01% | 0.90% | 101% | 99% |

Conclusions

In this contribution, two aspects are proposed for high-accuracy template matching. It is recommended to study the proposed method in EE2.

Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).