| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0156 |
| --- | --- |

| Title: | EE1-1.1: Complexity Reduction on Neural-Network Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P Badya Tong Shao, Arjun Arora, Peng Yin, Fangjun Pu, Taoran Lu, Sean McCarthy | Email: | {jay.shingala, }@ittiam.com {tshao, pyin}@dolby.com |
| Source: | Ittiam Systems, Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of EE1-1.1 based on JVET-AC0106. In this round of EE1-1.1 tests, the focuses are on training crosscheck and SADL Int16 implementation. EE1-1.1 contains two subtests. In EE1-1.1.1, the model with CP decomposition plus fusing adjacent 1x1 convolution is tested. In EE1-1.1.2 (if time permits), the model with CP decomposition, fusing adjacent 1x1 convolution and split architecture for luma and chroma components is tested.

This contribution reports EE1-1.1.1 results. (EE1-1.1.2 results are reported in EE1-related JVET-AD0157.)

The AI and RA results of {Y, Cb, Cr} BD-Rate gain are as follows compared to NNVC4.0 anchor:

EE1-1.1.1: CP-decomposition with fusing, worst case block level complexity of 16.2 KMAC/Pixel

- SADL, fp32:  AI {-4.66%, -5.86%, -5.94%}, RA {-5.28%, -7.60%, -6.70%}

- SADL, int16:  AI {-xx%, -xx%, xx%}, RA {-xx%, -xx%, -xx%}

The training crosscheck is reported in JVET-ADxxxx by Ericsson.

# Introduction

In this contribution, two subtests defined in EE1-1.1 [1]: the CP fused model (EE1-1.1.1) and the CP fused model with split luma chroma (24L, 8C) (EE1-1.1.2) are implemented on top of NNVC common software NNVC-4.0. This EE is the continuation of EE1-1.10 as described in JVET-AC0106 [2]. It focuses on the two aspects: training crosscheck and Int16 SADL implementation.

The training scripts were provided to the crosschecker, and the training crosscheck for EE1.1.1 are reported in JVET-ADxxxx. Crosscheck results are not yet available for EE1-1.1.2.

This contribution reports experimental results using SADL float32 and int16 compared to NNVC-4.0 anchor [5] .

# Proposed methods

Figure 1 shows the application of CP-decomposition of 3x3 convolution layers and fusion of adjacent 1x1 pointwise convolutions of the baseline JVET-X0140 Model [3]. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R followed by fusion of adjacent 1x1 convolution:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

The block-level complexity in KMACs/Pixel is shown in Table 1.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

QStep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

Figure 1 CP Decomposition + fusion of 1x1 conv layers of JVET-X0140 Baseline Model (EE1-1.1.1)

Table 1 Block-level complexity measure in KMacs/Pixel

| Models | complexity in KMAC/Pixel |
| --- | --- |
| JVET-X0140 baseline | 33.6 |
| Baseline CP + fusion (EE1-1.1.1) | 16.2 |
| Split(24L, 8C) CP + fusion (EE1-1.1.2) | 17.7 |

JVET-AC0106 implemented SADL float32 and int32. For SADL int16, the following changes are made to reduce the weights with very large magnitudes. The convolution weights and bias are normalized by scaling factors which are adjusted across layers such that the output is mathematically identical for SADL floating point operation. After this process, the maximum weight of each layer will be equal to geometric mean across each layer’s maximum weight. SADL int16 performance results show minimal coding loss with this approach. The script provided in the repository does this normalization before conversion to int16 models.

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI and RA configurations [5]. BD-Rate results compared to the NNVC-4.0 anchor are reported.

Tables 2-5 show the AI and RA performances of CP fused model in EE1-1.1.1 compared against NNVC-4.0 anchor. SADL fp32, SADL int16 results are reported respectively. Note that the SADL fp32 and SADL int16 encode and decode times are higher due to lack of SIMD acceleration in the current SADL library.

Table 2 AI performance of EE1-1.1.1 (SADL, fp32, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.54% | -3.85% | -4.93% | 134% | 7623% |
| Class A2 | -4.18% | -6.05% | -5.50% | 143% | 5965% |
| Class B | -4.14% | -6.45% | -6.30% | 127% | 6038% |
| Class C | -4.61% | -7.33% | -7.21% | 127% | 4619% |
| Class E | -6.20% | -4.73% | -5.11% | 138% | 7139% |
| Overall | -4.66% | -5.86% | -5.94% | 132% | 6070% |
| Class D | -5.04% | -5.84% | -6.94% | 120% | 4700% |

Table 3 RA performance of EE1-1.1.1 (SADL, fp32, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.67% | -4.51% | -4.67% | 153% | 10661% |
| Class A2 | -5.60% | -6.25% | -4.68% | 147% | 10120% |
| Class B | -4.87% | -8.12% | -7.57% | 155% | 11256% |
| Class C | -5.24% | -10.27% | -8.63% | 139% | 10487% |
| Class E |  |  |  |  |  |
| Overall | -5.28% | -7.60% | -6.70% | 149% | 10696% |
| Class D | -6.47% | -9.31% | -9.18% | 138% | 11163% |

Table 4  AI performance of EE1-1.1.1 (SADL, int16, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

Table 5 RA performance of EE1-1.1.1 (SADL, int16, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |

# Training and inference information

The models are trained based on DIV2K [6] for AI and BVI-DVC [4] for RA. The raw DIV2K image data was compressed using AI configurations to create the training examples for AI. The raw video data was compressed using RA configurations to create the training examples where Intra model is trained with DIV2K image data. As in JVET-X0140 EE1-1.4.1, joint models are used for luma and chroma components. 2 models are used for Intra and 2 models are used for Inter. 1st stage RA training dataset was generated with trained intra models. 2nd stage RA training dataset was generated using trained intra models and inter models from 1st stage RA training.

For SADL int16, the model parameters are quantized to int16 precision, and the inference is also performed on the int16 precision basis.

More information related to the training & inference of models can be found in Table 6 and Table 7 below.

Table 6  Training information of baseline and split (24L, 8C) model + CP decomposition + fusion

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX3090 |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 400 for AI 40 for RA stage 1 30 for RA stage 2 |
|  | Batch size: | 16 |
|  | Training time: | Baseline CP Fused: - 125 hours for AI - 200 hours for RA stage 1 - 150 hours for RA stage 2 Split (24L, 8C) CP Fused: - 140 hours for AI - 240 hours for RA stage 1 - 180 hours for RA stage 2 |
|  | Training data information: | DIV2K(AI), BVI-DVC(RA) |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | AI: - 2E-4 0-300 epochs - 2E-5 300-350 epochs - 2E-6 350-400 epochs RA stage 1: - 2E-4 0-20 epochs - 1E-4 20-30 epochs - 2E-5 30-35 epochs - 2E-6 35-40 epochs RA stage 2: - 2E-4 0-10 epochs - 1E-4 10-20 epochs - 2E-5 20-25 epochs - 2E-6 25-30 epochs |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 7  Inference information of baseline and split (24L, 8C) model + CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Baseline CP Fused |  | Split LC (24L, 8C) CP Fused |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type | CPU only |  |  |
|  | Framework: | SADL float, SADL int16 |  |  |
|  | Number of GPUs per Task | 0 |  |  |
|  | Number of Parameters (Each Model) | 52566 |  | 57014 |
|  | Total Parameter Number | 105132 |  | 114028 |
|  | Parameter Precision (Bits) | Floating (32 bits) / integer (16 bits) |  |  |
|  | Memory Parameter (MB) | 0.411 |  | 0.435 |
|  | MAC/Pixel (Kilo) | 16.2 |  | 17.7 |
| Optional |  |  |  |  |
|  | Total Conv. Layers | 46 |  | 88 |
|  | Total FC Layers | 0 |  |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Other information: |  |  |  |

# Conclusion

This contribution shows the potential of using CP decomposition plus fusion of 1x1 convolutional layer to achieve computational complexity reduction by approximately half compared to the JVET-X0140 low complexity model. RA luma gain over NNVC-4.0 anchor is about 5.28% with worst case complexity of 16.2 KMAC/Pixel for CP decomposition fused model in EE1-1.1.1. For AI case, the luma gain is about 4.66% for EE1-1.1.1. The coding performance results for the SADL int16 implementation are consistent with the results for the SADL float32 implementation.

The contribution proposes to adopt the NNLF low complexity filter as tested in EE1-1.1.1 into NNVC software.

# References

- E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang and Z. Xie, “Exploration Experiments on Neural Network-based Video Coding (EE1)”, JVET-AC2023, teleconference, Jan. 2022

- J. N. Shingala, S. Kadaramandalgi, A. Shyam, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “EE1-1.10: Complexity Reduction on Neural-Network Loop Filter”, JVET-AC0106, teleconference, Jan. 2022

- H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity”, JVET-X0140, teleconference, Oct 2021

- Di Ma, Fan Zhang, and David, R. Bull, "BVI-DVC: A Training Database for Deep Video Compression", arXiv preprint arXiv:2003.13552 (2020)

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AC2016, Jan. 2022

- R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool, https://data.vision.ee.ethz.ch/cvl/DIV2K/

- T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. Badya “EE1-1.1 related: Complexity Reduction on Neural-Network Loop Filter”, JVET-AD0157, Antalya, TR, April 2023

# Acknowledgement

We would like to thank Ericsson for crosschecking the training and inference results.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).