| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0157-v1 |
| --- | --- |

| Title: | EE1-related: Neural-network loop filters in EE1-1.1.2 with further complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tong Shao, Arjun Arora, Peng Yin, Fangjun Pu, Taoran Lu, Sean McCarthy Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P Badya | Email: | {tong.shao, pyin}@dolby.com {jay.shingala}@ittiam.com |
| Source: | Dolby Laboratories, Inc., Ittiam Systems |  |  |

_____________________________

# Abstract

This contribution reports the experimental results of EE1-1.1.2, CP fused model with split luma chroma (24L, 8C), as described in JVET-AC0106, as well as three even lower complexity neural-network loop filters based on the split luma and chroma model in EE1-1.1.2. In EE1-1.1.2, the model with CP decomposition, fusing adjacent 1x1 convolution and split architecture for luma and chroma components is tested. The model has a complexity of 17.7 KMAC/Pixel. To further explore the trade-off between coding performance and complexity of the split luma-chroma model, three even lower complexity variations are proposed and tested. The variations involve: 1) reduced number of luma and/or chroma channels; 2) CP decomposed first and last convolutional layer; and 3) reduced number of luma/chroma path hidden layers.

The architecture and parameter settings of the EE1-1.1.2 (24L, 8L) baseline model and three variations are as follows:

- (24L, 8C, 10H): 24 luma channels, 8 chroma channels, 10 hidden layers, with worst case block level complexity of 17.7KMAC/Pixel

- (16L, 8C, 10H): 16 luma channels, 8 chroma channels, 10 hidden layers, first conv. layer CP decomposed, with worst case block level complexity of 9.9 KMAC/Pixel

- (16L, 8C, 6H): 16 luma channels, 8 chroma channels, 7 hidden layers, first conv. layer CP decomposed, with worst case block level complexity of 7.3 KMAC/Pixel

- (12L, 4C, 10H): 16 luma channels, 8 chroma channels, 10 hidden layers, first conv. layer CP decomposed, with worst case block level complexity of 4.9 KMAC/Pixel

The AI and RA results of {Y, Cb, Cr} BD-Rate gain are as follows compared to NNVC-4.0 anchor:

- (24L, 8C, 10H): fp32: AI {-4.99%, -7.46%, -7.03%}, RA {-5.66%, -9.30%, -7.63%}

- (16L, 8C, 10H): fp32: AI {-4.07%, -7.97%, -7.99%}, RA {-4.55%, -8.85%, -10.42%}

- (16L, 8C, 7H): fp32: AI {-3.54%, -6.07%, -5.85%}, RA {-%, -%, -%}

- (12L, 4C, 10H): fp32: AI {-3.38%, -3.74%, -3.43%}, RA {-%, -%, -%}

# Introduction

In this contribution, the experimental results of EE1-1.1.2 [1] CP fused model with split luma chroma (24L, 8C) as described in JVET-AC0106 [2] are reported. Additional experimental results are reported for three even lower complexity variations. All additional model variants have fewer luma and/or chroma channels and the first and last 3x3 convolutional layer in the common path is also CP decomposed. In one variant, the number of luma/chroma path hidden layers is reduced from 10 to 6.

The models are implemented on top of NNVC common software NNVC-4.0. The results are reported compared to the NNVC-4.0 anchor.

# Proposed methods

Figure 1 shows the split luma chroma architecture combined with CP decomposition and fusing of adjacent 1x1 conv layer.[3]. The 3x3 convolutions of each hidden layer are decomposed into 4 layers with rank R followed by fusion of adjacent 1x1 convolution:

- 1st layer: 1x1xKxR pointwise convolution

- 2nd layer: 3x1xRxR separable convolution

- 3rd layer: 1x3xRxR separable convolution

- 4th layer: 1x1xRxK pointwise convolution

Figure 1 Split luma and chroma model + CP Decomposition + fusion of 1x1 conv layer (EE1-1.1.2)

The three even lower complexity model variants are based on the EE1-1.1.2 (24L, 8C) baseline, in which the luma path has 24 channels while the chroma path has 8 channels, and the number of hidden layers is 10, as shown in figure 1. Compared to EE1-1.12, the three variations have reduced number of luma and/or chroma channels and the first 3x3 convolutional layer in the common path is also CP decomposed. For one variant, the number of hidden layers is reduced from 10 to 7.

The architecture and parameter settings of the EE1-1.1.2 (24L, 8L) baseline model and three variations as well as their corresponding worst case block level complexity are listed in Table 1.

Table 1 Model architecture and block-level complexityin KMacs/Pixel

| Models | Num. of luma channels | Num. of chroma channels | Num. of luma/chroma path hidden layers | Is first and last conv. layer CP decomposed? | Complexity in KMAC/Pixel |
| --- | --- | --- | --- | --- | --- |
| (24L, 8C), EE1-1.1.2 baseline | 24 | 8 | 10 | no | 17.7 |
| (16L, 8C, 10H) | 16 | 8 | 10 | yes | 9.9 |
| (16L, 8C, 7H) | 16 | 8 | 6 | yes | 7.3 |
| (12L, 4C, 10H) | 12 | 4 | 10 | yes | 4.9 |

# Simulation results

Simulations are performed based on the common test conditions for neural-network-based video coding technology for AI and RA configurations [5]. BD-Rate results compared to the NNVC-4.0 anchor are reported.

Tables 2-9 show the AI and RA performances of the EE1-1.1.2 (24L, 8L) baseline model and three variations compared against NNVC-4.0 anchor. SADL fp32 is reported.

Table 2 AI performance of EE1-1.1.2 (SADL, fp32, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -4.62% | -5.80% | -6.37% | 143% | 6773% |
| Class A2 | -4.53% | -8.11% | -5.94% | 147% | 5218% |
| Class B | -4.43% | -7.76% | -7.39% | 132% | 5319% |
| Class C | -5.03% | -8.00% | -7.70% | 129% | 3977% |
| Class E | -6.66% | -7.27% | -7.30% | 144% | 6342% |
| Overall | -4.99% | -7.46% | -7.03% | 137% | 5329% |
| Class D | -5.48% | -6.80% | -8.58% | 130% | 4105% |

Table 3 RA performance of EE1-1.1.2 (SADL, fp32, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.85% | -6.90% | -6.23% | 159% | 8199% |
| Class A2 | -6.01% | -9.04% | -5.40% | 151% | 7302% |
| Class B | -5.28% | -9.85% | -8.88% | 161% | 8623% |
| Class C | -5.74% | -10.59% | -8.78% | 146% | 7859% |
| Class E |  |  |  |  |  |
| Overall | -5.66% | -9.30% | -7.63% | 155% | 8056% |
| Class D | -6.94% | -10.04% | -10.44% | 145% | 7865% |

Table 4 AI performance of (16L, 8C, 10H) (SADL, fp32, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.89% | -5.87% | -7.12% |  | 2767% |
| Class A2 | -3.80% | -8.38% | -6.79% |  | 2163% |
| Class B | -3.68% | -8.40% | -8.68% |  | 2207% |
| Class C | -3.96% | -8.75% | -8.56% |  | 1666% |
| Class E | -5.30% | -7.88% | -8.13% |  | 2600% |
| Overall | -4.07% | -7.97% | -7.99% |  | 2205% |
| Class D | -4.43% | -8.05% | -9.49% |  | 1726% |

Table 5 RA performance of (16L, 8C, 10H) (SADL, fp32, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -5.03% | -5.66% | -8.92% |  | 4318% |
| Class A2 | -4.88% | -7.81% | -7.64% |  | 3925% |
| Class B | -4.17% | -9.85% | -11.71% |  | 4563% |
| Class C | -4.42% | -10.78% | -12.04% |  | 4222% |
| Class E |  |  |  |  |  |
| Overall | -4.55% | -8.85% | -10.42% |  | 4289% |
| Class D | -5.36% | -9.89% | -12.97% |  | 4397% |

Table 6 AI performance of (16L, 8C, 7H) (SADL, fp32, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.23% | -4.73% | -5.24% |  |  |
| Class A2 | -3.27% | -6.50% | -5.05% |  |  |
| Class B | -3.20% | -6.42% | -6.40% |  |  |
| Class C | -3.57% | -6.52% | -6.16% |  |  |
| Class E | -4.66% | -5.82% | -5.93% |  |  |
| Overall | -3.54% | -6.07% | -5.85% |  |  |
| Class D | -4.08% | -6.02% | -6.62% |  |  |

Table 7 RA performance of (16L, 8C, 7H) (SADL, fp32, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -3.41% | -8.10% | -6.64% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -4.24% | -7.97% | -7.98% |  |  |

Table 8 AI performance of (12L, 4C, 10H) (SADL, fp32, NNVC-4.0 anchor)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -3.06% | -2.91% | -3.23% |  | 7053% |
| Class A2 | -3.22% | -3.90% | -2.73% |  | 5485% |
| Class B | -3.09% | -3.94% | -3.77% |  | 5579% |
| Class C | -3.29% | -4.39% | -3.63% |  | 4145% |
| Class E | -4.44% | -3.18% | -3.52% |  | 6637% |
| Overall | -3.38% | -3.74% | -3.43% |  | 5574% |
| Class D | -3.67% | -3.79% | -3.74% |  | 4283% |

Table 9 RA performance of (12L, 4C, 10H) (SADL, fp32, NNVC-4.0 anchor)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  | 7292% |
| Class A2 |  |  |  |  | 6453% |
| Class B |  |  |  |  | 7610% |
| Class C | -3.23% | -5.59% | -2.16% |  | 6928% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  | 7120% |
| Class D | -3.79% | -5.30% | -2.28% |  | 6898% |

# Training and inference information

The models are trained based on DIV2K [6] for AI and BVI-DVC [4] for RA. The raw DIV2K image data was compressed using AI configurations to create the training examples for AI. The raw video data was compressed using RA configurations to create the training examples where Intra model is trained with DIV2K image data. As in JVET-X0140 EE1-1.4.1, joint models are used for luma and chroma components. 2 models are used for Intra and 2 models are used for Inter. 1st stage RA training dataset was generated with trained intra models. 2nd stage RA training dataset was generated using trained intra models and inter models from 1st stage RA training.

More information related to the training & inference of models can be found in the Table 10 and Table 11 below.

Table 10  Training information of split models + CP decomposition + fusion

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX3090 |
|  | Framework: | Tensorflow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 400 for AI 40 for RA stage 1 30 for RA stage 2 |
|  | Batch size: | 16 |
|  | Training time: | - 140 hours for AI - 240 hours for RA stage 1 - 180 hours for RA stage 2 |
|  | Training data information: | DIV2K(AI), BVI-DVC(RA) |
|  | Training configurations for generating compressed training data (is different to VTM CTC): | QP=20,25,30,35,40,45 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |
|  | Learning rate: | AI: - 2E-4 0-300 epochs - 2E-5 300-350 epochs - 2E-6 350-400 epochs RA stage 1: - 2E-4 0-20 epochs - 1E-4 20-30 epochs - 2E-5 30-35 epochs - 2E-6 35-40 epochs RA stage 2: - 2E-4 0-10 epochs - 1E-4 10-20 epochs - 2E-5 20-25 epochs - 2E-6 25-30 epochs |
|  | Optimizer: | ADAM |
|  | Loss function: | Weighted L2 |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Other information: |  |

Table 11  Inference information of split models + CP decomposition + fusion

| Network Information in Inference Stage |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | (24L, 8C) EE1-1.1.2 | (16L,8C,10H) All layers CP Fused | (16L,8C,7H) All layers CP Fused | (12L,4C,10H) All layers CP Fused |
| Mandatory | HW environment: |  |  |  |  |
|  | GPU Type | CPU only |  |  |  |
|  | Framework: | SADL float, SADL int16 |  |  |  |
|  | Number of GPUs per Task | 0 |  |  |  |
|  | Number of Parameters (Each Model) | 57014 | 32430 | 23790 | 16270 |
|  | Total Parameter Number | 114028 | 64860 | 47580 | 32540 |
|  | Parameter Precision (Bits) | Floating (32 bits) / integer (16 bits) |  |  |  |
|  | Memory Parameter (MB) | 0.435 | 0.247 | 0.181 | 0.124 |
|  | MAC/Pixel (Kilo) | 17.7 | 9.88 | 7.27 | 4.89 |
| Optional |  |  |  |  |  |
|  | Total Conv. Layers | 88 | 97 | 67 | 97 |
|  | Total FC Layers | 0 |  |  |  |
|  | Total Memory (MB) |  |  |  |  |
|  | Batch size: |  |  |  |  |
|  | Patch size | 72x72 (4 luma and 2 chroma planes) |  |  |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |  |
|  | Other information: |  |  |  |  |

# Conclusion

This contribution reports coding performance and complexity results for EE1-1.1.2 (split luma-chroma model combining with CP decomposition plus fusion of 1x1 convolutional layer). The EE1-1.2 complexity reported is approximately half of the complexity of the JVET-X0140 low complexity model (EE1-1.4.1). The AI luma gain is -4.99% while the RA luma gain is -5.60% with worst case complexity of 17.7 KMAC/pixel for (24L,8C) split. The contribution also explores three even lower complexity variations of the split luma chroma model with different number of hidden layer and/or first convolution layer CP decomposed. Experimental results show that the split luma chroma model can also achieve good gain vs. complexity trade-off at lower complexities: 9.9 KMAC/Pixel, 7.3 KMAC/Pixel and 4.9 KMAC/Pixel.

The contribution proposes to further study the split luma chroma models in the next EE.

# References

- E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, L. Wang and Z. Xie, “Exploration Experiments on Neural Network-based Video Coding (EE1)”, JVET-AC2023, teleconference, Jan. 2022

- J. N. Shingala, S. Kadaramandalgi, A. Shyam, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy, “EE1-1.10: Complexity Reduction on Neural-Network Loop Filter”, JVET-AC0106, teleconference, Jan. 2022

- H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity”, JVET-X0140, teleconference, Oct 2021

- Di Ma, Fan Zhang, and David, R. Bull, "BVI-DVC: A Training Database for Deep Video Compression", arXiv preprint arXiv:2003.13552 (2020)

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-AC2016, Jan. 2022

- R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool, https://data.vision.ee.ethz.ch/cvl/DIV2K/

# Acknowledgement

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).