| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0159-v1 |
| --- | --- |

| Title: | AHG9: Explicit identification of NNPF input pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Martin Pettersson Mitra Damghanian | Email: | rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution claims that JVET-AC2005-v1 specifies the input pictures for the neural-network post-filter based on their order of output from the decoder such that a discarded or lost coded input picture may not be detected. The contribution proposes to add NNPFC picture identifier syntax elements aiming to support such detection.

In an option a, the contribution proposes to add a syntax element for each input picture as follows:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_delta_pic_identifier_minus1[ i ] | ue(v) |
| … |  |

In an option b, the contribution proposes to add a single picture identifier delta syntax element for the NNPF input picture that precedes all other input pictures in output order as follows:

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( nnpfc_num_input_pics_minus1 > 0 ) |  |
| nnpfc_delta_pic_identifier_minus1 | ue(v) |
| … |  |

The authors claim that the proposal will enable detection of lost or discarded coded pictures, that may happen due to removal of temporal layers or removal of non-reference pictures. Proposed specification text is included in the contribution. The authors claim that this text does not add any new flexibility regarding input picture ordering, only that the possibility to detect mismatch between the encoder and decoder NNPF input pictures is added.

# Problem Statement

The input pictures to the neural-network post-filter (NNPF) are currently specified in JVET-AC2005-v1 using an array inputPicPoc[ i ]. The PicOrderCntVal for the coded picture for which the NNPF is activated is put into the array as the first entry: inputPicPoc[ 0 ]. Then the PicOrderCntVal for the N closest preceding pictures in output order are added to the list in decreasing PicOrderCntVal order, where N is signalled as nnpfc_num_input_pics_minus1.

This design does not specify the input pictures to use. This means that the decoder would not detect whether removal of temporal layers or removal of non-reference pictures would affect the input pictures to use. This may cause a mismatch between the encoder and decoder regarding what input pictures to use as input to the filter, without any means for the decoder to detect the mismatch.

# Proposal

We propose that syntax elements for specifying the PicOrderCntVal for the NNPF input pictures are added. This will enable a decoder to detect any mismatch concerning NNPF input picture identifications.

In an option a, we propose to add a syntax element for each input picture that precedes, in output order, the picture (currCodedPic) for which the NNPFC SEI message is activated.

In an option b, we propose to add a single picture identifier delta syntax element for the NNPF picture that is earliest in output order.

The proposed text changes are presented below where added text is highlighted using green, and removed text is highlighted using yellow strikethrough.

Note that sections 8.28.1 and 8.28.2 are VSEI text and D.12.11 is VVC text.

# Proposed text

## Option a

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_delta_pic_identifier_minus1[ i ] | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| … |  |

…

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_delta_pic_identifier_minus1[ i ] plus 1 represents a picture identifier difference value between two NNPF input pictures. The value of nnpfc_delta_pic_identifier_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

…

### D.12.11	Use of the neural network post-filter characteristics SEI message

…

The array inputPicPoc[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the picture order count values of the input pictures for the NNPF, is derived as follows:

–	inputPicPoc[ 0 ] is set equal to PicOrderCntVal of currCodedPic.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of 1 to numInputPics − 1, inclusive, in increasing order of i:

–	inputPicPoc[ i ] is set equal to inputPicPoc[ i − 1 ] − nnpfc_delta_pic_identifier_minus1[ i − 1 ] − 1

–	If currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, inputPicPoc[ i ] is setshould be equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1 and is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	Otherwise (currCodedPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5), inputPicPoc[ i ] is setshould be equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1.

…

## Option b

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) |  |
| nnpfc_delta_pic_identifier_minus1 | ue(v) |
| if( ( nnpfc_purpose & 0x02 ) != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| … |  |

…

    - Neural-network post-filter characteristics SEI message semantics

…

nnpfc_delta_pic_identifier_minus1 plus 1 represents a picture identifier difference value between the current picture and the NNPF input picture that precedes all other input pictures in output order.

…

### D.12.11	Use of the neural network post-filter characteristics SEI message

…

The array inputPicPoc[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the picture order count values of the input pictures for the NNPF, is derived as follows:

–	inputPicPoc[ 0 ] is set equal to PicOrderCntVal of currCodedPic.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If currCodedPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1 and is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	Otherwise (currCodedPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5), inputPicPoc[ i ] is set equal to PicOrderCntVal of the picture that precedes, in output order, the picture associated with index i − 1.

–      nnpfc_delta_pic_identifier_minus1 should be equal to inputPicPoc[ 0 ] − inputPicPoc[ numInputPics – 1 ] – 1. If this is not the case, the pictures indicated by the array inputPicPoc[ i ] may not match the input pictures used by the encoder.

…

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).