| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0162-v3 |
| --- | --- |

| Title: | EE1-2.1-related: DRF Model without QP Input |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianghao Jia, Ding Ding, Wenhui Meng, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {jiajh2021, dingding, whmeng, zzchen}@whu.edu.cn {zizhengliu, xiaozhongxu, shanl}@tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution presents a refined approach to the deep reference frame (DRF) generation method tested in EE1-2.1, where the QP map input to the network is removed. Based on VTM-11.0_NNVC-4.0, the proposed method achieves {RA: -4.70%/-10.96%/-10.39%, LDB: -3.47%/-10.41%/-6.86%} bitrate savings for Y/U/V components, respectively. In addition, we also integrate the method into ECM-8.0 and provide partial results tested under RA configuration.

In the v3 version, the LDB overall results are fixed with ClassE included, and the ECM results are updated.

Introduction

In JVET-AC0114 [1], we proposed a deep reference frame generation method to enhance VVC inter prediction. The method was introduced into EE in the 29th JVET meeting. In this contribution, the QP map input to the DRF network is removed to further improve the performance.

Proposed Method

Fig. 1: The architecture of the DRF network

Fig. 1 shows the architecture of the proposed DFR network. The architecture is similar to the one in EE1-2.1 except without QP map input. The identical dataset (Vimeo90K) and strategy in EE1-2.1 are used to train the new models for RA and LDB configurations. In addition, we also integrated the proposed method into ECM. The models used in ECM are trained by Vimeo90K, which is compressed by ECM instead of VTM for EE1-2.1. The integration approaches for VTM-11.0_NNVC-4.0 and ECM-8.0 in this contribution are not modified. Other details can be found in our previous work.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 1080ti x 8 x 12GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 8 |
|  | Epoch | 200 |
|  | Batch size | 3Kx24 (RA) 4Kx18 (LDB) |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 168h |
|  | Training data information | Vimeo (compressed) |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | QP = 22, 27, 32, 37, 42 |
| Optional | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 7797K (RA) 11512K (LDB) |
|  | Total Number of Parameters (All Models) | 19309K |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 29.8 (RA) 44.0 (LDB) |
|  | Multiply Accumulate (kMAC/pixel) | 986 (RA) 1191 (LDB) |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size |  |
|  | Patch size | 512x512 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

We conducted the experiments according to the JVET NNVC CTC [2]. Table 3 and Table 4 show the experimental results of the proposed DRF method compared to the results of VTM-11.0_NNVC-4.0 under RA and LDB configurations, respectively. The comparison between this method and EE1-2.1 are shown in Table 5 and Table 6. Moreover, Table 7 shows the performance of the method based on ECM-8.0.

Table 3: Performance of the proposed method compared to VTM-11.0_NNVC-4.0 (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.66% | -8.03% | -8.12% | -4.13% | -9.36% | -8.96% | 157% | 65726% |
| Class A2 | -5.43% | -11.72% | -9.38% | -6.24% | -11.54% | -9.18% | 147% | 60992% |
| Class B | -4.07% | -10.32% | -9.84% | -4.74% | -11.16% | -10.77% | 159% | 79960% |
| Class C | -5.72% | -13.39% | -13.55% | -7.29% | -13.47% | -14.26% | 123% | 47989% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.70% | -10.96% | -10.39% | -5.60% | -11.49% | -11.02% | 146% | 63562% |
| Class D | -7.22% | -14.15% | -15.62% | -6.79% | -14.83% | -15.22% | 113% | 32008% |

Table 4: Performance of the proposed method compared to VTM-11.0_NNVC-4.0 (LDB, full length)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -2.88% | -10.18% | -5.40% | -3.29% | -9.14% | -4.46% | 175% | 100913% |
| Class C | -3.62% | -15.42% | -13.62% | -4.50% | -13.49% | -11.07% | 140% | 57999% |
| Class E | -4.25% | -4.12% | -0.29% | -4.18% | -2.28% | 4.71% | 310% | 158588% |
| Overall | -3.47% | -10.41% | -6.86% | -3.92% | -8.88% | -4.37% | 188% | 93940% |
| Class D | -3.94% | -13.33% | -12.30% | -3.84% | -9.29% | -8.68% | 128% | 37691% |

Table 5: Performance of the proposed method compared to EE1-2.1 (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-2.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.13% | -0.22% | 0.45% | 0.03% | -0.04% | 1.09% | 99% | 97% |
| Class A2 | -0.74% | 0.46% | -1.70% | -0.80% | 1.93% | -1.18% | 98% | 96% |
| Class B | -0.46% | -0.12% | 0.41% | -0.60% | 0.78% | 1.53% | 98% | 97% |
| Class C | -0.27% | -0.90% | -0.83% | -0.44% | -0.74% | -0.21% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.40% | -0.23% | -0.34% | -0.47% | 0.44% | 0.44% | 98% | 97% |
| Class D | -0.06% | -1.86% | -1.99% | -0.31% | -1.99% | -1.73% | 99% | 99% |

Table 6: Performance of the proposed method compared to EE1-2.1 (LDB, full length)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-2.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.32% | -0.31% | 2.11% | -0.05% | 2.03% | 5.75% | 99% | 99% |
| Class C | -0.28% | -1.23% | -1.86% | 0.12% | 0.52% | -1.82% | 99% | 98% |
| Class E | -0.43% | 2.11% | 2.11% | 0.37% | 1.84% | 5.41% | 98% | 99% |
| Overall | -0.33% | -0.01% | 0.79% | 0.11% | 1.48% | 3.14% | 99% | 99% |
| Class D | -0.31% | -4.22% | -2.72% | 0.34% | -5.96% | -3.42% | 99% | 99% |

Table 7: Performance of the proposed method compared to ECM-8.0 (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over ECM-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class B | -1.66% | -5.28% | -5.07% | -2.30% | -6.16% | -6.20% | 104% | 16059% |
| Class C | -2.38% | -6.83% | -7.26% | -3.96% | -7.71% | -8.64% | 94% | 9808% |
| Class D | -2.34% | -6.51% | -7.89% | -3.16% | -8.15% | -9.45% | 94% | 6377% |

Conclusions

This contribution presents a refined approach of the deep reference frame (DRF) generation method. Based on VTM-11.0_NNVC-4.0, our method achieves 4.70%/10.96%/10.39% and 3.47%/10.41%/6.86% coding efficiency improvements under RA and LDB configurations, respectively. By applying the proposed method to ECM-8.0, additional BD-rate saving can also be achieved.

Reference

[1] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11–20 January 2023.

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).