| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 30th Meeting: Antalya, TR, 21–28 April 2023 | Document: JVET-AD0165-v1 |
| --- | --- |

| Title: | EE2-2.4: Template matching-based subblock motion refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |  |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This proposal presents the performance of EE2-2.4. It is reported that on top of ECM-8.0, simulation results of EE2-2.4 are as below:

RA: -0.07 % (Y), -0.03 % (U), -0.01 % (V), 101% (EncT), 100% (DecT)

LB: -0.07 % (Y), 0.19 % (U), -0.03 % (V), 102% (EncT), 102% (DecT)

- Introduction

In ECM, affine and SbTMVP modes are used to provide subblock-level motion information. Affine mode can represent non-translational subblock motions based on control point motion vectors (CPMVs). Regarding SbTMVP mode, a motion shift is employed to obtain TMVPs at subblock level. In EE2-2.4, TM is utilized to refine subblock motions.

- Proposed methods

In EE2-2.4, two aspects are proposed to further improve subblock-based motion derivation. Firstly, after the candidate list is constructed for affine merge mode, uni-directional affine candidates will perform CPMV refinement based on TM. In particular, a unified MV offset is assigned to all the CPMVs of an affine candidate, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost can be used to refine the corresponding affine candidate if CRefined<T×COri, where CRefined  denotes the TM cost after CPMVs are refined by the optimal offset, COri is the TM cost calculated by the original CPMVs, and T is a constant threshold. Secondly, the initial motion shift to specify SbTMVP may be further refined with TM.

- Experimental results

Simulations are conducted on top of ECM-8.0 under the common test conditions [1], the results are listed as follows:

Table 1: Coding performance of Test 2.4.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.05% | -0.01% | 103% | 100% |  |  |  |  |  |
| Class A2 | -0.11% | -0.07% | 0.00% | 101% | 99% |  |  |  |  |  |
| Class B | -0.02% | 0.08% | 0.03% | 102% | 101% | -0.05% | -0.13% | -0.10% | 103% | 100% |
| Class C | -0.07% | -0.13% | -0.07% | 100% | 101% | -0.12% | 0.15% | -0.18% | 102% | 103% |
| Class E |  |  |  |  |  | -0.05% | 0.78% | 0.28% | 102% | 102% |
| Overall | -0.07% | -0.03% | -0.01% | 101% | 100% | -0.07% | 0.19% | -0.03% | 102% | 102% |
| Class D | 0.03% | 0.30% | 0.11% | 102% | 106% | -0.08% | -0.45% | -0.69% | 101% | 97% |
| Class F | -0.01% | -0.05% | 0.04% | 100% | 101% | 0.05% | -0.08% | 0.67% | 101% | 102% |

- Conclusions

This proposal presents the performance of EE2-2.4. Simulation results indicate promising coding gain in RA/LDB configurations with acceptable computational complexity. It is recommended to adopt this contribution in ECM.

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).