| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0166-v1 |
| --- | --- |

| Title: | EE1-1.7: Optimization of training and network for NNVC filter set 0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-AC0197. The proposed single-model filter in JVET-AC0197 is fine-tuned based on different training scripts for a better performance. Besides, a new combination network of resblock from filter set 0 and transformer block is designed to further improve the coding gains. Based on NNVC-4.0, the test results are shown in order of RA and AI configurations as follows.

Test 1.7.a (Test the fine-tuned model by updating the training datasets):

RA: -xx.xx%	-xx.xx%	-xx.xx% 	EncT: xxx%	DecT: xxxxx%

AI : -x.xx%	-xx.xx%	-xx.xx% 	EncT: xxx%	DecT: xxxxx%

Test 1.7.b (Test the model fine-tuned on new training datasets with additional data for I slices):

RA: -10.27%	-22.33%	-21.90%	EncT: 149%	DecT: 49252%

AI : -7.85%	-17.50%	-18.35%	EncT: 143%	DecT: 27781%

Test 1.7.c (Test the new combination network of resblock and transformer):

RA: -10.83%	-29.64%	-29.21%	EncT: 156%	DecT: 57869%

AI : -8.86%	-23.88%	-25.33%	EncT: 150%	DecT: 34139%

Introduction

In this contribution, the model from JVET-AC0197 [] is fine-tuned on different training scripts and a new combination network of resblock and transformer is proposed to further improve the performance. Specifically, the training dataset is updated by using the training scripts of NNVC-4.0 filter set #0 in Test 1.7.a. In Test 1.7.b, additional training data of I slices is introduced to fine-tune the filter set #0 model. In test 1.7.c, a new combination network of resblock and transformer is proposed.

Proposed method

The network architecture is firstly presented in Section 2.1 and Section 2.2, then the network information in both training stage and inference stage is provided in Section 2.3 and Section 2.4, respectively. Section 2.5 presents the related implementation details.

Network architecture of Test 1.7.a and Test 1.7.b

The network structure of Test 1.7.a and Test 1.7.b are the same as the network structure of NNVC-4.0 filter set #0. The network architecture is shown in Fig. 1.

Fig. 1: Network architecture of Test 1.7.a and Test 1.7.b

Network architecture of Test 1.7.c

In Test 1.7.c, a combination network of resblock and transformer is proposed. The architecture of combination network is shown in Fig. 2.

Fig. 2: Network architecture of Test 1.7.c

For the input part of network, the reconstructed image (rec_yuv) and some additional side information such as the prediction image (pred_yuv) [], slice QP, base QP and slice type are fed into the network. As for the structure of ResTransBlock, the transformer block is introduced into the resblock of filter set #0. Specifically, the ResTransBlock is composed of three convolution layers, one activation layer and one transformer block. The convolution and activation layers are the same as that of filter set #0, and the transformer block is placed after the last 3x3 convolutional layer to extract more effective features. K and M are set to 64 and 160, respectively. The channel numbers of transformer are also set to 64.

Network information in training stage

The training datasets of Test 1.7.a are regenerated by using the training scripts of NNVC-4.0 filter set #0 and the model of NNVC-4.0 filter set #0 is finetuned on the updated training datasets. Considering the coding QPs of I slices are different under RA and AI, additional training data of I slices under QP19, 24, 29, 34, 37 is introduced in Test 1.7.b.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 for test a/b, 2 for test c |
|  | Epoch: | ~200 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted L1 and L2 |
|  | Training time (for 1 model): | ~12 days |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In Test 1.7.a and Test 1.7.b, SADL [] is adopted as the inference library and all the models in these two tests are quantized into int16 precision. In Test 1.7.c, Libtorch is adopted as the inference library and the inference is based on float-point precision.

| - Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Test a and b | Test c |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL | Libtorch |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 1.9M | 1.64M |
|  | Total Number of Parameters (All Models) | 1.9M | 1.64M |
|  | Parameter Precision (Bits) | int16 | float |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all | 6.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) | 418K (w/o block extension) 529K (w/ block extension) |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers | 101 | 131 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- Multiple results are inferred by the common one model but with the different inputs {BaseQP, BaseQP - 5, BaseQP + 5} into the BaseQP interface of the network, where the index is decided at the slice-level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the experiments are performed based on the common test conditions in []. The test results of EE1-1.7.a, EE1-1.7.b and EE1-1.7.c are provided in Section 3.1, Section 3.2 and Section 3.3, respectively. For the convenient comparison, the test results of NNVC-4.0 (filter set #0) is provided in Section 3.4. The average results can also be summarized as follows.

EE1-1.7.a:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -xx.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |
| AI | -x.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |

EE1-1.7.b:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -10.27% | -22.33% | -21.90% | 149% | 49252% |
| AI | -7.85% | -17.50% | -18.35% | 143% | 27781% |

EE1-1.7.c:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -10.83% | -29.64% | -29.21% | 156% | 57869% |
| AI | -8.86% | -23.88% | -25.33% | 150% | 34139% |

NNVC-4.0 (filter set #0):

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.75% | -21.09% | -20.54% | 148% | 49115% |
| AI | -7.52% | -16.82% | -17.43% | 143% | 27772% |

Results of EE1-1.7.a

Table 1. Performance of EE1-1.7.a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -10.22% | -23.89% | -23.59% | 133% | 42842% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -11.47% | -22.97% | -24.27% | 131% | 38046% |
| Class F | -4.80% | -14.92% | -13.75% | 184% | 22023% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.19% | -15.38% | -17.96% | 190% | 37004% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -7.92% | -17.41% | -19.92% | 119% | 18934% |
| Class E | -10.28% | -17.13% | -19.05% | 144% | 30736% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -7.83% | -16.01% | -19.19% | 115% | 17654% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of EE1-1.7.b

Table 2. Performance of EE1-1.7.b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.68% | -19.03% | -21.65% | 160% | 52729% |
| Class A2 | -10.61% | -21.51% | -16.29% | 153% | 50443% |
| Class B | -9.71% | -22.85% | -23.21% | 154% | 51945% |
| Class C | -10.42% | -24.77% | -24.65% | 132% | 43005% |
| Class E |  |  |  |  |  |
| Overall | -10.27% | -22.33% | -21.90% | 149% | 49252% |
| Class D | -11.62% | -24.33% | -25.73% | 131% | 38077% |
| Class F | -4.91% | -14.96% | -13.98% | 183% | 22248% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.13% | -16.57% | -18.91% | 189% | 36618% |
| Class A2 | -6.93% | -17.66% | -14.43% | 147% | 30557% |
| Class B | -7.00% | -16.72% | -17.91% | 138% | 28386% |
| Class C | -8.12% | -18.53% | -20.58% | 119% | 18954% |
| Class E | -10.51% | -18.20% | -19.50% | 144% | 30782% |
| Overall | -7.85% | -17.50% | -18.35% | 143% | 27781% |
| Class D | -7.96% | -17.50% | -20.73% | 115% | 17592% |
| Class F | -4.86% | -13.64% | -13.28% | 119% | 22813% |

Results of EE1-1.7.c

Table 3. Performance of EE1-1.7.c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.27% | -24.91% | -27.57% | 164% | 53390% |
| Class A2 | -11.10% | -29.96% | -33.00% | 154% | 50971% |
| Class B | -10.49% | -34.90% | -30.25% | 160% | 64079% |
| Class C | -10.71% | -26.38% | -26.32% | 147% | 59524% |
| Class E |  |  |  |  |  |
| Overall | -10.83% | -29.64% | -29.21% | 156% | 57869% |
| Class D | -11.83% | -27.44% | -27.58% | 151% | 61881% |
| Class F | -5.40% | -17.69% | -16.72% | 204% | 30396% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.26% | -20.94% | -25.73% | 195% | 38893% |
| Class A2 | -8.08% | -26.68% | -28.64% | 150% | 31394% |
| Class B | -8.15% | -25.51% | -24.36% | 143% | 36812% |
| Class C | -8.67% | -21.96% | -24.08% | 127% | 26761% |
| Class E | -11.66% | -23.89% | -24.93% | 160% | 39762% |
| Overall | -8.86% | -23.88% | -25.33% | 150% | 34139% |
| Class D | -8.15% | -21.75% | -23.99% | 125% | 29439% |
| Class F | -5.65% | -16.74% | -16.20% | 122% | 30196% |

Results of NNVC-4.0 (filter set #0)

Table 4. Performance of NNVC-4.0 (filter set #0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.99% | -16.19% | -20.23% | 159% | 52385% |
| Class A2 | -10.19% | -19.85% | -15.94% | 151% | 50283% |
| Class B | -9.20% | -22.08% | -21.09% | 153% | 51929% |
| Class C | -9.92% | -24.44% | -23.53% | 132% | 42887% |
| Class E |  |  |  |  |  |
| Overall | -9.75% | -21.09% | -20.54% | 148% | 49115% |
| Class D | -10.97% | -23.77% | -23.47% | 131% | 37936% |
| Class F | -4.62% | -14.46% | -12.77% | 183% | 21918% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.99% | -15.27% | -17.94% | 189% | 36899% |
| Class A2 | -6.52% | -16.95% | -14.22% | 147% | 30412% |
| Class B | -6.67% | -16.85% | -17.02% | 137% | 28387% |
| Class C | -7.77% | -17.63% | -19.66% | 119% | 18935% |
| Class E | -10.11% | -17.09% | -17.84% | 144% | 30669% |
| Overall | -7.52% | -16.82% | -17.43% | 143% | 27772% |
| Class D | -7.69% | -16.96% | -19.44% | 115% | 17586% |
| Class F | -4.71% | -13.65% | -12.65% | 117% | 23214% |

Conclusions

In this contribution, the performance of NNVC-4.0 (filter set #0) is further improved. The models are fine-tuned by updating the training datasets. Compared with NNVC-4.0 (filter set #0), an additional 0.52% (from 9.75% to 10.27%) BD-rate luma gain is observed under RA configuration without any increase in both MAC result and memory size. Besides , a new combination network mainly composed of resTransBlock is proposed to further improve the performance. Compare with NNVC-4.0 (filter set #0), an additional 1.08% (from 9.75% to 10.83%) BD-rate luma gain is observed under RA configuration.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.1-related: More refinements on NN based in-loop filter. JVET-AC0197, 29th Meeting, by teleconference, 11–20 January 2023.

[] H. Zhu, X. Xu, S. Liu. Deep learning based in-loop filter with QP and residual distribution information. AVS-M5654, August 2020.

[] F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AB0086, 28th Meeting, Mainz, DE, 20–28 October 2022.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.