| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0170-v2 |
| --- | --- |

| Title: | EE1 related: Performance Improvement of AC0052 filter for RPR-based SR in RA configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shimin Huang Cheolkon Jung Yang Liu Ming Li | Email: | shimin_huang2022@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution aims to improve the performance of JVET-AC0052 filter for RPR-based super-resolution (SR) in RA configuration. The LMSDANet model for B-frames is trained with 10% bit-rate matching, and TVD and BVI-DVC datasets are used to generate B-frames. In BVI-DVC dataset, 200 2K and 200 4K video sequences are selected for training, while in TVD dataset all video sequences are used. They are compressed in RA configuration with enabled RPR functionality to generate a B-frame training set. At each GOP level, the encoder can adaptively select a scale factor from 1.0x and 2.0x, and the LMSDANet model is applied to the latter case. Compared with VTM-11.0_NNVC-2.0, the proposed CNN filter achieves overall {-2.01% (Y), -1.16% (U), -0.71% (V)} BD-rate gains in all classes, which shows performance improvement in RA configuration.

Introduction

JVET-AC0052 [1] shows coding gains by using the convolutional neural network (CNN)-based super-resolution method combined with existing RPR functionality. However, since we only use I-frame training models, the performance of the network in RA configuration is not good. Therefore, in this contribution, we use the generated B-frames to additionally train the model to improve its performance in RA configuration. For tests, we use two methods of JVET-AC0052 [1] and JVET-AC0196 [2] to achieve 10% bit-rate matching.

Proposed method

Network architecture

Fig. 1 shows LMSDANet for Y channel that consists of 8 MMSDABs, four convolutional layers, one concatenate layer, one shuffle layer and five shortcut connections. Except for the 1x1 and last convolutional layers, all convolutional layers and MMSDABs have 64 input and output channels. 1x1 convolution is used to reduce the number of channels and reduce the complexity of the operation. The output channel of the last convolutional layer is 4 so that the pixel shuffle layer can do 2.0x upsampling. Fig. 2 shows LMSDANet for U/V channel. The backbone of LMSDANet for U/V channel is consistent with the Y channel, and their main difference lies in the input and head of the network. For the LMSDANet applied to the U/V channel, its input contains three channels of Y, U, and V. The size of the guided component Y is twice of the U/V channel, thus the Y channel needs to be first downsampled. 3x3 convolution with stride 2 is chosen for downsampling. As shown in Fig. 3, LMSDAB is as the basic unit of LMSDANet. LMSDAB consists of three parts: the first is feature extraction part, the second is feature fusion part, and the third is attention enhancement part. The feature extraction part contains one 1x1 convolution layer and three 3x3 convolution layers. The feature fusion part concatenates features in the channel dimension and uses a 1x1 convolution layer for fusion and dimension reduction. The attention enhancement part uses MSSAB and CAB to enhance the fused features in both spatial and channel dimensions.

Fig. 1 Network architecture of the proposed LMSDANet for Y channel.

Fig. 2 Network architecture of the proposed LMSDANet for U/V channel.

Fig. 3 Network architecture of the modified LMSDAB.

As shown in Fig. 4, the multi-scale spatial attention is mainly composed of three spatial attention blocks, a concatenation layer and a convolution layer. The input of three SABs is respectively from the output of three branches of the feature extraction part in LMSDAB, and each proposed feature is subject to a spatial attention operation. Then, three spatial attention maps are concatenated together and go through a 3x3 convolution layer for fusion to obtain the final multi-scale spatial attention map.

Fig. 4 Network architecture of the MSSAB.

Model Generation for B-frames

To generate B-frames, we use TVD and BVI-DVC datasets, where we select 200 2K and 200 4K video sequences from BVI-DVC dataset with all video sequences from TVD dataset. These video sequences are compressed by VTM-11.0_NNVC-2.0 in RA configuration with enabled RPR functionality. To form a B-frame training set, some frames from the LR video output are selected as shown in Fig. 5.

Fig. 5 Process of B-frame training set generation.

Training and Inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed CNN filter testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel Core i9 12900k @3.9GHz |
|  | Framework: | LibTorch v1.8 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model: 1.13M/model chroma up-sampling model: 1.18M/model |
|  | Total Parameter Number | 23.15M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 20 models in total: 92.6 MB |
|  | Multiply Accumulate (MAC) | 964kMAC/pixel |
| Optional | Total Conv. Layers | 91 for up-sampling the luma, 92 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

DIV2K [3], BVI_DVC [4] and TVD [5] datasets are used for training. The official NNVC-2.0 anchor (VTM-11.0 [6]) is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by a factor of 2 before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training. We train a separate model for each QP for the three channels Y, U, V, thus there are 20 models in total. The models are fixed.

Table 2 Network information for the proposed CNN filter testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA 3090 24GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | luma:50, chroma:50 |
|  | Batch size: | 32 |
|  | Training time: | ~26h/model for I, ~50h/model for B |
|  | Training data information: | DIV2K, BVI-DVC & TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0_NNVC-4.0 , QP {22, 27, 32, 37, 42} |
|  | Loss function: | L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

First, we used VTM-11.0_NNVC-2.0 and combined it with the 10% bit-rate matching method proposed in JVET-AC0052 filter [1] to test the performance in RA configuration according to the common test conditions (CTC) [7] as shown in Table 3.

- In RA configuration, the proposed CNN filter achieves overall {-2.01% (Y), -1.16% (U), -0.71% (V)} BD-rate gains over VTM-11.0_NNVC-2.0 in all classes. In JVET-AC0052 [1], the proposed CNN filter achieved overall {-1.0% (Y), -0.2% (U), 0.5% (V)} BD-rate gains in RA configuration.

Table 3 BD rates in RA configuration over VTM-11.0_NNVC-2.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-2.0 |  |  |  |  |
| Sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.61% | -8.24% | -6.42% | 70% | #VALUE! |
| Class A2 | -3.50% | 2.10% | 4.12% | 65% | #VALUE! |
| Class B | 0.04% | 0.20% | -0.76% | 100% | #VALUE! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | #VALUE! |
| Class E |  |  |  |  |  |
| Average on A1 and A2 | -5.06% | -3.07% | -1.15% | 68% | #VALUE! |
| Overall | -2.01% | -1.16% | -0.71% | 85% | #VALUE! |

Second, we used the 10% bit-rate matching method proposed in JVET-AC0196 filter [2]. This 10% bit-rate matching method has been implemented in VTM-11.0_NNVC-4.0. BD-rates of the proposed CNN filter over VTM-11.0_NNVC-4.0 in RA configuration are shown in Table 4.

- In RA configuration, the proposed CNN filter achieves overall {-1.88% (Y), -1.48% (U), -0.58% (V)} BD-rate gains over VTM-11.0_NNVC-4.0 in all classes.

Table 4 BD rates in RA configuration over VTM-11.0_NNVC-4.0.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 NnlfOption=0 |  |  |  |  |
| Sequences | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -6.14% | -8.60% | -6.40% | 70% | #VALUE! |
| Class A2 | -3.23% | 1.21% | 3.59% | 65% | #VALUE! |
| Class B | 0.00% | 0.00% | 0.00% | 100% | #VALUE! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | #VALUE! |
| Class E |  |  |  |  |  |
| Average on A1 and A2 | -4.69% | -3.70% | -1.40% | 68% | #VALUE! |
| Overall | -1.88% | -1.48% | -0.58% | 85% | #VALUE! |

Conclusion

This contribution improves the performance of JVET-AC0052 filter in RA configuration by training the LMSDANet model for B-frames. However, the B-frame model increases the total number of LMSDANet models. Thus, our future study includes integrating different QP models into a single model to reduce the number of models, and making the proposed CNN filter lighter by depth separable convolution.

References

- S. Huang, C. Jung, Y. Liu, and M. Li, EE1-2.4: CNN filter Based on RPR-based SR Combined with GOP Level Adaptive Resolution, JVET-AC0052, 2023.

- R. Chang, L. Wang, X. Xu, S. Liu, EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution, JVET-AC0196, 2023.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/, R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- D. Ma, F. Zhang, and D. R. Bull, BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn).

- XU X, LIU S, LI Z. Tencent video dataset (tvd): A video dataset for learning-based visual data

compression and analysis[J]. arXiv preprint arXiv:2105.05961, 2021.

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- S. Liu, A. Segall, E. Alshina, and R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, 2021.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).