| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April. 2023 | Document: JVET-AD0172-v1 |
| --- | --- |

| Title: | AHG12: Adaptation of LFNST/NSPT for low-delay configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Fabrice Le Léannec Karam Naser Tangi Poirier Saurabh Puri | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable LFNST/NSPT in low-delay configuration with certain adaptation to balance the coding gain and run time. Specifically, it is proposed to activate LFNST/NSPT for the lower TiD slices in LDB configuration.

The following results are obtained compared to ECM-8.0 in LDB configuration:

Overall :	-0.59%	-1.03%	-1.16%	(Y Cb Cr)  102% (Enc)	98% (Dec)

# Introduction

In ECM CTC [2], LFNST/NSPT is enabled only for All Intra and Random Access configurations, although RD gains apply in low-delay scenarios as well. This is mainly because of the unbalanced trade-of between encoding time and coding gain. In this proposal, LFNST is enabled for LDB configuration.

To provide a better trade-off regarding encoder complexity and RD performance, LFNST is enabled for the lower temporal Id (TiD) slices only.

# Proposed Solution

It is proposed to limit LFNST/NSPT usage for slices with TiD lower than 3. A picture header flag is signaled to indicate LFNST/NSPT usage for certain pictures. If disabled, lfnst_idx is not signalled in the bitstream.

# Experimental Results

The proposed method is implemented on ECM-8.0 [1]. The simulations were performed following the CTCs [2]. We also display the results obtained by activating default LFNST for LDB configuration on both ECM-8.0 and VTM-11.0Ecm8 anchor used for ECM evaluation against VVC.

Table 1. Experimental results of default LFNST activated in LDB over ECM-8.0

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -1.24% | -1.32% | 111% | 103% |
| Class C | -0.95% | -1.17% | -0.79% | 107% | 100% |
| Class E | -0.65% | -0.51% | -1.44% | 104% | 101% |
| Overall | -0.63% | -1.04% | -1.18% | 108% | 101% |
| Class D | -0.22% | -0.79% | -1.46% | 107% | 100% |
| Class F | -0.67% | -1.03% | -0.61% | 107% | 102% |

Table 2. Experimental results of the proposed method in LDB over ECM-8.0

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -1.04% | -1.09% | 103% | 101% |
| Class C | -0.91% | -0.93% | -0.72% | 101% | 96% |
| Class E | -0.67% | -1.13% | -1.88% | 102% | 96% |
| Overall | -0.59% | -1.03% | -1.16% | 102% | 98% |
| Class D | -0.31% | -1.18% | -1.26% | 105% | 99% |
| Class F | -0.46% | -0.08% | 0.80% | 101% | 100% |

Table 3.Experimental results of LFNST activated in LDB over VTM-11.0Ecm8

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0Ecm8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.87% | -1.64% | 109% | 100% |
| Class C | -0.44% | -0.48% | -1.05% | 110% | 98% |
| Class E | -0.39% | -1.58% | -1.90% | 106% | 104% |
| Overall | -0.39% | -0.92% | -1.51% | 108% | 100% |
| Class D | -0.14% | -0.26% | -0.08% | 108% | 107% |
| Class F | -0.39% | -0.82% | -0.24% | 105% | 105% |

# Conclusion

We recommend further study on the proposed LFNST adaptation for low delay configurations by including it in EE2.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).