| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0173-v1 |
| --- | --- |

| Title: | EE2-1.22: Template-based intra MPM list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Jinrong Zhang (vivo) | Email: | chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the simulation results of EE2 Test 1.22. In Test 1.22, an intra most probable modes optimization method. Firstly, the intra modes derived from the neighbouring blocks and DIMD are sorted with SAD cost using a template, and added to the general MPM list in ascending order of SAD cost after the Planar mode. Then the remaining entries in the general MPM list are the sorted directional modes with added offset. There are two tests, the experimental results are summarized as follows:

On top of ECM-8.0

AI: -0.06% (Y), -0.08% (U), -0.09% (V),  100% (EncT),  101% (DecT)

RA : -0.05% (Y), 0.04% (U), 0.03% (V),  100% (EncT),  100% (DecT)

Introduction

In ECM8.0 [1], a general MPM list with 22 entries is constructed first, and then the first 6 entries in this general MPM list are included into the PMPM list, and the rest of entries form the SMPM list. The first entry in the general MPM list is the planar mode. The remaining entries are composed of the intra modes derived from neighbouring blocks and DIMD, the directional modes with added offset from the first two available directional modes of neighbouring blocks, and the default modes.

An intra most probable modes optimization method was proposed in JVET-AC0120 [2] and included as Test 1.22 in EE2 [3] in the 29th JVET meeting. This contribution reports the simulation results of EE2 Test 1.22.

Proposed method

In the test, the MPM list is constructed by the following three steps:

- The first entry in the general MPM list is always planar mode.

- The next entries are composed of the intra modes of neighboring blocks, and DIMD modes, which are sorted in ascending order of SAD cost. Up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template.

- The sorted directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed.

For the signaling part, MPM list is equally divided into four groups and the group index is parsed first. Then, a mode index is further parsed to indicate which mode in the selected group is used.

Simulation results

The proposed method is tested on top of ECM8.0 under the common test conditions [4].

Table 2. Simulation results of the proposed method

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.01% | -0.08% | 100% | 100% |
| Class A2 | -0.04% | 0.03% | 0.03% | 100% | 100% |
| Class B | -0.04% | -0.10% | -0.08% | 101% | 100% |
| Class C | -0.09% | -0.16% | -0.26% | 101% | 102% |
| Class E | -0.10% | -0.15% | -0.03% | 100% | 100% |
| Overall | -0.06% | -0.08% | -0.09% | 100% | 101% |
| Class D | -0.11% | -0.26% | -0.11% | 100% | 102% |
| Class F | -0.06% | -0.17% | -0.02% | 100% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.10% | -0.15% | 101% | 101% |
| Class A2 | -0.01% | 0.02% | 0.14% | 101% | 100% |
| Class B | -0.03% | -0.01% | 0.13% | 100% | 100% |
| Class C | -0.09% | 0.06% | -0.04% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.04% | 0.03% | 100% | 100% |
| Class D | -0.05% | 0.04% | -0.38% | 99% | 99% |
| Class F | -0.08% | -0.51% | -0.35% | 100% | 100% |

References

- M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, Jan. 2023.

- C. Zhou and et. al., “Non-EE2: template based intra MPM list construction,” JVET-AC0120, Jan 2023.

- V. Seregin and et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, Jan 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).