| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0175-v2 |
| --- | --- |

| Title: | AHG8/AHG9: On object mask auxiliary picture and object mask information SEI message for VSEI and HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Shurun Wang Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to add a new type of auxiliary picture to represent the object mask for the detected or tracked object within the compressed video bit-stream in VSEI and HEVC. To support the proposed object mask auxiliary picture, the associated SEI message called the object mask information (OMI) SEI message is also proposed to VSEI and HEVC.

Motivation

VSEI, HEVC and AVC adopted annotated region SEI message for the object detection and tracking applications, which are the common workload for end-to-end video systems. With the annotated region SEI message, the object detection and tracking task only needs to be performed in encoder-side and the results are sent to the decoder through the SEI message so that the decoder doesn’t need to perform the video analysis again. It has many advantages compared with performing those video tasks in decoder-side. First, the power consumption of the decoder can be reduced, and second the object detecting and tracking results will be more accurate as usually the encoder is more computationally powerful than the decoder, and the encoder can perform the video analysis on the original video instead of the compressed one which is what the decoder has to do.

Annotated region SEI uses a bounding box to describe the region covered by an object of interest, However, the detailed shape information of the object is not carried by the annotated region SEI message. In some applications, information about the object shape is essential. For example, in the intelligent traffic system, the shape information of people and vehicles is used to recognize the traffic accident. And in the video conferencing system, the shape information could help to switch the virtual background. To have detailed shape information signaled in the bitstream, mask information was proposed for annotated region SEI message in JVET-AF0024 [1] in July 2018. However, JVET-AF0024 proposes to send the mask itself (which is a binary picture) inside the SEI. So, it was commented at the JCTVC-AF meeting that “this is basically sending coded bi-level pictures inside an SEI message and that this seems like it is not what we would ordinarily do with SEI messages. It could also require a substantial payload.” And some experts suggest using auxiliary picture as the mask picture. As more video is being coded for machine analysis purposes, it is appropriate to revisit carrying mask information as part of the video bitstream because this allows important information obtained from machine tasks such as instance segmentation performed at the encoder to be sent to the decoder.

Thus, in this contribution, it is proposed to add a new auxiliary picture type for object mask in VSEI and HEVC, and the related SEI messages are also proposed.

Proposal

In this contribution, it is proposed to add a new auxiliary picture type for object mask. That is to say, the object mask is coded as an auxiliary picture and transmitted in the bitstream together with the associated primary picture.

Usually, the mask is a binary matrix wherein “0” represents background and “1” represents the foreground. But for the auxiliary picture with luma sample bit-depth equal to BitDepthY, there are 1<<BitDpethY different values for each sample. To distinguish different masks within one picture, in this contribution we use the sample value of auxiliary picture as the ID of the mask. That is to say, in the auxiliary picture, the samples with the same value form a mask. And the regions with different sample values represent different masks. For example, as shown in Figure 1, it is a 16x8 auxiliary picture, and numbers in the figure denote the sample values. So, there are four different masks in the auxiliary. The samples with value 5 (the blue region) form a mask with ID equal to 5; the samples with value 10 (the green region) form a mask with ID equal to 10; the samples with value 20 (the yellow region) form a mask with ID equal to 20; and the samples with value 0 (the while region) form a mask with ID equal 0 (the special mask could be labelled as “background”.

Figure 1 An example of masks representation in auxiliary picture

As the mask can be overlapped, multiple object mask auxiliary pictures (one auxiliary picture in one layer) can used for one primary picture to handle overlapping case. In that case, the samples with same position but in the different mask picture could belong to different masks overlapped with each other. For example, as shown in Figure 2, there are two 4x4 object mask auxiliary pictures. In the picture 0, the top-left 2x2 block is form a mask with ID 5, and in the picture 1, the center 2x2 block is form a mask with ID 10. When decoder receive these two auxiliary pictures, it is clearly that there are two masks being overlapped at position (1,1).

Another benefit of using multiple mask auxiliary pictures is that we can support more different masks. For example, for 10-bit the auxiliary picture, we can only support 1024 masks in one picture. But by using multiple mask auxiliary picture, we can support more than 1024 masks.

Figure 2 An example of overlapped mask case

To support object mask auxiliary picture, we also need to send related information. Thus, a new type SEI message named object mask information (OMI) SEI message is proposed in this contribution as well.

The proposed OMI SEI message use persistence mechanism. It only needs to be re-sent when there are any changes of the object masks. In the case of sending the OMI message, only the updated part of mask information needs to be sent.

Basically, the main information sent in OMI SEI message includes

- Depth information. In case two or more masks overlapped, the depth information is used to determine which object is visible and which object is occluded.

- Confidence. It represents the degree of the confidence of the object mask, which is inherited from annotated region SEI message.

- Label. Each mask can have a label to indicate what this object is and any other attribute of the object. This is also supported in annotated region SEI message

All these information is optionally signalled in SEI message, so we have three present flags before sending these information.

The syntax and the semantics of the proposed OMI SEI message for VSEI is given in section 2.1. And the syntax and the semantics of the proposed OMI SEI message for HEVC is given in section 2.2.

For VSEI, unspecified value of sdi_aux_id[i] is used for the type of object mask auxiliary picture which is indicated in the OMI SEI message. And for HEVC, unspecified value of AuxId[nuhLayerIdA] is used for the type of object mask auxiliary picture which is also indicated in the OMI SEI message.

For VVC, as we can support multiple primary picture layers with multiple auxiliary picture layers. So, in the proposed OMI SEI message, it is indicated that which primary picture layers this SEI is applied to. And from the SDI SEI message, it can be known that which auxiliary picture layers are associated with the primary picture layer to which this OMI SEI message is applied to. So we don’t signal auxiliary layer ID in OMI SEI message.

For HEVC, there is no SDI SEI message and VPS extension only tells you which layers are the primary picture layers and which layers are the auxiliary picture layers. So for each time when you update the auxiliary picture, we first indicate the layer ID of the auxiliary picture to be updated.

Syntax and semantics of the proposed OMI SEI message for VSEI

Table 1 syntax of OMI SEI message for VSEI

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if(!om_cancel_flag) { |  |
| //high level info |  |
| omi_aux_id_minus128 | ue(v) |
| omi_num_primary_pic_layer_minus1 | ue(v) |
| for(i=0; i<=omi_num_primary_pic_layer_minus1;i++) |  |
| omi_primary_pic_layer_id[i] | ue(v) |
| omi_mask_id_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if(omi_mask_label_language_present_flag) { |  |
| while(!byte_aligned()) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_lable_language | st(v) |
| } |  |
| } |  |
| // individual mask information |  |
| for (i=0; i<= omi_num_primary_pic_layer; i++) { |  |
| for(j=0;j< numAuxLayer[omi_primary_pic_layer_id[i]]; j++){ | u(1) |
| om_mask_pic_update_flag[ i ][ j ] | f(1) |
| if(omi_mask_pic_update_flag[ i ] [ j ]) { |  |
| om_num_mask_in_pic_update[ i ] [ j ] | ue(v) |
| for(k=0; k<omi_num_mask_in_pic_update[ i ] [ j ]; k++) { |  |
| omi_mask_id[ i ] [ j ] [ k ] | u(v) |
| if(maskIdExist[i][ j ][on_mask_id[ i ] [ j ] [ k ]]) { |  |
| omi_mask_cancel[ i ] [ j ] [ k ] | u(1) |
| maskIdExist[i] [ j ] [on_mask_id[ i ] [ j ] [ k ]]= !on_mask_cancel[ i ] [ j ] [ k ] |  |
| } |  |
| else { |  |
| maskIdExist[i] [ j ] [on_mask_id[ i ] [ j ] [ k ]]=1 |  |
| } |  |
| if(maskIdExist[i] [ j ] [on_mask_id[ i ] [ j ] [ k ]]) { |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ] [ j ] [ k ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ] [ j ] [ k ] | u(v) |
| while(!byte_aligned()) |  |
| omi_bit_equal_to_zero | f(1) |
| if(omi_mask_label_info_present_flag) |  |
| omi_mask_label[ i ] [ j ] [ k ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The variable numAuxLayer[i] indicated the number of the auxiliary picture layers associated with primary picture layer with nuh_layer_id equal to i. The variable associatedAuxLayer[j][i] indicates the value of nuh_layer_id of i-th auxiliary picture layer associated with primary picture layer with nuh_layer_id equal to j. numAuxLayer[i] and associatedAuxLayer[j][i] are derived by SDI SEI message as follows.

For(i=0; i<=sdi_max_max_layers_minus1;i++){

numAuxLayer[sdi_layer_id[i]]=0;

}

for(i=0; i<=sdi_max_max_layers_minus1;i++){

if(sdi_aux_id[i]== omi_aux_id_minus128+128){

for(j=0;j< sdi_num_associated_primary_layers_minus1[ i ];j++){

primaryLayerId= sdi_layer_id[sdi_associated_primary_layer_idx[ i ][ j ]];

associatedAuxLayer[primaryLayerId][NumAuxLayer[primaryLayerId]]= sdi_layer_id[i]

numAuxLayer[primaryLayerId]++;

}

}

}

The object mask information (OMI) SEI message provides information about object mask pictures coded as auxiliary pictures. Object mask auxiliary pictures have nuh_layer_id equal to nuhLayerIdA, sdi_layer_id[i] equal to nuhLayerIdA and sdi_aux_id [ i ] in the range of 128 to 159, inclusive, for any value of i in range of 0 to sid_max_layers_minus1, inclusive.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an object mask auxiliary layer by an OMI SEI message, a primary picture picB in a layer, with nuh_layer_id equal to nuhLayerIdB, that is indicated as an primary layer by the OMI SEI message, OMI SEI message persists in output order until one or more of the following conditions are true:

– A CLVS containing the auxiliary picture picA ends.

– A CLVS containing the primary picture picB ends.

– A CVS ends.

– The bitstream ends.

The value of sdi_aux_id[ sdi_associated_primary_layer_idx[ i ][ j ] ] shall be equal to 0.

Omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. Om_cancel_flag equal to 0 indicates that object mask information signaled in this SEI message.

Omi_aux_id_minus128 plus 128 indicates the value of sdi_aux_id of object mask auxiliary picture layer. Om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to om_aux_id_minus128 + 128 for at least one value of I, no picture in the CVS shall be associated with an OMI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to om_aux_id_minus128 + 128 for at least one value of I and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

Omi_num_primary_pic_layer_minus1 plus 1 indicates the number of primary picture layers associated with the object mask auxiliary picture layers to which this SEI message applies. The value of omi_num_primary_pic_layer_minus1 shall be in the range of 0 to sdi_max_layers_minus1.

Omi_primary_pic_layer_id[i] specifies the nuh_layer_id value of the i-th primary picture layer to which this OMI SEI message applies. The value of sdi_aux_id[j] shall be equal to 0 for any value of j in the range of 0 to sid_max_layers_minus1 so that sdi_layer_id[ j ] equal to omi_primary_pic_layer_id[i]

omi_mask_id_length_minus8 plus 8 indicates the number of bits used for coding omi_mask_id[ i ] [ j ][ k ] syntax elements.

Omi_mask_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ] [ j ] [ k ] syntax elements are present. Omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ] [ j ] [ k ]syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_mask_confidence_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

Omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ] [ j ] [ k ] syntax elements. It is a requirement of bitstream conformance that the value of omi_mask_confidence_length_minus1 shall be the same for all object_mask_info( ) syntax structures within a CLVS.

Omi_object_depth_info_present_flag equal to 1 indicates that omi_object_depth[ i ] [ j ] [ k ] syntax elements are present. Omi_object_depth_info_present_flag equal to 0 indicates that omi_object_depth[ i ] [ j ] [ k ]syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_object_depth_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

Omi_object_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_object_depth[ i ] [ j ] [ k ] syntax elements. It is a requirement of bitstream conformance that the value of omi_object_depth_length_minus1 shall be the same for all object_mask_info( ) syntax structures within a CLVS.

Omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ] [ j ] [ k ] are present. Omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ] [ j ] [ k ] are not present.

Omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_lable_language is present. Omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_lable_language is not present and that the language of the mask label is unspecified.

Omi_bit_equal_to_zero shall be equal to 0.

Omi_mask_lable_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_lable_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

Om_mask_pic_update_flag[ i ] [ j ] equal to 1 indicates the mask information of j-th object mask auxiliary picture associated with i-th primary picture is signalled. Om_mask_pic_update_flag[ i ] [ j ] equal to 0 indicates the mask information of j-th object mask auxiliary picture associated with i-th primary picture is not signalled. When the mask information of j-th object mask auxiliary picture associated with i-th primary picture is not present, the persistence mechanism is used, that is the information is inherited from the last OMI SEI message which signals the mask information of j-th object mask auxiliary picture associated with i-th primary picture.

Om_num_mask_in_pic_update[ i ] [ j ] indicates the number of object masks in the j-th auxiliary picture associated with i-th primary picture to be signalled. Omi_num_mask_in_pic[ i ] [ j ] shall be in the range of 0 to (1<<BitDepthY) – 1, inclusive, where BitDepthY is the bit depth for the samples of the luma component.

Omi_mask_id[ i ] [ j ] [ k ] indicates the identifier of k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture. The object mask identifier associated with the sample location (x, y) in the j-th object mask auxiliary picture is equal to p[ j ][ x ][ y ] where p[ j ][ x ][ y ] refers to the luma sample at location ( x, y ) in the decoded j-th object mask auxiliary picture.

The variable maskId[ i ] [ j ] [ k ]  specifying the object mask identifier of k-th object mask of the j-th object mask auxiliary picture associated with i-th primary picture in the SEI message is derived as follows,

for( i = 0; i <= omi_num_primary_pic_layer_minus1; i++ ) {

for( j = 0; j<= numAuxLayer[omi_primary_pic_layer_id[i]]; j++ ) {

for(k = 0; k <= on_num_mask_in_pic[ i ]; k++ ) {

maskId[ i ][ j ][ k ] += omi_mask_id[ i ] [ j ][k] * (j+1)

}

}

}

om_mask_cancel[ i ] [ j ] [ k ] equal to 1 cancels the persistence scop of object mask with identifier equal to om_mask_id[ i ] [ j ] [ k ]. Om_mask_cancel[ i ] [ j ] [ k ] equal to 0 indicates the information of object mask with identifier equal to om_mask_id[ i ] [ j ] is signaled.

The variable maskIdExist[ i ] [ j ] [ k ] equal to 1 indicates the object mask with identifier k in the j-th object mask auxiliary picture associated with i-th primary picture exists. The variable maskIdExist[ i ] [ j ] [ k ] equal to 0 indicates the object mask with identifier equal to k in the j-th object mask auxiliary picture associate with i-th primary picture does not exist. maskIdExist[ i ] [ j ] [ k ] is initialized with 0 before decoding the current CVS.

Omi_mask_confidence[ i ] [ j ] [ k ] indicates the degree of confidence associated with the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ] [ j ] [ k ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ] [ j ] [ k ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

Omi_mask_depth[ i ] [ j ] [ k ] indicates the object depth associated with the k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ] [ j ] [ k ] syntax element is omi_object_depth_length_minus1 + 1 bits.

Omi_mask_label[ i ] [ j ] [ k ] specifies the contents of the label associated with k-th object mask in the j-th object mask auxiliary picture associated with i-th primary picture. The length of the omi_mask_label[ i ] [ j ] [ k ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

Syntax and semantics of proposed OMI SEI message for HEVC

Table 2 syntax of OMI SEI message for HEVC

| Object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if(!om_cancel_flag) { |  |
| //high level info |  |
| omi_aux_id_minus128 | ue(v) |
| omi_num_mask_pic_minus1 | ue(v) |
| omi_mask_id_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( om_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_object_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_object_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if(omi_mask_label_language_present_flag) { |  |
| while(!byte_aligned()) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_lable_language | st(v) |
| } |  |
| } |  |
| // individual mask information |  |
| for (i=0; i<=om_num_mask_pic_minus1; i++) { |  |
| om_mask_pic_update_flag[ i ] | f(1) |
| if(om_mask_pic_update_flag[ i ]) { |  |
| omi_mask_pic_layer_id[ i ] | u(6) |
| om_num_mask_in_pic_update[ i ] | ue(v) |
| for(j=0; j<on_num_mask_in_pic_update[ i ]; j++) { |  |
| om_mask_id[ i ] [ j ] | u(v) |
| if(maskIdExist[i][on_mask_id[ i ] [ j ]]) { |  |
| om_mask_cancel[ i ] [ j ] | u(1) |
| maskIdExist[i][on_mask_id[ i ] [ j ]]= !on_mask_cancel[ i ] [ j ] |  |
| } |  |
| else |  |
| maskIdExist[i][on_mask_id[ i ] [ j ]]=1 |  |
| if(maskIdExist[i][on_mask_id[ i ] [ j ]]) { |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ] [ j ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ] [ j ] | u(v) |
| if(omi_mask_label_info_present_flag){ |  |
| while(!byte_aligned()) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label[ i ] [ j ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

The object mask information (OMI) SEI message provides information about object mask pictures coded as auxiliary pictures. Object mask auxiliary pictures have nuh_layer_id equal to nuhLayerIdA and AuxId[nuhLayerIdA] in range of 128 to 159, inclusive. Each overlay auxiliary picture layer is associated with one or more primary picture layers as specified below.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in output order that is associated with one or more primary picture layers to which this SEI applies. om_cancel_flag equal to 0 indicates that object mask information follows.

omi_aux_id_minus128 plus 128 indicates the value of AuxId of object mask auxiliary pictures. om_aux_id_minus128 shall be in the range of 0 to 31, inclusive.

om_num_mask_pic_minus1 plus 1 indicates the number of object mask auxiliary pictures associated with the same one or more primary picture. The value of om_num_mask_pic_minus1 shall be in the range of 0 to 63, inclusive. The value of om_num_mask_pic_minus1 shall be the same in all OMI SEI message within a CVS.

omi_mask_id_length_minus8 plus 8 indicates the number of bits used for coding omi_mask_id[ i ] [ j ] syntax elements.

omi_msak_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ] [ j ] syntax elements are present. omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ] [ j ] syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_mask_confidence_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ] [ j ] syntax elements. It is a requirement of bitstream conformance that the value of omi_mask_confidence_length_minus1 shall be the same for all object_mask_info ( ) syntax structures within a CLVS.

omi_object_depth_info_present_flag equal to 1 indicates that omi_object_depth[ i ] [ j ] syntax elements are present. omi_object_depth_info_present_flag equal to 0 indicates that omi_ object _depth[ i ] [ j ] syntax elements are not present. It is a requirement of bitstream conformance that the value of omi_object_depth_info_present_flag shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_object_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_object_depth[ i ] [ j ] syntax elements. It is a requirement of bitstream conformance that the value of omi_object_depth_length_minus1 shall be the same for all object_mask_info ( ) syntax structures within a CLVS.

omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ] [ j ] are present. omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ] [ j ] are not present.

omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_lable_language is present. omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_lable_language is not present and that the language of the mask label is unspecified.

omi_bit_equal_to_zero shall be equal to 0.

omi_mask_lable_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_lable_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

om_mask_pic_update_flag[ i ] equal to 1 indicates the mask information of i-th object mask auxiliary picture is signalled. om_mask_pic_update_flag[ i ] equal to 0 indicates the mask information of i-th object mask auxiliary picture is not signalled. When the mask information of i-th object mask auxiliary picture is not present, the persistence mechanism is used, that is the information is inherited from the last OMI SEI message which signals the mask information of i-th object mask auxiliary picture.

omi_mask_pic_layer_id [ i ] indicates the nuh_layer_id value of the the i-th auxiliary picture layer. AuxId[omi_mask_pic_layer_id [ i ]] shall be qual to omi_aux_id_minus128 + 128 for all values of in the range of 0 to om_num_mask_pic_minus1, inclusive.

om_num_mask_in_pic_update[ i ] indicates the number of mask in the i-th auxiliary picture to be signalled. omi_num_mask_in_pic[ i ] shall be in the range of 0 to (1<<BitDepthY) - 1, inclusive, where BitDepthY is the bit depth for the samples of the luma component.

omi_mask_id[ i ] [ j ] indicates the identifier of j-th object mask to be updated in the i-th object mask auxiliary picture.

The object mask identifier associated with the sample location (x, y) in the i-th object mask auxiliary picture is equal to p[ i ][ x ][ y ] where p[ i ][ x ][ y ] refers to the luma sample at location ( x, y ) in the decoded i-th object mask auxiliary picture.

The variable maskId[ i ] [ j ] specifying the object mask identifier of j-th object mask of the i-th object mask auxiliary picture in the SEI message is derived as follows,

for( i = 0; i <= om_num_mask_pic_minus1; i++ ) {

for( j = 0; j <= on_num_mask_in_pic[ i ]; j++ ) {

maskId[ i ][ j ] += omi_mask_id[ i ] [ j ] * (i+1)

}

}

om_mask_cancel[ i ] [ j ] equal to 1 cancels the persistence scop of object mask with identifier equal to om_mask_id[ i ] [ j ]. om_mask_cancel[ i ] [ j ] equal to 0 indicates the information of object mask with identifier equal to om_mask_id[ i ] [ j ] is signaled.

The variable maskIdExist[i][id] equal to 1 indicates the object mask with identifier id in the i-th object mask auxiliary picture exists. The variable maskIdExist[i][id] equal to 0 indicates the object mask with identifier id in the i-th object mask auxiliary picture does not exist. maskIdExist[i][id] is initialized with 0 before decoding the current CVS.

omi_mask_confidence[ i ] [ j ] indicates the degree of confidence associated with the j-th object mask to be updated in the i-th object mask auxiliary picture, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ] [ j ] [ k ] indicates a higher degree of confidence. The length of the omi_mask_confidence [ i ] [ j ] [ k ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

omi_mask_depth[ i ] [ j ] indicates the object depth associated with the j-th object mask to be updated in the i-th object mask auxiliary picture. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ] [ j ] [ k ] syntax element is omi_object_depth_length_minus1 + 1 bits.

omi_mask_label[ i ] [ j ] specifies the contents of the label associated with j-th object mask to be updated in the i-th object mask auxiliary picture. The length of the omi_mask_label[ i ] [ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

Conclusion

The object mask auxiliary picture and object mask information SEI message are proposed in this contribution for both VSEI and HEVC to enhance VVC and HEVC’s capability of handling information from machine analysis tasks such as object detection, tracking, and segmentation. It is asserted that with the increasing importance of machine analysis and artificial intelligence applications, JVET should support more non-normative ways of signaling information obtained from machine analysis tasks at the encoder side. Therefore, it is suggested to adopt the proposed SEI message to VSEI and HEVC.

References

X. Zheng, X. Feng, L. Zhang, C. Zhao, “Mask information for Annotated region SEI message”, JVET-AF0024, July. 2018

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).