| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0177 |
| --- | --- |

| Title: | EE2-1.18: CIIP Extension with IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Karam Naser Philippe Bordes Franck Galpin Kevin Reuze Antoine Robert | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports test results of EE2-1.18 test. In this test, an extension to CIIP replacing the inter-prediction part by IntraTMP is proposed. On top of ECM-8.0, simulation results of the EE2-1.18 are reported as below:

EE2-1.18a:

- AI: -0.28%	-0.30%	-0.28%	100%	103%;

- RA: {%, %, %; %, %}.

EE2-1.18b:

- AI: -0.05%	-0.05%	-0.02% 101%	102%;

- RA: {%, %, %; %, %}.

# Introduction

CIIP extension with intra template matching was proposed in JVET-AC0201 [2] and established as Test 1.18 in JVET-AC EE2 [1]. It is proposed to signal a new mode, named spatial CIIP, inside the intra prediction syntax. This mode uses the same functionality as CIIP, except that the inter part is replaced by IntraTMP prediction. That is, the intra part remains unchanged, as well as the computation of weights.

Two tests are studied:

- EE2-1.18a: including the modification of the IntraTMP search range of EE2-1.14c

- EE2-1.18b: without the modification of the IntraTMP search range of EE2-1.14c

# Simulation Results

The proposed tests are implemented on top of ECM-8.0 [3]. Simulations are performed following the common test conditions of EE2 [4]. The simulation results of Test 1.18a and Test 1.18b compared to ECM-8.0 are shown in Table 1 and Table 2.

Table 1:Test results of EE2-1.18a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.09% | -0.01% | 100% | 101% |
| Class A2 | -0.40% | -0.48% | -0.48% | 98% | 102% |
| Class B | -0.30% | -0.35% | -0.32% | 100% | 102% |
| Class C | -0.16% | -0.10% | -0.15% | 101% | 102% |
| Class E | -0.50% | -0.67% | -0.43% | 103% | 106% |
| Overall | -0.28% | -0.30% | -0.28% | 100% | 103% |
| Class D | -0.31% | -0.48% | -0.45% | 97% | 100% |
| Class F | -0.05% | -0.08% | -0.20% | 99% | 98% |
| Class TGM | -0.10% | -0.07% | -0.04% | 104% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | -0.13% | 96% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.09% | -0.09% | -0.13% | 97% | 99% |
| Class C | -0.06% | -0.13% | -0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.31% | -0.23% | 99% | 102% |
| Class F | -0.09% | -0.46% | -0.14% | 99% | 101% |
| Class TGM | -0.09% | -0.14% | -0.12% | 100% | 106% |

Table 2 Test Results of EE2-1.18b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.05% | 98% | 101% |
| Class A2 | -0.05% | -0.04% | -0.02% | 104% | 106% |
| Class B | -0.06% | -0.07% | 0.01% | 99% | 100% |
| Class C | -0.05% | -0.06% | -0.12% | 102% | 102% |
| Class E | -0.10% | -0.08% | -0.02% | 101% | 100% |
| Overall | -0.05% | -0.05% | -0.02% | 101% | 102% |
| Class D | -0.01% | -0.21% | 0.01% | 98% | 97% |
| Class F | 0.02% | -0.05% | -0.03% | 98% | 98% |
| Class TGM | -0.01% | 0.03% | 0.03% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.06% | 97% | 105% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.22% | -0.06% | 97% | 100% |
| Class C | -0.05% | 0.05% | 0.03% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.06% | -0.06% | -0.36% | 103% | 105% |
| Class F | -0.04% | -0.41% | -0.05% | 101% | 102% |
| Class TGM | 0.02% | -0.01% | 0.07% | 102% | 106% |

# References

[1] V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AC2024, by teleconference, January 2023.

[2] K. Naser, P. Bordes, F. Galpin, K. Reuze, and A. Robert, “AHG12: CIIP extension with Intra Template Matching,” Document of Joint Video Experts Team, JVET-AC0201, by teleconference, January 2023.

[3] ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

[4] M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).