| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0178 |
| --- | --- |

| Title: | non EE2-4.3: IntraTMP Padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Karam Naser Tangi Poirier Antoine Robert Hassane Guermoud | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes extending IntraTMP process with candidates that are partially available and use padding for filling in non-available samples. Similar to EE2-3.4 that performs padding for IBC, it is further extended to IntraTMP mode. The following results are obtained:

- ClassF:

  - AI: {-0.10%, -1.40%, 0.00%; 98%, 98%}.

  - RA: {-0.04%	-1.62%	0.20%	98%	98%}.

- ClassTGM:

  - AI: {-0.21%	0.27%	-0.05%	102%	104%}.

  - RA: {-0.21%	0.32%	0.01%	102%	104%}.

# Introduction

In JVET-AC0193, the intra block copy (IBC) mode is augmented with prediction candidates that are partially decoded. Specifically, padding process is used. An explanation of the padding process is indicated in the figure below:

Figure 1: IBC padding of non-decoded samples (copied from JVET-AC0193)

Similar process is proposed for IntraTMP. However, median value of 3 surrounding samples is used. An explanation of the method is provided in the figure below:

x=median(AL,A,L)

overlapped area

Figure 2: Proposed IntraTMP padding

# Simulation Results

The proposed test is implemented on top of ECM-8.0. Simulations are performed following the common test conditions of EE2. The simulation results are shown in Table 1.

Table 1:IntraTMP padding vs ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.10% | -1.40% | 0.06% | 98% | 98% |
| Class TGM | -0.21% | 0.27% | -0.05% | 102% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.04% | -1.62% | 0.20% | 98% | 98% |
| Class TGM | -0.21% | 0.32% | 0.01% | 102% | 104% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).