| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0183-v2 |
| --- | --- |

| Title: | Non-EE2: SIF flag and BCW weight usage in OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Robert Antoine, Galpin Franck, Poirier Tangi, Chen Ya | Email: | antoine.robert@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to consider the SIF flags and BCW weights of the neighboring and current CUs into the OBMC process.

It is reported that this proposal achieves averagely, on top of ECM-8.0, -0.05% BD rate saving in RA and -0.08% BD rate saving in LDB without any encoding and decoding runtime increase.

# Introduction

In ECM [1], when OBMC is applied, top and left boundary pixels of a CU are refined using neighboring block’s motion information with a weighted prediction.

For the subblock based coding tools (affine, SbTMVP, DMVR), a subblock boundary OBMC is further proposed. It applies an equivalent blending to the top, left, bottom, and right subblock boundary pixels of each subblock using neighboring subblocks’ motion information.

The prediction of the current CU has already been calculated prior to the OBMC process by using all the motion information, including the SIF flag and BCW weight. While the motion compensation of the neighboring blocks and subblocks are performed again by considering only the motion vectors and reference frames.

# Proposal

This proposal aims at using the SIF flags and the BCW weights of the neighboring blocks and subblocks into the OBMC process as both BCW and AMVR indexes can modify the predicted samples in the OBMC motion compensation. It is then proposed to incorporate them into the OBMC process to better fit the real predicted signal.

When processing the top and left borders, the neighboring BCW weights and SIF flags are used in addition to the motion vectors and reference frames. The neighboring SIF flag allows setting the AMVR index to half-pel when it is on, otherwise the default AMVR index is used. For the internal subblock boundaries, the BCW and AMVR indexes of the current CU are also used.

# Experimental results

The proposed method has been tested on top of the ECM-8.0 software [2] under the CTC conditions [3]. The simulation results are depicted in Table 1.

Table 1: Proposed method on top of ECM-8.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.10% | 100% | 101% |
| Class A2 | -0.10% | -0.06% | 0.02% | 101% | 101% |
| Class B | -0.04% | -0.04% | -0.18% | 100% | 100% |
| Class C | -0.05% | -0.07% | -0.12% | 100% | 100% |
| Overall | -0.05% | -0.05% | -0.11% | 100% | 101% |
| Class D | -0.07% | 0.20% | 0.10% | 100% | 100% |
| Class F (optional) | -0.03% | 0.03% | 0.03% | 101% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.06% | -0.12% | -0.20% | 101% | 100% |
| Class C | -0.06% | 0.08% | -0.02% | 100% | 99% |
| Class E | -0.12% | 0.74% | 0.10% | 100% | 100% |
| Overall | -0.08% | 0.16% | -0.06% | 100% | 100% |
| Class D | -0.09% | -0.92% | 0.41% | 100% | 100% |
| Class F (optional) | 0.16% | 0.13% | 0.65% | 101% | 101% |

# Conclusion

This contribution proposes to consider the neighboring and current SIF flags and BCW weights within the OBMC process. A coding efficiency of -0.05% in RA and -0.08% in LDB has been achieved on top of the ECM-8.0 without any additional encoding and decoding runtime increase. We therefore suggest studying this proposal in the next EE2.

# References

- M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 8 (ECM 8)”, JVET-AC2025, January 2023.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January. 2022.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AC2024, January 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).