| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0185 |
| --- | --- |

| Title: | Non-EE2: ISP Extension for Chroma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia1, Kai Zhang1, Yang Wang2, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China | Tel: Email: | wei.jia@bytedance.com zhangkai.video@bytedance.com wangyang.cs@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Intra sub-partitions (ISP) divides a luma intra-predicted block vertically or horizontally into 2 or 4 sub-partitions depending on the block size. In this contribution, it is proposed to extend ISP to chroma components accordingly when dual-tree structure is applied. On top of ECM-8.0, simulation results of the proposed method are reported as below:

AI: {xxx%, xxx%,	xxx%; xxx%, xxx%};

Class F: {0.00%, -0.71%, -0.82%; xxx%, xxx%};

Class TGM: {-0.30%, -1.23%, -1.32%; xxx%, xxx%};

RA: {xxx%, xxx%, xxx%; xxx%, xxx%}.

# Introduction

In ECM, ISP can only be used on luma when dual-tree structure is applied. With ISP on luma, a luma intra-predicted block is split vertically or horizontally into 2 or 4 sub-partitions depending on the block size. Figure 1 shows examples of the possibilities of sub-partitions.

a) Examples of sub-partitions for 4×8 and 8×4 CUs

b) Examples of sub-partitions for CUs larger than 4×8 or 8×4

Figure 1 - Sub-partition depending on the block size.

# Proposed method

An extension of ISP on chroma components is proposed when dual-tree structure is applied. The proposed extension of ISP on chroma divides chroma intra-predicted blocks vertically or horizontally into 2 or 4 sub-partitions depending on the block size following a similar rule as ISP on luma. The minimum width or height of a prediction for subblocks is 2 chroma samples. In addition, the chroma ISP mode is harmonized with the direct block vector (DBV) mode. The BV of a chroma sub-partition is derived based on its collocated luma block.

# Simulation results

The proposed method is implemented on top of ECM-8.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results compared to ECM-8.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-8.0

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.00% | -0.22% | -0.14% | 99% | 94% |  |  |  |  |  |
| Class E | 0.03% | -0.43% | -0.26% | 98% | 91% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.03% | -0.44% | -0.31% | 99% | 111% |  |  |  |  |  |
| Class F | 0.00% | -0.71% | -0.82% | 96% | 93% |  |  |  |  |  |
| Class TGM | -0.30% | -1.23% | -1.32% | 105% | 90% |  |  |  |  |  |

# Conclusions

In this contribution, it is proposed to extend the current design of ISP on luma to chroma components accordingly when dual-tree structure is applied. It is recommended to investigate the proposed method in EE.

# Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).