| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0188-v1 |
| --- | --- |

| Title: | EE2-1.6: Non-local cross-component prediction and cross-component merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng 8910 University Center Lane, San Diego, CA 92122, USA Chia-Ming Tsai, Hsin-Yi Tseng, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | {zhangkai.video, lizhang.idm} @bytedance.com {chia-ming.tsai, ivy-hy.tseng, cheng-yen.chuang, cw.hsu, chingyeh.chen, peter.chuang, olena.chubach, yiwen-yw.chen yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Bytedance Inc., Mediatek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a joint solution of cross-component prediction (CCP) merge mode for chroma intra coding in EE2-1.6 from JVET-AC0176 and JVET-AC0315. The CCP merge mode implies that cross-component model parameters of the current chroma block are inherited from its spatial adjacent, spatial non-adjacent, history-based, or default CCP models. These CCP models are collected from the blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes, and further reordered based on the sum of absolute difference (SAD) cost, which is derived from a neighbouring template of the current block.

On top of ECM-8.0, simulation results of the proposed methods are reported as below:

Test 1.6a: The inherited CCP model is from spatial adjacent, spatial non-adjacent, or default models.
AI BD-rates = -0.16 % (Y), -1.82 % (U), -1.97 % (V); EncT=102 %; DecT=102 %
RA BD-rates = -0.xx % (Y), -0.xx % (U), -0.xx % (V); EncT=10x %; DecT=10x %

Test 1.6b: The inherited CCP model is from spatial adjacent, history-based, or default models.
AI BD-rates = -0.14 % (Y), -1.59 % (U), -1.79 % (V); EncT=102 %; DecT=102 %
RA BD-rates = -0.xx % (Y), -0.xx % (U), -0.xx % (V); EncT=10x %; DecT=10x %

Test 1.6c: The inherited CCP model is from spatial adjacent, spatial non-adjacent, history-based, or default models.
AI BD-rates = -0.19 % (Y), -2.12 % (U), -2.30 % (V); EncT=102 %; DecT=102 %
RA BD-rates = -0.xx % (Y), -0.xx % (U), -0.xx % (V); EncT=10x %; DecT=10x %

# Introduction

Cross-component prediction (CCP) including cross-component linear model (CCLM), convolutional cross-component model (CCCM), gradient linear model (GLM) and their variants is adopted in ECM to exploit the cross-component correlation. In the JVET AC-meeting, non-local cross-component prediction [1] and cross-component merge mode [2] are proposed to further improve the efficiency of CCP. During the EE study, methods in the two contributions are combined and the joint solutions are presented in this proposal.

# Proposed methods

In EE2-1.6, a CCP merge candidate list is constructed from the spatial adjacent, spatial non-adjacent, or history-based candidates. After including these candidates, default models are further included to fill the remaining empty positions in the merge list. In order to remove redundant CCP models in the list, pruning operation is applied. After constructing the list, the CCP models in the list are reordered depending on the SAD costs, which are obtained using the neighbouring template of the current block. More details are described below.

Spatial adjacent and non-adjacent candidates

The positions and inclusion order of the spatial adjacent and non-adjacent candidates are the same as those defined in ECM for regular inter merge prediction candidates.

History-based candidates

A history-based table is maintained to include the recently used CCP models, and the table is reset at the beginning of each CTU row. If the current list is not full after including spatial adjacent and non-adjacent candidates, the CCP models in the history-based table are added into the list.

Default candidates

CCLM candidates with default scaling parameters are considered, only when the list is not full after including the spatial adjacent, spatial non-adjacent, or history-based candidates. If the current list has no candidates with the single model CCLM mode, the default scaling parameters are {0, 1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}. Otherwise, the default scaling parameters are {0, the scaling parameter of the first CCLM candidate + {1/8, -1/8, 2/8, -2/8, 3/8, -3/8, 4/8, -4/8, 5/8, -5/8, 6/8}}. The offset parameter is derived according to the default scaling parameter, average neighbouring reconstructed luma sample value, and average neighbouring reconstructed Cb/Cr sample value.

A flag is signaled to indicate whether the CCP merge mode is applied or not. If CCP merge mode is applied, an index is signaled to indicate which candidate model is used by the current block. In addition, CCP merge mode is not allowed for the current chroma coding block when the current CU is coded by intra subpartitions (ISP) with single tree, or the current chroma coding block size is less than or equal to 16.

# Experimental results

Three tests, Test 1.6a, Test 1.6b, and Test 1.6c, are conducted with the merge list constructed by checking different potential candidates:

Test 1.6a: the merge list includes spatial adjacent, non-adjacent, and default candidates.
Test 1.6b: the merge list includes spatial adjacent, history-based, and default candidates.
Test 1.6c: the merge list includes spatial adjacent, non-adjacent, history-based, and default candidates.

In Test 1.6a and Test 1.6b, the maximum allowed list size is 6. In Test 1.6c, the size is changed to 12 to accommodate more potential candidates. Besides, the maximum allowed history-based table size is 6 in Test 1.6b and Test 1.6c.

The EE tests are implemented on top of ECM-8.0 [3] and tested following the common test conditions defined in [4]. The simulation results are summarized in below tables.

Table 1: Performance of Test #1.6a

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -2.06% | -3.13% | 101% | 102% |  |  |  |  |  |
| Class A2 | -0.32% | -3.07% | -3.21% | 102% | 102% |  |  |  |  |  |
| Class B | -0.12% | -2.15% | -2.13% | 101% | 102% | -0.03% | -1.71% | -1.51% | 102% | 101% |
| Class C | -0.07% | -0.68% | -0.84% | 102% | 101% | -0.05% | -0.29% | -0.06% | 100% | 100% |
| Class E | -0.06% | -1.29% | -0.80% | 101% | 101% |  |  |  |  |  |
| Overall | -0.16% | -1.82% | -1.97% | 102% | 102% |  |  |  |  |  |
| Class D | -0.07% | -1.00% | -0.69% | 102% | 101% | -0.03% | -0.24% | -0.65% | 100% | 100% |
| Class F | -0.14% | -1.19% | -1.23% | 101% | 102% | -0.14% | -1.30% | -1.15% | 100% | 101% |

Table 2: Performance of Test #1.6b

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -1.71% | -2.97% | 102% | 102% |  |  |  |  |  |
| Class A2 | -0.27% | -2.81% | -2.87% | 103% | 102% |  |  |  |  |  |
| Class B | -0.10% | -1.85% | -1.90% | 102% | 102% | 0.00% | -1.43% | -1.48% | 101% | 100% |
| Class C | -0.06% | -0.68% | -0.85% | 103% | 102% | -0.04% | -0.44% | -0.37% | 101% | 100% |
| Class E | -0.06% | -1.01% | -0.75% | 102% | 101% |  |  |  |  |  |
| Overall | -0.14% | -1.59% | -1.79% | 102% | 102% |  |  |  |  |  |
| Class D | -0.05% | -0.80% | -0.72% | 103% | 101% | 0.01% | -0.30% | -0.71% | 101% | 100% |
| Class F | -0.11% | -1.01% | -1.19% | 102% | 102% | -0.17% | -1.21% | -1.02% | 101% | 101% |

Table 3: Performance of Test #1.6c

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -2.44% | -3.79% | 101% | 103% |  |  |  |  |  |
| Class A2 | -0.36% | -3.45% | -3.64% | 103% | 102% |  |  |  |  |  |
| Class B | -0.14% | -2.45% | -2.51% | 101% | 101% | -0.04% | -1.98% | -2.14% | 99% | 100% |
| Class C | -0.10% | -0.88% | -0.95% | 103% | 101% | -0.06% | -0.49% | -0.44% | 100% | 100% |
| Class E | -0.06% | -1.58% | -0.92% | 102% | 104% |  |  |  |  |  |
| Overall | -0.19% | -2.12% | -2.30% | 102% | 102% |  |  |  |  |  |
| Class D | -0.08% | -0.96% | -0.86% | 103% | 103% | 0.02% | -0.50% | -0.92% | 101% | 99% |
| Class F | -0.21% | -1.72% | -1.73% | 103% | 102% | -0.17% | -1.51% | -1.53% | 100% | 100% |

# Conclusions

This contribution presents the joint solutions of the methods proposed in [1] and [2]. The proposed joint solutions contribute promising coding gains with negligible complexity overhead. Considering a good trade-off between coding efficiency and run time increase, it is recommended to adopt Test 1.6c into the ECM.

# References

- K. Zhang, L. Zhang, Z. Deng, “Non-EE2: Non-Local Cross-Component Prediction,” JVET-AC0176, Jan. 2023.

- C.-M. Tsai, H.-Y. Tseng, C.-Y. Chuang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, O. Chubach, Y.-W. Chen, Y.-W. Huang, S.-M. Lei, “Non-EE2: Cross-component merge mode for chroma intra coding”, Jan. 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediatek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).