| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0190-v1 |
| --- | --- |

| Title: | EE2-1.23: Modification to MPM list derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Yao-Jen Chang Vadim Seregin Marta Karczewicz (Qualcomm) | Tel: Email: | hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the test results of EE2-1.23, which studies a unified MPM generation for the regular intra prediction modes and template-based multiple reference line intra (TMRL) modes coding is studied. In the proposed method, an intra mode is derived for the MIP/intraTMP/Planar coded CUs to be used for future MPM list generation. Simulation results reportedly show that EE2-1.23 a achieves Y, U, V BD-rate change with encoder and decoder runtime as follows:

AI: -0.05%, -0.05%, -0.00%, 100%(EncT), 100% (DecT)

RA: -0.03%, -0.12%, -0.08%, 101%(EncT), 100%(DecT)

# Introduction

The test EE2-1.23 studies a unified MPM generation for the regular intra prediction modes and template-based multiple reference line intra (TMRL) modes, which proposed in [3]. Details of the method are described as follows:

- For spatial neighbors of the current block, if a neighbor block is coded by MIP/IntraTMP/Planar mode, the first non-planar mode of the MPM list of the neighbor block is selected as the derived mode and the derived mode is tagged as ‘reserved’ for the partial MPM reordering performed in ECM-8.0 when generating MPM list for the current block.

- Secondary MPM modes are derived by adding offset to all primary angular modes, this process is the same as in ECM-8.0 for TMRL modes, where the offset is added to DIMD modes first.

- If MPM list is incomplete, the same default modes are added as in ECM-8.0.

# Simulation results

The proposed methods are tested on top of ECM-8.0[1]. The test simulations were conducted using the Common Test Conditions as specified in [2]. Test results for AI and RA configurations are summarized in the following tables.

Table 1 Simulation results of EE2-1.23

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.05% | -0.01% | 101% | 99% |
| Class A2 | -0.06% | 0.03% | 0.02% | 100% | 100% |
| Class B | -0.04% | -0.10% | 0.05% | 101% | 100% |
| Class C | -0.06% | -0.05% | -0.04% | 100% | 101% |
| Class E | -0.08% | -0.13% | -0.01% | 100% | 101% |
| Overall | -0.05% | -0.05% | 0.00% | 100% | 100% |
| Class D | -0.05% | -0.21% | -0.04% | 100% | 100% |
| Class F | 0.01% | -0.08% | 0.10% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.15% | -0.13% | 100% | 100% |
| Class A2 | -0.01% | -0.21% | 0.07% | 100% | 99% |
| Class B | -0.01% | -0.15% | -0.19% | 101% | 100% |
| Class C | -0.05% | -0.22% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.12% | -0.08% | 101% | 100% |
| Class D | -0.02% | 0.10% | -0.14% | 101% | 99% |
| Class F | -0.04% | -0.33% | -0.19% | 102% | 100% |

# References

- ECM-8.0 https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- Exploration experiment on enhanced compression beyond VVC capability (EE2), V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, JVET-AC2024, Jan 2023

- Non-EE2: Modification to MPM list derivation, H. Wang, Y.-J. Chang, V. Seregin, M. Karczewicz, JVET-AC0191, Jan 2023

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).