| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0191-v3 |
| --- | --- |

| Title: | EE2-1.20e: Combination Test of Test 1.10 + Test 1.13 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeva Raj Arumugam, Ashwin Natesan, Vaibhav Valvaiker, Abhijeet Sarate, Jay N. Shingala (Ittiam) Fangjun Pu, Taoran Lu, Peng Yin, Sean McCarthy (Dolby) Fan Wang (OPPO) | Tel: Email: | {jeeva.raj, jay.shingala }@ittiam.com {fpu, tlu, pyin}@dolby.com wangfan6@oppo.com |
| Source: | Dolby Laboratories, Inc., Ittiam System, OPPO |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 IntraTMP combination Test 1.20e. This is the combination test of Test 1.10 (multi-candidate IntraTMP) and Test 1.13 (adaptive intraTMP fusion with up to two predicted blocks). The impact on coding efficiency and runtimes over ECM-8.0 is reportedly {for Y, U, V, EncT an DecT} as follows:

EE2-1.20e: Test 1.10 + Test 1.13

AI {-0.29% Y, -0.32% U, -0.32% V, 103% EncT, 101% DecT }

RA {-x.xx% Y, -x.xx% U, -x.xx% V, xxx% EncT, xxx% DecT }

# Proposed method

This contribution reports the simulation results of EE2 Test 1.20e which studies the joint combination of Test 1.10 [2] and Test 1.13 [3]. Test 1.10 tests multi-candidate IntraTMP, where a candidate list is constructed with the candidate BVs ranked in ascending order of their template matching costs, the index of selected candidate is signalled in the bitstream. This test also includes MTMP search. Test 1.13 tests adaptive intraTMP fusion with up to two predicted blocks. The search process is improved using updated template.

The combination is made as follows: based on 1.10, if intraTMP fusion is enabled, determine if updated templates from 1.13 should be used and apply intraTMP search and fusion accordingly. Otherwise, it is the default test 1.10.

Proposed syntax change is as follows:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp_fusion_flag

if(intra_tmp_fusion_flag) {

intra_tmp_template_update_flag

} else{

intra_tmp_idx

}

}

…

intra_tmp_flag indicates whether the intra prediction type for current block is IntraTMP or not.

intra_tmp_fusion_flag indicates whether fusion is used or not for current block.

intra_tmp_template_update_flag indicates whether template updating is applied or not for intra TMP.

intra_tmp_idx specifies the index of BV in the candidate list used for current block. The range of intra_tmp_idx is 0 to 14. Candidates from L-shape template, top template and left template are included in the same candidate list.

# Simulation results

The proposed method is implemented on top of ECM8.0 and evaluated using the common test conditions.

Table 2 Test 1.20e coding performance compared with ECM8.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.00% | 0.01% | 102% | 100% |
| Class A2 | -0.36% | -0.40% | -0.45% | 102% | 102% |
| Class B | -0.31% | -0.37% | -0.37% | 103% | 100% |
| Class C | -0.13% | -0.19% | -0.14% | 103% | 100% |
| Class E | -0.65% | -0.63% | -0.68% | 103% | 101% |
| Overall | -0.29% | -0.32% | -0.32% | 103% | 101% |
| Class D | -0.09% | -0.39% | -0.01% | 103% | 98% |
| Class F | -0.03% | -0.12% | 0.01% | 102% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.17% | -0.17% | 101% | 101% |
| Class C | -0.05% | -0.10% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.10% | -0.30% | 98% | 98% |
| Class F | -0.06% | -0.49% | -0.33% | 100% | 100% |

# Conclusion

This contribution reports the test results of EE2 IntraTMP combination: test 1.20e.

# References

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, April 2023.

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.10 multi-candidate IntraTMP”, JVET-AD0073, April 2023.

- J. R. Arumugam, A. Natesan, V. Valvaiker, A. Sarate, J. N. Shingala, F. Pu, T. Lu, P. Yin, T. Shao, A. Arora, S. McCarthy, “EE2-1.13: Test on Fusion of Intra Template Matching”, JVET-AD0158, April 2023.

# Acknowledgement

We would like to thank Alibaba for helping the crosschecks.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).