| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0193-v2 |
| --- | --- |

| Title: | EE2-2.11e: Adaptive OBMC control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Cui Zhi Zhang Han Huang Vadim Seregin Marta Karczewicz (Qualcomm) Xiang Li (Google) Zhipin Deng Kai Zhang Li Zhang (Bytedance) | Tel: Email: | kaicui@qti.qualcomm.com zhizhang@qti.qualcomm.com xlxiangli@google.com zhipin.deng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Inc., Google Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of Test-2.11e, where OBMC flag is inherited for affine merge mode and OBMC usage is conditioned on the coded mode of the neighboring blocks and max difference of the reference samples. The experimental results are summarized as follows.

| Test | RA (Y/U/V, [EncT, DecT]) | LDB (Y/U/V, [EncT, DecT]) |
| --- | --- | --- |
| Overall (class A-E) | -0.02%/0.00%/-0.01%, [101%, 100%] | -0.07%/0.08%/0.18% [100%, 99%] |
| Class F | -0.54%/-0.56%/-0.58%, [103%, 98%] | -0.38%/-0.70%/-0.30%, [102%, 102%] |
| Class TGM | 0.00%/0.00%/0.00%, [100%, 102%] | 0.00%/0.00%/0.00%, [101%, 103%] |
| Class M | -0.49%/-0.45%/-0.41%, [103%, 101%] | -1.32%/-1.04%/-0.91%, [102%, 107%] |

# Introduction

Test-2.11e [3] proposes modifications to Overlapped Block Motion Compensation (OBMC). The proposed modifications include the following aspects:

- OBMC flag is inherited from a neighboring affine block for affine merge mode.

- OBMC is not applied to a block if there is a neighbor block coded with IBC, palette, or BDPCM modes.

- When applying OBMC to a block, block boundary check whether OBMC is applied to the boundary is further made based on the reference samples of the current block. If any absolute difference between the prediction sample and non-interpolated (integer pel) reference sample is greater than a threshold, the OBMC is not applied to that boundary.

# Experimental results

This test was implemented on top of ECM-8.0 with applied bug fix for OBMC SPS setting for low delay B reported in MR 380.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8 + MR380 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.02% | 102% | 102% |
| Class A2 | -0.07% | -0.04% | 0.08% | 102% | 101% |
| Class B | 0.00% | -0.01% | -0.22% | 101% | 99% |
| Class C | -0.04% | 0.03% | 0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.01% | 101% | 100% |
| Class D | -0.08% | 0.30% | 0.00% | 98% | 97% |
| Class F | -0.54% | -0.56% | -0.58% | 103% | 98% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class M | -0.49% | -0.45% | -0.41% | 103% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8 + MR380 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.19% | -0.01% | 100% | 99% |
| Class C | -0.04% | 0.01% | 0.08% | 101% | 99% |
| Class E | -0.19% | 0.63% | 0.63% | 99% | 97% |
| Overall | -0.07% | 0.08% | 0.18% | 100% | 99% |
| Class D | -0.03% | 0.17% | 0.16% | 101% | 101% |
| Class F | -0.38% | -0.70% | -0.30% | 102% | 102% |
| Class TGM | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class M | -1.32% | -1.04% | -0.91% | 102% | 107% |

Class M includes three mixed content sequences that are from JVET-AC1015 “Common test conditions for SCM screen content coding”. It was asserted during the JVET-AC meeting that mixed-content is more challenging than pure screen content for adaptive OBMC on/off control.

# Reference

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 22nd Meeting, by teleconference, 20–28 Apr. 2021.

[2] K. Sühring, “Common test conditions for SCM screen content coding” JVET-AC1015, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023.

[3] V. Seregin, J. Chen, etc., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11–20 January 2023.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Google Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).