| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting: Antalya, TR, 21–28 April 2023 | Document: JVET-AD0194-v2 |
| --- | --- |

| Title: | EE2-1.19: IntraTMP with multiple modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin Jian-Liang Lin Vadim Seregin Marta Karczewicz | Email: | pohanlin@qti.qualcomm.com jilin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports experiment results for EE2 Test-1.19 on intra template matching. When IntraTMP is enabled, multiple template type (L-shape, left and above) are used to derive the template matching candidates and these candidates can be further fused using linear combination. Test-1.19a proposes to use only left template and above template to derive the template matching candidates. Test-1.19b proposes to fuse the multiple template matching candidates. The fusion weights are derived based on the template matching costs or using the derivation method of MSE minimization, which already exists in CCCM.

Test 1.19a:

AI: -0.13% (Y), -0.14% (U), -0.12% (V), 101% (Enc. time), 105% (Dec. time).

RA: -X.XX% (Y), -X.XX% (U), -X.XX% (V), XXX% (Enc. time), XXX% (Dec. time).

Test 1.19b:

AI: -0.20% (Y), -0.24% (U), -0.18% (V), 102% (Enc. time), 105% (Dec. time).

RA: -X.XX% (Y), -X.XX% (U), -X.XX% (V), XXX% (Enc. time), XXX% (Dec. time).

Test 1.19c:

AI: -0.28% (Y), -0.31% (U), -0.29% (V), 103% (Enc. time), 106% (Dec. time).

RA: -X.XX% (Y), -X.XX% (U), -X.XX% (V), XXX% (Enc. time), XXX% (Dec. time).

# Introduction

In this proposal, the simulation results of EE2 Test-1.19 on IntraTMP are reported.  In ECM8.0, IntraTMP finds the best prediction block from the reconstructed area of the current frame whose L-shape template matches the template of current block in a predefined search range. The encoder/decoder searches the most similar template in the search range which minimize the SAD cost.

# Proposal

This contribution proposes to introduce 3 additional IntraTMP modes: using only left template, using only above template to derive the candidates, and IntraTMP fusion mode which fuses multiple candidates with linear combination.

In Test-1.19a, 3 template types are defined. In addition to L-shape (left and above templates), using only left and only above templates as shown in Fig.1 is proposed. For each template type, the best 4 candidates are derived in the template matching process and selected by HAD cost. To signal the selected candidate (template type and candidate index), a flag is signaled to indicate which template type is used followed by the candidate index.

In Test-1.19b, 2 fusion methods are proposed to combine multiple candidates. The fusion weights are derived by template matching costs or using the derivation method of MSE minimization existed in CCCM. A flag is signaled to indicate which weight derivation method is used. The candidates to be fused are the best 2 L-shape candidates or the best 5 L-shape candidates. Another flag is signalled to indicate the number of candidates to be fused (2 or 5).

Test-1.19c is the combination of tests 1.19a and 1.19b.

Figure 1. Illustration of left template and top template.

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the tests 1.19 in ECM-8.0 following the common test conditions [2].

## Test 1.19a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.03% | 102% | 106% |
| Class A2 | -0.18% | -0.23% | -0.16% | 103% | 111% |
| Class B | -0.16% | -0.12% | -0.10% | 101% | 105% |
| Class C | -0.06% | -0.03% | -0.08% | 101% | 103% |
| Class E | -0.27% | -0.32% | -0.29% | 100% | 101% |
| Overall | -0.13% | -0.14% | -0.12% | 101% | 105% |
| Class D | -0.03% | -0.18% | -0.11% | 100% | 99% |
| Class F | 0.06% | 0.02% | -0.11% | 101% | 100% |
| Class TGM | -0.10% | -0.06% | -0.06% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.07% | -0.07% | -0.16% | 100% | 100% |
| Class C | -0.03% | -0.02% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | -0.04% | -0.13% | 100% | 100% |
| Class F | -0.09% | -0.46% | -0.17% | 98% | 100% |
| Class TGM | -0.10% | -0.18% | -0.12% | 98% | 101% |

## Test 1.19b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.03% | 103% | 106% |
| Class A2 | -0.30% | -0.36% | -0.32% | 103% | 109% |
| Class B | -0.26% | -0.29% | -0.21% | 102% | 107% |
| Class C | -0.07% | -0.15% | -0.09% | 102% | 104% |
| Class E | -0.34% | -0.44% | -0.34% | 100% | 102% |
| Overall | -0.20% | -0.24% | -0.18% | 102% | 105% |
| Class D | -0.02% | -0.30% | -0.04% | 101% | 98% |
| Class F | -0.01% | -0.22% | -0.03% | 101% | 100% |
| Class TGM | -0.19% | -0.08% | -0.05% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.12% | -0.20% | -0.20% | 100% | 100% |
| Class C | -0.02% | -0.26% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.05% | 0.18% | -0.21% | 100% | 100% |
| Class F | -0.04% | -0.55% | 0.16% | 98% | 100% |
| Class TGM | -0.13% | -0.11% | -0.15% | 98% | 100% |

## Test 1.19c

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | 0.01% | 104% | 105% |
| Class A2 | -0.36% | -0.47% | -0.40% | 104% | 110% |
| Class B | -0.35% | -0.38% | -0.37% | 102% | 107% |
| Class C | -0.11% | -0.12% | -0.12% | 102% | 104% |
| Class E | -0.52% | -0.53% | -0.55% | 101% | 103% |
| Overall | -0.28% | -0.31% | -0.29% | 103% | 106% |
| Class D | -0.05% | -0.31% | -0.21% | 101% | 98% |
| Class F | -0.06% | -0.13% | -0.25% | 100% | 101% |
| Class TGM | -0.27% | -0.23% | -0.14% | 100% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.16% | -0.13% | -0.19% | 101% | 100% |
| Class C | -0.07% | -0.07% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | -0.27% | -0.12% | 100% | 100% |
| Class F | -0.09% | -0.40% | -0.41% | 98% | 100% |
| Class TGM | -0.12% | -0.12% | -0.19% | 98% | 101% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] V. Seregin, J. Chen etc, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 29th Meeting, by teleconference, 11-20 January 2023.

[2] P.-H. Lin, J.-L. Lin, V. Seregin, M. Karczewicz. “Non-EE2: IntraTMP with multiple modes”, JVET-AC0198, Jan. 2023.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.