| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0195 |
| --- | --- |

| Title: | EE2-2.6: High-Precision MV Refinement for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-2.6.

In test EE2-2.6 (JVET-AC0182), high precision MV refinement for BDOF is proposed. It includes accurate BDOF parameter calculation, added weights, as well as adaptive BDOF DMVR subblock size.

On top of the ECM-8.0, simulation result of the proposed method is reported as:

RA: -0.20%, -0.19%, -0.18%, 100%, 101%.

# Introduction

BDOF sample adjustment was studied during VVC development activity and was adopted in the final standard. It derives a motion refinement (vx, vy) for a 4x4 subblock and adjusts its samples individually. A simplified parameter derivation process was proposed in JVET-O0304 (and adopted in VTM), with 0.02% loss on VTM-5.0.

In ECM, there are 2 kinds of BDOFs: BDOF as MV refinement (3rd stage of DMVR process), and BDOF as sample adjustment (similar to VVC, but derives motion adjustment (vx, vy) for each sample separately).

# Proposed method

In this contribution (as disclosed in JVET-AC0182 [1]), it is proposed to use the high-precision equations to derive the BDOF MV refinement parameters as:

Gx.Gx * vx  + Gx.Gy * vy  = dI . Gx   s1 * vx + s2 * vy = s3

Gx.Gy * vx  + Gy.Gy * vy  = dI . Gy   s2 * vx + s5 * vy = s6

where Gx/Gy are the summation of the 2 horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums, where weights depend on the position in the target region . The weights can also be applied to derive vx/vy in other cases.

Also, subblock size of BDOF DMVR is adaptively selected depending on the width×height. For blocks smaller than 256, subblock size of 4×4, and otherwise 8×8 is used.

# Experimental results

The proposed method has been implemented on top of the ECM-8.0 [2]. Testing results of EE2-2.6 under RA configuration is summarized in Table I. The simulations were performed following the common test conditions [3]. More detailed results can be found in the attached spreadsheet.

Table I: Performance of EE2-2.6

# Conclusions

In this contribution, improvement for BDOF is proposed. With a negligible impact on encoder and decoder complexity, it provides -0.20% RA coding gain. It is recommended to adopt this method into ECM-9.0.

# References

[1] M. Salehifar, Y. He, K. Zhang, L. Zhang, “Non-EE2: High-Precision MV Refinement for BDOF,” JVET-AC0182, Jan. 2023.

[2] ECM-8.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).