| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0197 |
| --- | --- |

| Title: | EE2-related: Worst-Case Timing Reduction for ECM RA Simulations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yang Wang, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com wangyang.cs@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Current ECM is drastically slower than VTM. Simulations for RA configuration takes 12 days to finish. In this contribution, it is proposed to reduce intra period from 64 to 32 for slow QP points of class A1/A2. This will lead to more parallelization and, thus allowing all RA simulations to be finished in 7 days.

Compared with ECM-8.0 in the current CTC setting, the overall BD loss is reported as:

RA: 0.10%, -0.13%, -0.14%, 101%, 101%.

# Proposed Modification

The slowest QP points for RA configurations are QP 22 of class A1/A2, as well as QP 27 of ParkRunning3. It is proposed that for these tests, intra period be reduced from 64 to 32. All other configurations are kept the same as the current common test conditions (CTC).

# Experimental results

The proposed modification has been tested on top of the ECM-8.0 [1].

Test #1: Intra period is modified to 32 for all QP points in class A1/A2

Test #2: Intra period is modified to 32 for slow QP points in class A1/A2

Testing results under RA configuration for tests 1 and 2 are summarized in Table I, and II respectively. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table I: Performance of test 1

Table II: Performance of test 2

# Conclusions

In this contribution, it is proposed to reduce intra period for slow QP points of class A1/A2 from 64 to 32. This will allow all RA simulations to be finished much faster (in 7 days, rather than 12 days). It is proposed to include this modification in CTC for ECM.

# References

[1] ECM-8.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology described in this contribution.