| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0200 |
| --- | --- |

| Title: | EE2-related: InterMTS for IBC and IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrick Garus, Muhammed Coban, Bappaditya Ray, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | {pgarus, mcoban, bray, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In EE2 Test 1.8, IBC with additional encoder optimizations is studied for camera captured content.

This contribution improves the coding gain on top of Test EE2-1.8 further by enabling interMTS kernels for IBC predicted blocks. Furthermore, IBC and intraTMP MTS Kernels are being aligned, i.e. intraTMP is using interMTS instead of intraMTS kernels.

The following results are provided on top of EE2-Test1.8a:

AI: -x.xx% (Y), -x.xx% (U), -x.xx % (V), xxx% (EncT), xx% (DecT).

RA: -x.xx% (Y), -x.xx% (U), -x.xx % (V), xxx% (EncT), xx% (DecT).

# Introduction

In EE2-Test1.8a, IBC with additional encoder optimizations is proposed for camera captured content. In IBC separable DCT-II and Transform Skip are supported. Motivated by the application of IBC for camera captured content, this proposal extends the transform options for IBC by interMTS kernels.

In ECM8.0 as well as in EE2-Test1.8a, IntraTMP currently supports adaptive MTS utilizing the intraMTS kernels by using DIMD. Due to the expected similarity between IntraTMP- and IBC-related residuals, an alignment is performed, i.e. it is proposed to utilize interMTS kernels instead of intraMTS kernels for IntraTMP.

# Proposal

This contribution proposes to add MTS to IBC-coded blocks and to utilize interMTS kernels for both, IBC and IntraTMP. As in interMTS for INTER-Blocks, four fixed candidates are tested and signaled. In the encoder MTS is tested only for IBC AMVP Mode.

# Test Results

The following results are provided on top of EE2-Test1.8a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.8a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.06% | -0.01% | 0.04% | 100% | 100% |
| Class C | -0.03% | 0.09% | -0.04% | 101% | 100% |
| Class E | -0.20% | -0.17% | -0.24% | 99% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | 0.12% | 0.00% | 101% | 101% |
| Class F | -0.02% | -0.03% | -0.05% | 104% | 101% |
| Class TGM | -0.04% | 0.02% | 0.04% | 98% | 93% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-1.8a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

In this contribution, we show the benefit of utilizing interMTS for IBC for camera captured content and an alignment of MTS with IntraTMP with neglectable runtime increase. It is recommended to further study MTS for IBC and IntraTMP in scope of the next EE.

# References

[1] ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).