| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0200-v2 |
| --- | --- |

| Title: | EE2-related: InterMTS for IBC and IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrick Garus, Muhammed Coban, Bappaditya Ray, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | {pgarus, mcoban, bray, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In EE2 Test-1.8, IBC with additional encoder optimizations is studied for camera captured content.

This contribution improves the coding gain on top of Test-1.8 further by enabling interMTS kernels for IBC predicted blocks. Furthermore, IBC and intraTMP MTS Kernels are being aligned, i.e. intraTMP is using interMTS instead of intraMTS kernels.

The following results are provided on top of EE2-Test1.8a:

AI: -0.07% (Y), -0.02% (U), -0.05 % (V), 100% (EncT), 100% (DecT).

# Introduction

In EE2 Test-1.8a, IBC with additional encoder optimizations is proposed for camera captured content. In IBC, separable DCT-II and Transform Skip are supported. Motivated by the application of IBC for camera captured content, this proposal extends the transform options for IBC by interMTS kernels.

In ECM-8.0 as well as in Test-1.8a, IntraTMP supports adaptive MTS utilizing the intraMTS kernels by using DIMD. Due to the expected similarity between IntraTMP- and IBC-related residuals, an alignment is performed, i.e. it is proposed to utilize interMTS kernels instead of intraMTS kernels for IntraTMP.

# Proposal

This contribution proposes to add MTS to IBC-coded blocks and to utilize interMTS kernels for both, IBC and IntraTMP. As in interMTS for inter coded blocks, four fixed candidates are tested and signaled. In the encoder, MTS is tested only for IBC AMVP Mode.

## Test Results on top of EE2 Test-1.8a

In this test, the proposal is implemented on top of EE2 Test-1.8a. The anchor is EE2 Test 1.8a. The simulation results are summarized in Table 1 in all intra configuration.

Table 1: Performance of the proposed method over EE2 Test 1.8a in all intra configuration.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 Test-1.8a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.10% | 99% | 100% |
| Class A2 | -0.07% | -0.01% | 0.01% | 101% | 102% |
| Class B | -0.06% | -0.01% | 0.04% | 100% | 100% |
| Class C | -0.03% | 0.09% | -0.04% | 101% | 100% |
| Class E | -0.20% | -0.17% | -0.24% | 99% | 100% |
| Overall | -0.07% | -0.02% | -0.05% | 100% | 100% |
| Class D | -0.02% | 0.12% | 0.00% | 101% | 101% |
| Class F | -0.02% | -0.03% | -0.05% | 104% | 101% |
| Class TGM | -0.04% | 0.02% | 0.04% | 98% | 93% |

## Test Results on top of ECM8+IBC

In this test, the proposal is implemented on top of ECM-8.0 and IBC is enabled. The anchor is ECM-8.0 with IBC enabled. The simulation results are summarized in Table 2 in all intra configuration.

Table 2: Performance of the proposed method over ECM8.0 in all intra configuration.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM8.0+IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.07% | 100% | 99% |
| Class A2 | -0.04% | 0.04% | 0.00% | 101% | 101% |
| Class B | -0.03% | 0.08% | 0.05% | 102% | 100% |
| Class C | -0.02% | 0.02% | 0.02% | 103% | 100% |
| Class E | -0.10% | -0.10% | -0.14% | 102% | 101% |
| Overall | -0.04% | 0.02% | -0.02% | 102% | 100% |
| Class D | -0.02% | -0.01% | 0.01% | 103% | 100% |
| Class F | -0.02% | -0.03% | -0.04% | 103% | 101% |
| Class TGM | -0.04% | 0.02% | 0.04% | 101% | 100% |

# Conclusion

In this contribution, we show the benefit of utilizing interMTS for IBC for camera captured content and an alignment of MTS with IntraTMP with neglectable runtime increase. It is recommended to further study MTS for IBC and IntraTMP in scope of the next EE2.

# References

[1] ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).