| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0202-v1 |
| --- | --- |

| Title: | EE2-1.2: CCCM using multiple downsampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Vadim Seregin, Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this EE test, it is proposed to apply multiple downsampling filters to reconstructed luma samples corresponding to the prediction shape of a convolutional cross-component model for chroma intra prediction. The linear combination of these variously downsampled samples separately multiplied by derived coefficients composes a chroma predictor. The experimental results over ECM-8.0 are summarized as follows:

AI:              -0.06 % (Y), -1.05 % (U), -0.90 % (V), 100 % (EncT), 100 % (DecT)

RA:             -0.01 % (Y), -0.56 % (U), -0.51 % (V), 101 % (EncT), 99 % (DecT)

# Introduction

Convolutional cross-component mode (CCCM) in ECM [1] applies a single downsampling filter to reconstructed luma samples corresponding to the cross prediction shape. The downsampled samples as well as a nonlinear term and a bias term are then combined together with filter coefficients to calculate the predicted chroma value. As an alternative mode of CCCM, a Gradient and location based CCCM (GL-CCCM) was later adopted into ECM-8.0 [1] to further combine the gradients of downsampled samples and the locations of the center luma sample into the formula to form the final predicted chroma value. The filter coefficients are derived using Gaussian elimination method.

# Proposed method

It is proposed to apply multiple downsampling filters to a group of reconstructed luma samples in a CCCM. The linear combination of these downsampled reconstructed samples is multiplied by derived filter coefficients to form the final chroma predictor. The horizontal or vertical location of the center luma sample may be also considered in the proposed model. The coefficients are derived by Gaussian elimination method as currently used by CCCM modes in ECM. The cross-component models shown below are tested as additional CCCM modes with a mode index signaled in the bitstream:

- Model 1: predChroma = c0 * H(C) + c1 *  G1(C) + c2 *  G2(C) + c3 *  G3(C) + c4 *  P(H(C)) + c5 *  P(G1(C)) + c6 * P(G2(C)) + c7 * X + c8 * Y + c9 * B

- Model 2: predChroma = c0 * H(C) + c1 *  H(W) + c2 *  H(E) + c3 *  G1(C) + c4 *  G1(W) + c5 *  G1(E) + c6 *  P(H(C)) + c7 *  P(H(W)) + c8 *  P(H(E)) + c9 * X + c10 * B

- Model 3: predChroma = c0 * H(C) + c1 *  H(NE) + c2 *  H(SW) + c3 *  G3(C) + c4 *  G3(NE) + c5 *  G3(SW) + c6 *  P(H(C)) + c7 *  P(H(NE)) + c8 *  P(H(SW)) + c9 * Y + c10 * B

where H(·), G1(·), G2(·), G3(·) are various downsampling filters as indicated in Figure 1, C denotes the current chroma sample position, and N, S, W, E, NE, SW are the positions around C as indicated in Figure 2, ci are filter coefficients, P and B are nonlinear term and bias term, and X and Y are the horizontal and vertical locations of the center luma sample with respect to the top-left coordinates of the block. It is noted that Model 1 is a 1x1 prediction shape using current chroma sample only while the other models are one-directional prediction models using 3 chroma samples.

Figure 1. Various downsampling filters used in the proposed cross-component models

Figure 2. The positions of chroma samples

# Experimental results

The proposed method was tested on top of ECM-8.0 under the common test conditions [2]. Table 1 shows the performance results.

Table 1. Performance results

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -1.22% | -1.12% | 101% | 101% | 0.02% | -0.60% | -0.50% | 101% | 99% |
| Class A2 | -0.08% | -1.20% | -1.34% | 102% | 100% | -0.04% | -0.64% | -0.40% | 100% | 98% |
| Class B | -0.06% | -1.22% | -0.97% | 100% | 99% | 0.00% | -0.82% | -0.77% | 101% | 99% |
| Class C | -0.06% | -0.63% | -0.64% | 101% | 100% | -0.03% | -0.15% | -0.29% | 100% | 99% |
| Class E | -0.03% | -1.00% | -0.49% | 99% | 100% |  |  |  |  |  |
| Overall | -0.06% | -1.05% | -0.90% | 100% | 100% | -0.01% | -0.56% | -0.51% | 101% | 99% |
| Class D | -0.05% | -0.46% | -0.55% | 101% | 99% | -0.02% | -0.29% | -0.59% | 95% | 95% |
| Class F | -0.07% | -0.60% | -1.04% | 101% | 100% | -0.14% | -0.85% | -0.79% | 100% | 99% |
| Class TGM | -0.09% | -0.26% | -0.28% | 101% | 101% | -0.05% | -0.15% | -0.12% | 99% | 97% |

# Conclusion

This contribution proposes to apply multiple downsampling filters to a group of reconstructed luma samples in CCCM. The results show that CCCM can be further improved, and it is suggested to adopt the proposed method into next ECM.

# References

- M. Coban, F. Le Léannec, R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 8 (ECM 8),” JVET-AC2025, Apr. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).