| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0203-v2 |
| --- | --- |

| Title: | EE2-related: Reconstruction-Reordered TMP based on lineal filter model for screen content coding (RRTMP-LFM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) D. Ruiz Coll, O. Patino, P. Andrivon | Tel: Email: | duizcoll@ofinno.com, opatino@ofinno.com, pandrivon@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution proposes to extend the linear filter model for IntraTMP, presented in the EE-2-Test-1.15a and Test 1.15b (TMP-LFM), to the Reconstruction-Reordered TMP (RR-TMP) mode proposed in the EE2-Test.3.1. Instead of using the regular templates of the reference block and the current block, the horizontal or vertical flipped templates are used for the filter coefficients derivation. After that, the linear model is applied to the flipped reference block and used as a prediction block. When the IntraTMP mode is activated, the encoder signals using this mode with a flag. Simulation results on top of ECM-8.0 are reported below:

AI: Class F -0.12%, -0.49%, -0.17%, 102%, 102%; Class TGM -0.13%, -0.14%, -0.09%, 102%, 104%

RA: Class F -0.11%, -0.55%, -0.05%, 101%, 101%;    Class TGM -0.10%, -0.19%, -0.21%, 100%, 101%

# Introduction

The EE2-Test-3.1 [1] proposes extending the IntraTMP to the Reconstruction-Reordered TMP (RR-TMP) for screen content coding. RR-TMP searches in a constrained area of the regular IntraTMP search region for the current block's most similar horizontal or vertical flipped templates. The TM cost for the regular IntraTMP, horizontal-flipped, and vertical-flipped TMP is computed, and the location in the search region with less TMP cost is selected as the prediction block. Consequently, no signaling is required to indicate the use of horizontal or vertical flipped modes in the IntraTMP.

The RR-TMP BV is also introduced in the IBC Merge/AMVP list to be used for the following current block as a spatial neighbor candidate, and it is handled according to its horizontal or vertical flip mode.

EE-2-Test-1.15a [2] and EE-2-Test-1.15b [3] propose to apply a linear filter model of 6-tap to the prediction of Intra TMP (Intra TMP-FLM). The linear filter consists of 5 spatial luma samples and a bias term. The coefficients of the filter model are derived for each prediction block using the regression-based minimized MSE on samples between the reference regular templates and current regular templates. If the linear filter model is applied to the reference block in IntraTMP, a flag is signaled at the CU level to the decoder.

# Proposal

This contribution proposes to extend the linear filter model for IntraTMP (TMP-LFM), presented in the EE-2-Test-1.15a and Test 1.15b (TMP-LFM), to the Reconstruction-Reordered TMP (RR-TMP) mode proposed in the EE2-Test.3.1 for screen content coding. This new mode is called Reconstruction-Reordered TMP using Linear Filter Mode (RRTMP-LFM).

In addition to the regular top and left template matching used by the IntraTMP mode, RRTMP also searches for the best match of the current block's horizontal and vertical flipped templates. Therefore, the reference block uses the top and right templates for the RR-TMP horizontal flipping mode and the left and bottom templates for the vertical flipping mode.

In TMP-LFM, the reconstructed luma samples over the regular templates of the reference block and the regular template of the current block are used to derivate the filter coefficients during the training stage. Coefficients parameters are derived using the same regression-based MSE minimization technique utilized by other tools, such as CCCM in the ECM. TMP-LFM uses a 6-tap filter which takes spatial samples from the reconstructed reference block and a middle value (b) of the operating bit depth.

The input to the spatial 5-tap component of the filter consists of a center (C) luma sample and its above/north (N), below/south (S), left/west (W), and right/east (E) neighbors. Fig. 1.a depicts an example of the coefficients derivation for the luma sample B3 in the regular reference template using the colocated B3' luma sample in the regular template of the current block as a target, where k is the coefficient for the middle value term:

B3'=  N*B2 +W*A3 + C*B3 + E*C3 +S*B4 +  k*b

- TMP-LFM

- RRTMP-LFM

Figure 1. Coefficient model derivation for (a) TMP-LFM and (b) Horizontal flip RRTMP-LFM.

In RRTMP-LFM, the coefficient derivation is computed using the same reference flipped or non-flipped templates selected by the RRTMP. Fig. 1.b depicts a horizontal flip RRTMP-LFM example. In this case, the coefficients derivation for the luma sample F3 in the reference block template uses the colocated K3' luma sample in the horizontally flipped template of the current block as a target:

K3'=  N*F2 +W*E3 + C*F3 + E*G3 +S*F4 +  k*b

In the same way as TMP-LFM, the prediction block is calculated as a convolution between the filter coefficients and the reference block's luma samples. If RRTMP determines a flip mode instead of the regular non-flip mode, the prediction block is conveniently horizontally or vertically flipped. The intra-prediction mode is obtained to determine the MTS and LFNST transform used to encode the residual block by applying the DIMD tools on the filtered and flipped reference block.

The RRTMP-LFM mode is signaled at the CU level with a dedicated flag combined if the intraTMP flag is active. RRTMP-LFM flag is inferred to be zero if the intraTMP flag is 0.

# Experimental Results

The proposed method is tested on top of ECM-8.0 software [4] under JVET ECM common test conditions [5] in AI and RA configurations. The following results were obtained:

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.12% | -0.49% | -0.17% | 102% | 102% |
| Class TGM | -0.13% | -0.14% | -0.09% | 102% | 104% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.11% | -0.55% | -0.05% | 101% | 101% |
| Class TGM | -0.10% | -0.19% | -0.21% | 100% | 101% |

- References

- Damian Ruiz Coll, Oscar Patino, Pierre Andrivon, "EE2-3.1: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)," JVET-AD0171, April 2023.

- J.-Y. Huo, W.-H. Qiao, X. Hao, Z.-Y. Zhang, H.-Q. Du, Y.-Z. Ma, F.-Z. Yang, "EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model," JVET-AD0112, April 2023.

- R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema, " EE2-1.15b: Filtered template matching based intra prediction," JVET-AD0099, April 2023.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).