| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0206 |
| --- | --- |

| Title: | AHG 12: CABAC Initialization for GDR Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes that CABAC initialization shall be invoked for GDR pictures, even though inter pictures can inherit CABAC states from previously coded inter pictures in ECM. Otherwise, decoding cannot start at GDR pictures without leaks or mismatch. The simulation results demonstrate that the proposal performs better than ECM8.0 GDR with TempCabacInit set to 0 by -0.43% for class B, -0.53% for class C, -0.82% for class D, -1.25% for class E and -3.02% for class F, respectively.

# Introduction

In VVC [1], CABAC initialization is invoked for each input picture, regardless of intra or inter picture, as shown in Fig. 1.

Fig. 1 CABAC initialization is invoked for each picture regardless of intra or inter picture.

In order to improve the performance, ECM allows an inter picture to inherit CABAC states from previously coded inter picture with the same picture type, temporal ID and quantization parameter, but still invokes CABAC initialization for intra pictures [2,3], as shown in Fig. 2.

Fig. 2.  Inter pictures are allowed to inherit CABAC states from previously coded inter pictures.

Inheritance of CABAC states from previously coded pictures may cause problems for GDR. GDR pictures are inter pictures. So, they, as inter pictures, will inherit CABAC states from previously coded inter pictures with the same picture type, temporal ID and quantization parameter.

According to the current ECM design, decoding can start at an IRAP picture or a GDR picture. If decoding starts at a GDR picture, decoder may have no prior-knowledge about the history involving in coding of the GDR picture. CABAC states inherited by the GDR picture at encoder may not be available at decoder. Encoder and decoder, therefore, may not be sync and they may not produce the same refreshed areas in the reconstructed GDR pictures that will be used as reference for the associated recovering pictures, which will result in leaks or mismatch at the recovery point picture.

# Proposed Solution

With the current GDR design in ECM, CABAC initialization may have to be invoked for all the pictures to avoid leaks or mismatch, if decoding will start at GDR pictures. To maintain the coding gain from CABAC state inheritance and also to be able to start decoding at GDR pictures, it is proposed that

- CABAC initialization shall be invoked for GDR pictures or GDR pictures shall not inherit CABAC states from previously coded inter pictures,

- But, other inter pictures can still inherit CABAC states from previously coded inter pictures (including previously coded GDR pictures).

Specifically, before coding of GDR pictures, CABAC state table shall be cleaned up and CABAC initialization shall be invoked at both encoder and decoder. After coding of GDR pictures, CABAC states of the coded GDR pictures may be stored and inherited by other inter pictures, except GDR pictures.

Fig. 3 illustrates the basic idea, where before coding of GDR picture of POC(n), CABAC initialization is invoked and after coding of GDR picture of POC(n), CABAC states of the GDR picture of POC(n) are stored and inherited by the following inter pictures.

Fig. 3. CABAC initialization still needs to be performed for GDR pictures.

# Experimental Results

Simulations were conducted on ECM8.0 with GdrEnabled=1 and TempCabacInit=0/1 over JVET test sequences using low-delay B configuration with the following setting,

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every second followed by 4 regular B pictures.

- GOP interval (distance between GDR picture and associated recovery point picture) is set to frame rate,

- Each inter picture refers up to 2 immediately preceding pictures in decoding order,

- The same QP is used for all pictures.

In the simulations, CABAC initialization is invoked for GDR pictures even with TempCabacInit=1 so that decoding can start at any GDR pictures.

Table 1 shows the comparison results. The BD rates of the proposal over ECM8.0 with GdrEnabled=1 and  TempCabacInit=0 are -0.43% for class B, -0.53% for class C, -0.82% for class D, -1.25% for class E and -3.02% for class F, respectively.

Table 1. Comparison Results of the proposal over ECM8.0 GDR with TempCabacInit=0

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | -0.43% | 0.68% | 1.22% | 102% | 103% |
| Class C | -0.53% | -0.39% | -0.30% | 102% | 104% |
| Class E | -0.82% | 3.34% | 2.38% | 99% | 99% |
| Overall | -0.56% | 0.99% | 1.01% | 101% | 102% |
| Class D | -1.25% | 1.14% | 0.76% | 103% | 103% |
| Class F | -3.02% | -2.08% | -2.04% | 99% | 98% |

# Conclusions

With the CABAC states table inheritance in the current ECM, decoding cannot start at GDR pictures without leaks or mismatch. This contribution proposes CABAC initialization at GDR pictures even if inter pictures are allowed to inherit CABAC states from previously coded pictures. Simulation results demonstrates that the proposal performs better than ECM8.0 with GdrEnabled=1 and TempCabacInit=0 by an overall coding gain of -0.56% for classes B, C and E. It is proposed to adopt CABAC initialization for GDR pictures in ECM.

# References

- B. Bross, J. Chen, S. Liu, Y-K Wang, “Versatile Video Coding”, JVET-O2001-vE, June 2020.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- Vadim Seregin, Jie Dong, Nan Hu and Marta Karczewicz, “AHG 12: CABAC initialization from previous inter slice”, VET-Y0181, Jan. 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).