| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0207-v1 |
| --- | --- |

| Title: | EE1-Related: Additional results for EE1-1.3.1 and EE1-1.3.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yun Li Samuel Eadie Dmytro Rusanovskyy Marta Karczewicz | Email: | yli30@qti.qualcomm.com seadie@qti.qualcomm.com dmytror@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Input document JVET-AD0205 reports results for EE1-1.3, which proposed improvements of the NN-based in-loop filter from JVET-AC0155. This contribution presents a training strategy refinement for the architecture of EE1-1.3 and reports achieved coding gain improvement without changes to SW or network architecture.

Compared to the EE1-1.3.1, proposed change provides:

Int16: {-XX%, -XX%, -XX%} and {-0.30%, -1.2%, -0.92%} for RA and AI, respectively.

Compared to the EE1-1.3.5.b, proposed change provides:

Int16: {-XX%, -XX%, -XX%} and {-XX%, -XX%, -XX%} for RA and AI, respectively.

Compared to NNVC-4.0 anchor (NN tools OFF), tests 1.3.1 and 1.3.5b with refined strategy provide:

Test 1.3.1 complexity, kMAC/pxl: 605.9 (478.76 frame-wise)

Int16: {-XX%, -XX%, -XX%} and {-8.11%, -21.01%, -21.95%} for RA and AI, respectively.

Test 1.3.5.b complexity, kMAC: 439.1 (346.94 frame-wise)

Int16: {-XX%, -XX%, -XX%} and {-XX%, -XX%, -XX%} for RA and AI, respectively.

Test 1.3.5b with refined strategy outperforms FilterSet1 by {-XX, XX, XX%} and {-XX%, -XX%, -XX%} with 39% lower computational complexity.

Simulation results presented here demonstrate that the architecture of 1.3.5b has noticeable potential for improvements and possesses a good performance/complexity trade off; it is proposed to adopt the filter of 1.3.5.b to NNVC SW.

# Introduction

EE1-1.3 [1] studies simplifications of the Neural Network In Loop Filter (NN-ILF) network architecture from JVET-AC0155 [2] in order to improve the performance-computational complexity trade-off. The baseline architecture (EE1-1.3.1, originally from JVET-AC0155) is shown in Figure 1. The decomposition of expensive 3x3 convolutions was studied in EE1-1.3.5 to reduce the computational complexity. The resultant network architecture (Figure 2) reduces the computational complexity by 28%.

Despite the fundamental change in network architecture in EE1-1.3.5, EE1-1.3 standardised the training hyperparameters (from JVET-AC0155) across all sub-tests. This proved sub-optimal, in particular for model quantisation. As such, this contribution improves the training process for EE1-1.3.5b, and based on the findings, retrains the baseline model EE1-1.3.1 as well.

Figure 1: Network architecture of EE1-1.3.1

Figure 2: Network Architecture of EE1-1.3.5b

# Network Information

The network information in the training and inference stages are outlined in Table 1 and Table 2, respectively, in accordance with the NN CTC [3].

Table 1: Network Information in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 120 |
|  | Batch size: | 64 |
|  | Training time: | ~120h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Table 2: Network Information in Inference Stage

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
| Sub-test |  | 1.3.1 | 1.3.5b |
| HW environment: |  |  |  |
| GPU Type |  | N/A |  |
| Framework: |  | SADL |  |
| Number of GPUs per Task |  | 0 |  |
| Total Parameter Number (M) |  | 6.008 | 4.274 |
| Parameter Precision (Bits) |  | fp32, int16 |  |
| Memory Parameter (MB) |  |  |  |
| kMAC/pxl | Block | 605.9 | 439.1 |
|  | Frame | 478.76 | 346.94 |
| Total Conv. Layers per Model |  | 81 | 103 |
| Total FC Layers |  | 0 |  |
| Total Memory (MB) | Float | 22.76 | 16.42 |
|  | Int16 | 11.43 | 8.28 |
| Batch size: |  | 1 |  |
| Patch size |  | 128x128, 256x256 |  |
| Changes to network configuration or weights required to generate rate points |  |  |  |
| Peak Memory Usage |  |  |  |
| Other information: |  |  |  |

# Modification

Analysis of the validation L2 loss during the training of EE1-1.3.5b (Figure 3) suggests that the original learning rate schedule (red) utilized in the training is sub-optimal for this architecture. Based on this observation, a new schedule for loss functions switching was proposed and demonstrated better training results, shown in blue curve of Figure 3.

Figure 3: Example L2 validation loss showing training strategy refinement (from red to blue).

# Simulation results

Here, we present a summary of the simulation results for the refined training strategy for the models from EE1-1.3.1 and EE1-1.3.5b over NNVC-4.0, FS1, the old training strategy from EE1-1.3, and the floating-point results using the refined training strategy.

## Performance of refined training strategy over NNVC-4.0

The BD-rate savings of the refined training strategy over NNVC-4.0 is reported in Table 3 and Table 4 for the 1.3.1 and 1.3.5b models, respectively.

Table 3: Proposed method BD-rate savings for 1.3.1 (Int16) with refined training strategy over NNVC-4.0

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.1-Rel_int16 Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | -7.05% | -19.76% | -20.98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | -7.10% | -21.57% | -20.85% |
| Class B | -10.07% | -26.24% | -26.51% | -7.44% | -22.38% | -23.07% |
| Class C | -11.05% | -24.56% | -25.90% | -8.47% | -18.73% | -21.17% |
| Class E |  |  |  | -10.82% | -22.49% | -23.17% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -8.11% | -21.01% | -21.95% |
| Class D | -12.83% | -26.78% | -28.37% | -8.21% | -17.15% | -20.31% |
| Class F | -6.22% | -17.59% | -16.73% | -6.56% | -18.18% | -17.43% |

Table 4: Proposed method BD-rate savings for 1.3.5b(Int16) with refined training strategy over NNVC-4.0

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.5b-Rel_int16 Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | -7.25% | -22.47% | -23.13% |
| Class C | -10.78% | -25.37% | -25.59% | -8.22% | -19.48% | -22.25% |
| Class E |  |  |  | -10.61% | -22.70% | -23.85% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -12.67% | -27.01% | -28.79% | -7.97% | -18.07% | -21.93% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Performance of refined training strategy over FS1

The BD-rate savings of the refined training strategy over Filter Set 1 is reported in Table 5 and Table 6 for the 1.3.1 and 1.3.5b models, respectively.

Table 5: Proposed method Int16 BD-rate savings for 1.3.1 with refined training strategy over FS1

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.1-Rel_int16 Over FS1 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | -0.54% | -4.37% | -3.65% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | -0.42% | -2.96% | -6.92% |
| Class B | -1.07% | -5.46% | -4.90% | -0.79% | -5.71% | -2.32% |
| Class C | -0.69% | -2.22% | -2.47% | -1.13% | 0.56% | 1.15% |
| Class E |  |  |  | -0.89% | -3.30% | -1.21% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.78% | -3.23% | -2.35% |
| Class D | -0.06% | -3.72% | -1.79% | -0.96% | 2.21% | 3.32% |
| Class F | -0.79% | -3.95% | -4.98% | -1.47% | -3.43% | -3.96% |

Table 6: Proposed method Int16 BD-rate savings for 1.3.5b with refined training strategy over FS1

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.5b-Rel_int16 Over FS1 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.59% | -5.92% | -2.44% |
| Class C | -0.40% | -3.27% | -2.10% | -0.86% | -0.63% | -0.49% |
| Class E |  |  |  | -0.65% | -3.75% | -2.30% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.12% | -4.04% | -2.34% | -0.70% | 1.01% | 1.08% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Performance of refined training strategy over EE1-1.3

The BD-rate savings of the refined training strategy over the original training strategy is reported in Table 7 and Table 8 for the 1.3.1 and 1.3.5b models, respectively. This substantial improvement for 1.3.5b is largely due to the problems with model quantisation with the previous training strategy.

Table 7: Int16 BD-rate savings for 1.3.1 with refined training strategy over EE1-1.3.1

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.1-Rel_int16 Over 1.3.1_int16 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | -0.20% | -1.40% | -1.09% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | -0.20% | -0.79% | -2.21% |
| Class B | -0.34% | -1.78% | -1.96% | -0.28% | -2.06% | -1.46% |
| Class C | -0.41% | 0.05% | -0.96% | -0.39% | -0.20% | 0.69% |
| Class E |  |  |  | -0.39% | -1.13% | -0.69% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.30% | -1.17% | -0.92% |
| Class D | -0.35% | -1.03% | -0.39% | -0.34% | 0.17% | 0.99% |
| Class F | -0.26% | -0.23% | 0.96% | -0.32% | 0.43% | 0.59% |

Table 8: Int16 BD-rate savings for 1.3.5b with refined training strategy over EE1-1.3.5b

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.5b-Rel_int16 Over 1.3.5b_int16 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | -0.55% | -0.74% | -0.64% |
| Class C | -0.85% | -1.31% | -0.92% | -0.58% | -0.74% | -0.65% |
| Class E |  |  |  | -0.84% | -0.88% | -1.19% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -1.09% | -1.04% | -1.25% | -0.45% | -0.44% | -0.95% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

## Quantisation error of refined training strategy

The BD-rate loss from quantisation of the refined training strategy is reported in Table 9 and Table 10 for the 1.3.1 and 1.3.5b models, respectively.

Table 9: BD-rate loss from quantisation of 1.3.1 with refined training strategy

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.1-Rel_int16 Over 1.3.1-Rel |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0.02% | 1.09% | 1.39% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0.10% | 1.82% | 1.91% |
| Class B | 0.04% | 0.45% | 0.33% | 0.07% | -1.16% | -0.62% |
| Class C | -0.04% | 0.27% | 0.31% | 0.03% | 1.98% | 2.64% |
| Class E |  |  |  | 0.07% | 1.43% | 1.66% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 0.06% | 0.84% | 1.24% |
| Class D | 0.03% | 0.22% | 0.50% | 0.03% | 2.29% | 3.15% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 0.07% | 2.46% | 1.82% |

Table 10: BD-rate loss from quantisation of 1.3.5b with refined training strategy

|  | Random access Main10 |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate 1.3.5b-Rel_int16 Over 1.3.5b-Rel |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0.10% | -0.75% | -0.10% |
| Class C | 0.11% | 0.29% | 0.24% | 0.09% | 0.40% | 0.52% |
| Class E |  |  |  | 0.11% | 0.34% | 0.34% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.07% | 0.04% | 0.17% | 0.10% | 0.59% | 0.55% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

This contribution provides additional results for tests EE1-1.3.1 and EE1-1.3.5.b. This contribution presents a refinement of the training strategy for the architectures from EE1-1.3 and reports achieved coding gain improvement without changes to SW or network architecture. This proposal achieves a better coding performance-complexity operating point for NN-based In-Loop filtering, comparing to benchmark methods. Specifically, the refined 1.3.5b demonstrates an improvement over Filter Set 1 of {XX%, XX%, XX%} and { XX%, XX%, XX%} for RA and AI, respectively, with a 39% lower computational complexity. It is therefore proposed to adopt the refined EE1-1.3.5 to the reference NNVC SW.

# References

- EE1-1.3: Reduced complexity CNN-based in-loop filtering, JVET-AD0205, S. Eadie, Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

- EE1-1.9: Reduced complexity CNN-based in-loop filtering, JVET-AC0155, S. Eadie, M. Coban, M. Karczewicz (Qualcomm)

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).