| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0208-v2 |
| --- | --- |

| Title: | EE2-1.8/1.9: IBC adaptation for coding of natural content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, 2Bappaditya Ray, Muhammed Coban, Hongtao Wang, Vadim Seregin, Marta Karczewicz (Qualcomm) 3Wei Chen, 4Xiaoyu Xiu, Changyue Ma, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang (Kwai) | Email: | 1chunchic@qti.qualcomm.com 2bray@qti.qualcomm.com 3chenwei06@kwai.com 4xiaoyuxiu@kwai.com |
| Source: | Qualcomm and Kwai Inc. |  |  |

# Abstract

This contribution reports the testing results of the Test-1.8a method (IBC adaptation for coding of natural contents) and Test-1.8c method (a combination of Test-1.8b and Test-1.9a to support fractional-pel block vector for Intra Block Copy (IBC)) in the context of Exploration Experiments 2 (JVET-AC2024). The first method proposes a IBC tool set and encoder optimization to adapt IBC for the coding of nature contents. The latter extends the representation of IBC block vector to fractional-pel resolution. The experimental results on top of ECM-8.0 are reported, as follows:

Test-1.8a:

AI(Y/U/V): -0.59%/-0.76%/-0.74%, EncT 102%, DecT 100%

RA(Y/U/V): -0.20%/-0.23%/-0.16%, EncT 100%, DecT 99%

Test-1.8c (combination of Test-1.8b and Test-1.9a):

AI(Y/U/V): -0.99%/-1.37%/-1.32%, EncT 103%, DecT 100%

RA(Y/U/V): -0.32%/-0.46%/-0.44%, EncT 100%, DecT 99% (w/ 4 missing rate points replaced by using anchor’s)

# Introduction

In 29th JVET meeting, the operability of adapting IBC to coding of natural contents was investigated in JVET-AC0161 [1] and JVET-AC0172 [2]. The technologies as proposed in the two proposals could be summarized in two aspects. The first aspect is high level tool control of various IBC coding tools and encoder optimization. The second aspect is introducing fractional pixel block vector for IBC. These aspects are studied in EE2 Test-1.8a and Test-1.8c, respectively.

# Test description

## IBC adaptiation for coding of natural contents (Test-1.8a)

# This aspect involves high level tool control of various IBC coding tools and encoder optimization. The changes as described below are only applied for the coding of natural content, thus the performance for screen contents is unchanged.

- High level tool control

- IBC merge modes are disabled for natural content. An SPS level flag is implemented to disable the associated CU level signaling. So, only IBC AMVP modes is activated when it is indicated explicitly by the sps flag.

- RR-IBC, TM-IBC and IBC-CIIP are disabled for natural content. For RR-IBC and TM-IBC, corresponding SPS flags are implemented (as ECM-8.0 does not have SPS control for these tools).

- IBC is applied to intra slices for natural content. This is indicated by high-level syntax to remove CU level signaling of IBC flag. For screen contents, IBC is applied to all slices.

- Encoder optimization

- At encoder, IBC block vector search is optimized for natural content, and the RDO process may be skipped for an IBC AMVP mode if its SAD cost is much worse than the lowest SAD cost of all Intra modes.

- IBC AMVP modes may not be evaluated when the best Intra mode has less than 3 nonzero coefficients.

- In ECM-8.0 encoder [3], some partitioning depth in an inter slice is skipped depending on the POC distance between the current picture and its nearest reference picture. When IBC is enabled from SPS level, ECM-8.0 always sets this POC distance equal to 0, which does not align with the configuration used in CTC for Random Access and Low Delay. In this test, for inter slices, the true POC distance is used instead of setting to 0.

## Fraction-pel extension on IBC (Test-1.8c)

In aspect 2, the representation of IBC block vectors is extended to fractional-pel resolution. An interpolation filter is thus required to derive the prediction samples located at a non-integer phase in the reconstructed area of the current frame. The following summarizes the proposed method on top of Test-1.8a.

- The option of block vector resolutions is extended to include quarter-pel resolution in additional to full-pel and 4-pel ones in ECM-8.0. Similar to inter AMVR syntax, the first bin of AMVR syntax is signalled to indicate whehter BV is in quarter-pel resolution, while the second bin may be signalled to switch between full-pel and 4-pel resolutions.

- The interpolation filters applied to the luma and chroma components of IBC blocks are the VVC style 8-tap luma filter and the same chroma filter as used in motion compensation, resectively. An exception is that a 2-tap bilinear interpolation filter is applied to generating template prediction blocks, when needed in IBC-realated coding tools.

- Reference sample padding may be needed when some of them are not available or located outside valid IBC reference area in the current frame. When needed, it performs in horizontal direction first and then vertical direction.

# Experimental results

The proposed method was implemented on top of the ECM-8.0 software [3] and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AC2024). Two pieces of SW are provided on JVET git repository (https://vcgit.hhi.fraunhofer.de/ecm/jvet-ac-ee2/ECM) at branches TEST_1.8 and TEST_1.8c for conducting the two tests, respectively. The testing results of Test-1.8a and Test-1.8c are summarized below.

- Performance results of Test-1.8a method relative to ECM-8.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.02% | 0.00% | 102% | 101% |
| Class A2 | -0.97% | -1.39% | -1.37% | 103% | 99% |
| Class B | -0.69% | -0.86% | -0.85% | 102% | 100% |
| Class C | -0.23% | -0.40% | -0.36% | 102% | 100% |
| Class E | -1.07% | -1.19% | -1.16% | 103% | 99% |
| Overall | -0.59% | -0.76% | -0.74% | 102% | 100% |
| Class D | -0.44% | -0.86% | -0.75% | 102% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | -0.09% | 100% | 99% |
| Class A2 | -0.39% | -0.50% | -0.18% | 100% | 100% |
| Class B | -0.33% | -0.30% | -0.30% | 101% | 99% |
| Class C | -0.05% | -0.09% | -0.03% | 101% | 100% |
| Class E | x | x | x | x | x |
| Overall | -0.20% | -0.23% | -0.16% | 100% | 99% |
| Class D | -0.09% | 0.25% | -0.57% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

- Performance results of Test-1.8c method relative to ECM-8.0 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.13% | -0.03% | 102% | 100% |
| Class A2 | -1.41% | -2.05% | -2.03% | 103% | 99% |
| Class B | -1.01% | -1.52% | -1.49% | 103% | 100% |
| Class C | -0.45% | -0.70% | -0.62% | 104% | 100% |
| Class E | -2.17% | -2.58% | -2.57% | 105% | 99% |
| Overall | -0.99% | -1.37% | -1.32% | 103% | 100% |
| Class D | -0.67% | -1.10% | -1.01% | 103% | 100% |
| Class F | -1.82% | -2.00% | -1.84% | 103% | 102% |
| Class TGM | -0.76% | -0.47% | -0.45% | 102% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1* | -0.02% | 0.01% | 0.04% | 99% | 99% |
| Class A2* | -0.59% | -0.84% | -0.54% | 100% | 100% |
| Class B | -0.50% | -0.74% | -0.91% | 100% | 100% |
| Class C | -0.13% | -0.17% | -0.13% | 101% | 99% |
| Class E | x | x | x | x | x |
| Overall* | -0.32% | -0.46% | -0.44% | 100% | 99% |
| Class D | -0.07% | -0.15% | -0.18% | 102% | 100% |
| Class F | -0.74% | -1.23% | -1.00% | 99% | 100% |
| Class TGM | -1.37% | -1.86% | -1.57% | 98% | 99% |
| * Four missing rate points (1 in Tango, 1 in DaylightRoad and 2 in ParkRunning) are replaced by using anchor results. |  |  |  |  |  |

# References

- B.Ray, et al., “Non-EE2: IBC adaptation for coding of natural content”, JVET-AC0161, Jan 2023.

- W. Chen et al. “Non-EE2: IBC with fractional block vectors”, JVET-AC0172, Jan 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).