| vJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0209-v2 |
| --- | --- |

| Title: | EE2-2.3: SbTMVP with MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Roman Chernyak, Xin Zhao, Xiaozhong Xu, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A | Tel: Email: | lienfeichen@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of EE2-2.3. In EE2-2.3, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. The syntax element, MMVD index, is signaled to indicate the MMVD offset for the SbTMVP merge candidate, and this MMVD offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset. Different subblock-based motion field could be obtained by using the different DV offset at different MMVD offset position. In EE2-2.3, there are two tests, EE2-2.3a and EE2-2.3b. In EE2-2.3a, it is a simplest implementation of SbTMVP with MMVD by inserting all available MMVD candidate into the candidate list in spiral scanning ordering. The maximum candidate list size is 16. Alternative method, called EE2-23b, 40 candidates will be scanning and inserted in the candidate list if that candidate is available. Then, the subblock-based template-matching is used to reorder all available SbTMVP-MMVD candidates by using template-matching cost in ascending order and we keep 16 candidates with the smallest template-matching cost. The overall simulation results on top of ECM-8.0 reference software are reported as below.

In EE2-2.3a

- RA: -0.04%/-0.02%/-0.01% with 101% EncT and 100% DecT

- LB: -0.23%/-0.06%/-0.02% with 102% EncT and 101% DecT

In EE2-2.3b

- RA: -0.06%/-0.04%/-0.04% with 102% EncT and 102% DecT

- LB: -0.34%/-0.21%/-0.09% with 105% EncT and 112% DecT

# Introduction

In ECM 7[1], affine MMVD mode is proposed to improve the coding gain by using subblock-based motion information with affine parameter. Although, SbTMVP is one of the candidates in subblock-based merge list, MMVD is not supported for SbTMVP in current ECM design; And the derived SbTMVP from the neighboring block may not be the optimal subblock-based motion field in ECM 7[1]. In order to improve the SbTMVP, the SbTMVP with MMVD was proposed in JVET-AC0213[2] and was adopted in EE2-2.3 test.

# Proposed Method

In subblock MMVD merge list, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. The syntax element, MMVD index, is signaled to indicate the MMVD offset for the SbTMVP merge candidate, and this MMVD offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset. By using the different DV offset, different subblock-based motion field could be obtained. In EE2-2.3, the spiral scanning order is used to scan all MMVD candidate and the available candidate, which has the valid center motion vector, will be put into the candidate list in sequence. There are two tests in EE2-2.3. The first one, EE2-2.3a, is the simplest implementation of SbTMVP with MMVD. Regarding to the MMVD offset, the step size in test is designed as {4, 8, 12, 16, 20} and the unit in the step size is the integer pixel unit, and the number of directions is kept as 8 which is used in affine MMVD as well. Like the affine MMVD, the total number of the SbTMVP-MMVD candidates is less than or equal to 16. Here, the spiral scanning order is applied and up to 16 available candidates will be put into the candidate list. Alternative test, EE2-2.3b, all 40 MMVD candidates will be scanned in spiral order and be put into the candidate list if it is an available candidate. Then, the subblock-based template-matching is used to reorder all SbTMVP-MMVD candidates by using template-matching cost in ascending order and up to 16 candidates with the smallest template-matching cost will be kept. In order to improve the coding gain and runtime, some aspects are adopted in EE2-2.3, the detailed is listed as below.

- Picture level flag of SbTMVP-MMVD mode is used in RA case for both of EE2-2.3a and EE2-2.3b. This flag is used to indicate whether the SbTMVP-MMVD is enabled or not when both of the SPS flag of SbTMVP-MMVD and the SPS control flag in picture header for SbTMVP-MMVD are true. The picture level enabling and disabling is determined based on the temporal ID and the SbTMVP-MMVD is only enable at highest two temporal layers for RA case in EE2-2.3.

- The constraint of the size of collocated position is extended from one CTU row to three CTU rows. This extension is applied for SbTMVP-MMVD only, and there is no size extension for TMVP and SbTMVP. This changing is adopted in EE2-2.3b only.

# Simulation Results

The proposed EE2-2.3 is implemented on top of ECM-8.0 reference software[3] and tested by following the common test conditions (CTC) as described in [4].

## Test 2.3a

Table 1 shows the overall performance of EE2-2.3a for both of RA and LDB cases on top of ECM-8.0[3].

Table 1: simulation results on EE2-2.3a on top of ECM-8.0

| Test 2.3a | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | 0.00% | 101% | 100% |
| Class A2 | -0.03% | -0.02% | -0.03% | 101% | 99% |
| Class B | -0.02% | -0.02% | -0.02% | 101% | 100% |
| Class C | -0.09% | -0.03% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.02% | -0.01% | 101% | 100% |
| Class D | -0.01% | 0.01% | 0.02% | 101% | 101% |
| Class F | 0.00% | -0.02% | -0.01% | 101% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test 2.3a | Low delay B Main10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.14% | 0.04% | 103% | 101% |
| Class C | -0.24% | 0.03% | 0.05% | 102% | 101% |
| Class E | -0.50% | -0.03% | -0.22% | 101% | 102% |
| Overall | -0.23% | -0.06% | -0.02% | 102% | 101% |
| Class D | -0.19% | -0.92% | -0.40% | 102% | 102% |
| Class F | -0.01% | -0.02% | 0.48% | 103% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test 2.3b

Table 2 shows the overall performance of EE2-2.3b for both of RA and LDB on top of ECM-8.0[3].

Table 2: simulation results on EE2-2.3b on top of ECM-8.0

| Test 2.3b | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.07% | 102% | 102% |
| Class A2 | -0.05% | -0.02% | -0.06% | 102% | 101% |
| Class B | -0.03% | -0.03% | -0.03% | 102% | 101% |
| Class C | -0.11% | -0.03% | -0.02% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.04% | -0.04% | 102% | 102% |
| Class D | -0.03% | 0.02% | 0.00% | 102% | 102% |
| Class F | 0.00% | -0.04% | -0.02% | 102% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Test 2.3b | Low delay B Main10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.40% | -0.26% | 106% | 108% |
| Class C | -0.26% | -0.07% | 0.15% | 104% | 111% |
| Class E | -0.73% | -0.09% | -0.14% | 106% | 119% |
| Overall | -0.34% | -0.21% | -0.09% | 105% | 112% |
| Class D | -0.31% | -0.83% | 0.01% | 103% | 112% |
| Class F | 0.06% | 0.11% | 0.94% | 104% | 108% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

In EE2-2.3, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. Two tests are tested in EE2-2.3, EE2-2.3a and EE2-2.3b. On top of EE2-2.3a, EE2-2.3b utilizes the subblock-based template-matching to reorder all SbTMVP-MMVD candidates by using template-matching cost in ascending order. According to our simulation results, our method could achieve -0.23%/-0.06%/-0.02% coding gain for Y/U/V channels in LB case and achieve -0.34%/-0.21%/-0.09% coding gain in LB case with candidate list reordering by using subblock-based template-matching. We recommend adopting EE2-2.3 in next ECM.

# Reference

- M. Coban, F. L. Léannec. R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 7 (ECM 7)”, ISO/IEC JTC1/SC29 JVET-AB2024, Oct. 2022.

- L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, and S. Liu, “Non-EE2: SbTMVP with MMVD”, ISO/IEC JTC1/SC29 JVET-AC0213, Jan. 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29 JVET-Y2017, April 2022.

- X. Li, L.-F. Chen, Z. Deng, J. Gan, E. Francois, H.-J. Jhu, R.-L. Liao, and H. Wnag, “JVET AHG report: ECM tool assessment (AHG7)”, ISO/IEC JTC1/SC29 JVET-AD0007, April 2023.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).