| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0213-v1 |
| --- | --- |

| Title: | EE2-Test2.7: Improvements on local illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Ning Yan, Changyue Ma Hong-Jheng Jhu, C.-W. Kuo, Wei Chen, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Exploration Experiment 2 Test-2.7 for LIC improvements. There are two sub-tests in the EE test. In Test-2.7a, the existing LIC design is extended from uni-prediction to bi-prediction, where two sets of LIC parameters are derived and applied to L0 and L1 separately. Test-2.7b is on top of Test-2.7a, which additionally enables the OBMC for the LIC blocks and considers the LIC parameters when generating the OBMC prediction samples of one current block. The simulation results are summarized as

- Test-2.7a

- RA: -0.30% (Y), -0.06% (U), -0.07% (V), 103% (Enc), 101% (Dec)

- LB: -0.25% (Y), -0.10% (U), -0.09% (V), 102% (Enc), 101% (Dec)

- Test-2.7b

- RA: -0.36% (Y), -0.18% (U), -0.25% (V), 102% (Enc), 101% (Dec)

- LB:	 -0.37% (Y), -0.18% (U), -0.23% (V), 102% (Enc), 101% (Dec)

# Introduction

At JVET-AC meeting, JVET-AC0164 [1] was proposed to improve the efficiency of the LIC mode in the ECM and was included in Exploration Experiment 2 (EE2) as Test-2.7 [2] for further evaluation. There are two sub-tests in Test-2.7. In Test-2.7a, the existing LIC is extended from uni-prediction to bi-prediction, where two sets of LIC parameters (i.e., scale and offset) are separately derived and applied to L0 and L1 prediction samples that are then combined to form the final bi-prediction. Test-2.7b is on top of Test-2.7a. Additionally, in Test-2.7b, the OBMC is enabled for the inter blocks that apply the LIC. Besides, when one neighboring block is coded with the LIC, it is proposed to consider its LIC parameters when generating the corresponding prediction samples for the OBMC of one current block.

# Technical aspects

## Bi-predictive LIC

In the method, the LIC mode is extended to bi-predictive CUs. Specifically, two different linear models are applied to the two prediction blocks which are then combined to generate the bi-prediction samples of the current CU, i.e.,

P'x,y=1-ω∙p0'x,y+ω∙p1'x,y

and

p0'x,y=α0∙P0x,y+β0

p1'x,y=α1∙P1x,y+β1

where α0 and β0, and α1 and β1 indicate the scales and the offsets in L0 and L1, respectively; ω indicates the weight (as indicated by the CU-level BCW index) for the weighted combination of L0 and L1 predictions. The same derivation scheme of the LIC mode is reused and applied in one iterative manner to derive the L0 and L1 LIC parameters. Specifically, the method firstly derives the L0 parameters by minimizing difference between L0 template prediction T0 and the template T and the samples in T are updated by subtracting the corresponding samples in T0.  Then, the L1 parameters are calculated that minimizes the difference between L1 template prediction T1 and the updated template. Finally, the L0 parameter is refined again in the same way.

Following the current LIC design, one flag is signaled for AMVP bi-predicted CUs for the indication of the LIC mode while the flag is inherited for merge related inter CUs. Additionally, the LIC is disabled when decoder-side motion vector refinement (DMVR) (including multi-pass DMVR, adaptive DMVR and affine DMVR) and bi-directional optical flow (BDOF) is applied.

## OBMC with LIC

In the method, the OBMC is enabled for the inter blocks that are coded with the LIC mode. And, to reduce the complexity, the OBMC is only applied to the top and left CU boundaries while being always disabled for the boundaries of the internal sub-blocks of one LIC CU. Additionally, when one neighboring block is coded with the LIC, its LIC parameters are applied to generate the corresponding prediction samples for the OBMC of one current block.

# Simulation results

The proposed scheme was implemented on top of ECM-8.0 [3] and tested using the common test conditions as defined in [4]. Table 1 and Table 2 show the simulations results of Test-2.7a and Test-2.7b. Table 3 and Table 4 show complementary results of two tests when MR380 is applied to fix the encoder bug on setting the OBMC SPS flag. In Table 3 and Table 4, the fix of MR380 is applied to both anchor and the test. Because MR380 has no impact on natural content sequences and RA configuration, only the results of class F in LB configuration are shown.

Table 1 Results of Test-2.7a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.06% | -0.31% | 105% | 101% |
| Class A2 | -0.28% | 0.02% | -0.01% | 104% | 101% |
| Class B | -0.31% | -0.10% | -0.08% | 103% | 100% |
| Class C | -0.14% | -0.06% | 0.08% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.06% | -0.07% | 103% | 101% |
| Class D | -0.06% | 0.16% | 0.13% | 103% | 101% |
| Class F | -0.41% | -0.05% | -0.06% | 103% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.33% | -0.23% | 104% | 102% |
| Class C | -0.29% | -0.17% | 0.02% | 101% | 102% |
| Class E | -0.22% | 0.39% | -0.01% | 102% | 101% |
| Overall | -0.25% | -0.10% | -0.09% | 102% | 101% |
| Class D | 0.06% | -0.22% | -0.57% | 103% | 102% |
| Class F | -0.18% | -0.12% | 0.45% | 104% | 101% |

Table 2 Results of Test-2.7b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.57% | -0.31% | -0.45% | 102% | 100% |
| Class A2 | -0.31% | -0.18% | -0.18% | 103% | 100% |
| Class B | -0.38% | -0.08% | -0.24% | 103% | 101% |
| Class C | -0.20% | -0.22% | -0.15% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.18% | -0.25% | 102% | 101% |
| Class D | -0.15% | 0.08% | -0.01% | 102% | 101% |
| Class F | -0.31% | -0.31% | -0.24% | 103% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.40% | -0.17% | 104% | 101% |
| Class C | -0.36% | -0.04% | -0.44% | 101% | 102% |
| Class E | -0.34% | 0.01% | -0.05% | 102% | 100% |
| Overall | -0.37% | -0.18% | -0.23% | 102% | 101% |
| Class D | -0.15% | -0.20% | -0.58% | 103% | 102% |
| Class F | -0.26% | 0.08% | 0.11% | 103% | 100% |

Table 3 Results of Test-2.7a with MR380

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0-MR380 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.24% | -0.01% | 0.41% | 104% | 101% |

Table 4 Results of Test-2.7b with MR380

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0-MR380 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | 0.05% | 0.15% | 104% | 100% |

# Conclusion

The contribution presents the results of EE2 Test-2.7. Simulation results show that the tests provide promising coding performance improvements and it is recommended to adopt the proposed methods into ECM.

# Reference

- X. Xiu, N. Yan, H.-J. Jhu, W. Chen, C.-W. Kuo, C. Ma, X. Wang, “Non-EE2: Improvements on local illumination compensation in ECM7.0”, JVET-AC0164, Jan. 2023.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, Jan. 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).