| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0214-v1 |
| --- | --- |

| Title: | Non-EE2: On search region of intra template matching in ECM8.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Ning Yan, Changyue Ma Hong-Jheng Jhu, C.-W. Kuo, Wei Chen, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In ECM-8.0, intra template matching (intraTMP) is applied to predict one current block from the already reconstructed samples of the current picture. Currently, the search region of the intraTMP mode is composed of four different regions which contain the reconstructed sample from the top and left CTUs and partial reconstructed samples from the same CTU that the current block belongs to. This contribution proposes to include more reconstructed samples within the current CTU to further improve the intraTMP efficiency. Simulation results show that the proposed change reportedly provides BD-rate reduction of 0.19% under AI configuration with enc/dec runtime of 101%/102%. On top of EE2 Test-1.14c, its corresponding AI gain is 0.xx% with enc/dec runtime of xx%/xx%.

# Introduction

In ECM-8.0 [1], intra template matching (intraTMP) is applied as one intra mode which predicts one current block the already reconstructed samples in the current picture. As shown in Figure 1, the search region of the existing intraTMP is divided into four different regions R1, R2, R3 and R4 which contains the reconstructed samples from the top and left CTUs as well as partial reconstructed samples with the current CTU that are located above and left to the current block. In Figure 1, W and H are the width and height of the current coding block and the gray blocks describe the region of the top-left corners of all considered reference blocks in the reconstructed area.

Figure 1 The search region of the intraTMP in ECM8.0

Additionally, in EE2 Test-1.14c [2], it is proposed to extend the regions R3 and R4 to include more reconstructed samples that are located above and left to the current block. Additionally, the maximum horizontal and vertical search size, i.e., searchRangeWidth and searchRangeHeight, are set to max(5W, 64) and max(5H, 64) respectively. Figure 2 depicts the modified search region of the intraTMP in Test-1.14c.

Figure 2 The extended search region of the intraTMP in EE2 Test-1.14c

# Proposal

In Figure 1 and Figure 2, only the samples above and left to the current block inside the current CTU are included in the intraTMP search region. However, because of the quad/binary/ternary-tree partition structure used in the ECM, certain samples with positions above-right or below-left relative to the current block could be already reconstructed prior to the decoding of the block. Depending on the current block’s position, those samples could be closer to the current block (i.e., with higher correlation) and can potentially provide a better intraTMP prediction.

As shown in Figure 3, it is proposed to include two additional regions R5 and R6, which are positioned above-right or below-left to the current block, to further improve the coding efficiency of the intraTMP.

Figure 3 The proposed intraTMP search region

Additionally, in order to reduce the complexity, one constraint is applied to the intraTMP search in R5 and R6 such that the maximum horizontal distance of the reference candidate block cannot exceed a*W from the current block, respectively, where a is set to 5.

Since the samples in R5 and R6 may be decoded after the current block, in the proposed search regions, availability checking needs to be done to decide whether one reference block is available or not. In case there is any unavailable sample in one reference block, it is regarded as an invalid intraTMP candidate. On the other side, because all reference blocks are in rectangular shape and the coding order of child sub-partitions nodes under one parent partition node is always from left to right and top to bottom, it only needs to check the availability of the bottom-right corner for each reference block to decide whether it is available.

# Simulation results

The proposed method was implemented on top of ECM-8.0 [1] and tested using the common test conditions as define in [3]. Table 1 shows the BD-rate performance of the proposed method on top of ECM-8.0. Table 2 shows its corresponding BD-rate improvement on top of the software of EE2-Test1.14c.

Table 1 The BD-rate of the proposed method over ECM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.01% | -0.09% | 101% | 101% |
| Class A2 | -0.40% | -0.57% | -0.54% | 101% | 104% |
| Class B | -0.27% | -0.33% | -0.26% | 101% | 101% |
| Class C | -0.07% | -0.14% | -0.12% | 101% | 101% |
| Class E | -0.15% | -0.26% | -0.11% | 102% | 103% |
| Overall | -0.19% | -0.26% | -0.22% | 101% | 102% |
| Class D | -0.11% | -0.18% | -0.15% | 101% | 103% |
| Class F | -0.01% | 0.01% | 0.03% | 101% | 102% |

Table 2 The BD-rate of the proposed method over EE2-Test1.14c

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2-Test1.14c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.04% | -0.10% | 103% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.19% | -0.25% | -0.22% | 101% | 103% |
| Class C | -0.05% | -0.18% | 0.03% | 101% | 100% |
| Class E | -0.14% | -0.23% | -0.16% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.02% | 0.14% | -0.12% | 101% | 101% |
| Class F | 0.01% | 0.08% | -0.36% | 101% | 101% |

# Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-AC2024, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).