| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0216-v1 |
| --- | --- |

| Title: | Non-EE2: IBC with non-adjacent spatial candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to utilize non-adjacent spatial neighbors to improve the efficiency of IBC merge and AMVP modes. Specifically, several new candidates, which are derived from non-adjacent spatial blocks, are added in-between the adjacent spatial candidates and the HBVP candidates of the candidate lists of both IBC merge and IBC AMVP. The simulation results on top of ECM-8.0 are summarized as follows:

AI:

Class F : {-0.34%, -0.35%, -0.42%, 100%, 102%}, Class TGM : {-0.42%, -0.37 %, -0.35%, 100%, 106%};

RA:

Class F : {-0.26%, -0.54 %, -0.24%, 100%, 101%}, Class TGM : {-0.31%, -0.34%, -0.41%, 99%, 101%};

# Introduction

In ECM 8.0, only the block vectors (BVs) of adjacent spatial neighbors of one current block are used to generate the candidate list of IBC merge and IBC AMVP. On the other side, for regular inter, an improved candidate list, which also include several motion candidates from non-adjacent spatial neighbors, is applied to improve the efficiency of regular merge and AMVP modes. Similar to the regular motion field, there could be strong correlation between the BVs of the blocks that are not neighboring to each other. Therefore, in this contribution, it is proposed to also use non-adjacent spatial neighbors for the IBC mode.

# Proposal

In this contribution, several new candidates, which are obtained from the BVs of non-adjacent spatial neighboring blocks, are inserted into the candidate lists of IBC merge modes and IBC AMVP. The IBC merge modes include regular IBC merge, IBC template matching (TM), and IBC merge with block vector difference (MBVD).

As shown in Fig. 1, the similar grid pattern that is used for the non-adjacent spatial candidates of regular inter is used to fetch the BVs of non-adjacent spatial neighbors in the proposed scheme, where the distances between non-adjacent candidates and current block are also defined based on the width and height of current coding block. The BVs of the resulting non-adjacent candidates are inserted in-between the adjacent spatial candidates and the HBVP candidates for both IBC merge and IBC AMVP.

Figure 1. Non-adjacent spatial candidates for IBC mode.

# Experimental results

The proposed method was implemented on top EMC-8.0 [1] and tests were performed following ECM CTC [2]. The test results of the proposed method are summarized in the following table.

Table 1: Test result of IBC with non-adjacent spatial candidates

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | -0.35% | -0.42% | 100% | 102% |
| Class TGM | -0.42% | -0.37% | -0.35% | 100% | 106% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.54% | -0.24% | 100% | 101% |
| Class TGM | -0.31% | -0.34% | -0.41% | 99% | 101% |

# Conclusion

The contribution proposes to insert some non-adjacent spatial candidates between adjacent spatial candidates and HBVP candidates for IBC AMVP and merge candidate list. Simulation results show the effectiveness of the proposed method. It is recommended to study the proposed method in the next round of EE.

# References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).