| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0217-v1 |
| --- | --- |

| Title: | Non-EE2: Filtered Intra Block Copy (FIBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Ning Yan, Changyue Ma, Xianglin Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | { jhuhong-jheng, xiaoyuxiu, cheweikuo, chenwei06, yanning03, machangyue, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, filtered intra block copy (FIBC) mode is proposed where the prediction samples of the IBC are enhanced by applying a linear filter. The proposed filter consists of five spatial terms and one bias term. The filter coefficients are derived via the regression-based minimization of the difference between the template samples and their corresponding reference samples. Compared to ECM-8.0 anchors, simulation results are summarized as follows:

For class F sequences:

AI { -0.92%, -1.10%, -1.04%, xxx%, xxx% }, RA { -0.29%, -0.67%, -0.72%, xxx%, xxx% }

For TGM sequences:

AI { -0.17%, -0.19%, -0.15%, xxx%, xxx% }, RA { -0.15%, -0.13%, -0.18%, xxx%, xxx% }

# Introduction

In ECM-8.0 [1], intra block copy (IBC) is applied for screen content sequences which predict the samples of one current block from the reconstructed sample of its neighboring regions. In the existing IBC design, the reconstructed samples (as indicated by the corresponding block vector) are directly used as the prediction samples of the current block. This may not be optimal due to the fact that there may exist certain coding noises in the reconstructed samples.

In EE2 Test-15a [2] and Test-15b [3], a linear filter is applied to the prediction samples of intra template matching (intraTMP). The filter coefficients are derived from the neighboring reconstructed samples and their corresponding reference samples.

# Proposal

To further improve the prediction accuracy, filtered IBC (FIBC) is proposed which applies one linear filter to the prediction samples of the IBC. As shown in Fig. 1, the proposed filter consists of five spatial terms and a bias term. The five spatial terms consist of a center (C) position and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors.

predVal = α0∙C + α1∙N + α2∙S + α3∙W + α4∙E + α5∙β

Fig. 1 Filter shape.

where αi is the coefficient and β is the offset. Same to EE2 Test-15a [2] and Test-15b [3], up to 4 lines/columns of samples above and left to the current CU are applied to derive the filter coefficients. The filter coefficients are derived based on the minimization of the difference between the template samples and their corresponding reference samples via the same regression-based minimization technique in the ECM that is used by other tools such as CCCM [4].

For the signaling, one extra indication flag is introduced for the FIBC which is signaled after the IBC-LIC flag. Specifically, when the IBC-LIC flag is true, the flag is signaled and used to indicate whether the FIBC is applied to the current block or not.

# Simulation Results

The proposed method was implemented based on the ECM-8.0 [1] reference software. The performance was evaluated using the common test conditions as defined in [5].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.92% | -1.10% | -1.04% |  |  |
| Class TGM | -0.17% | -0.19% | -0.15% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.29% | -0.67% | -0.72% |  |  |
| Class TGM | -0.15% | -0.13% | -0.18% |  |  |

# Conclusions

This proposal proposes to improve the prediction efficiency of the IBC by filtering the IBC prediction samples based on an adaptive linear filter. The results show that the method provides promising coding gains with negligible encoding/decoding impacts. It is recommended to study the proposed method into EE.

# References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

- J. Huo, W. Qiao, X. Hao, Z. Zhang, H. Du, Y. Ma, F. Yang, “EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model”, JVET-AD0112, 30th Meeting, Antalya, TR, 21–28 April 2023.

- R. G. Youvalari, D. Bugdayci Sansli, P. Astola, J. Lainema, “EE2-1.15b: Filtered template matching based intra prediction”, JVET-AD0099, 30th Meeting, Antalya, TR, 21–28 April 2023.

- P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone, “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, 27th Meeting, by teleconference, 13–22 July 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).