| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, 21–28 April 2023 | Document: JVET-AD0221-v0 |
| --- | --- |

| Title: | EE2-4.1: Additional Fixed Filter for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In the current adaptive loop filter (ALF) design, online-trained filters contain 4 kinds of taps: spatial taps, reconstruction-before-DBF-based taps, residual-based taps and fixed-filter-output-based taps. In this contribution, an additional fixed filter is proposed and several additional-fixed-filter-output-based taps are introduced into online-trained filters to provide more texture information for ALF.

On top of ECM-8.0, simulation results of EE2-4.1 are reported as below:

AI: -0.07%, 0.01%, 0.01%, %, %.

RA: -0.07%, 0.03%, 0.02%, %, %.

LB: %, %, %, %, %.

Introduction

In ALF of ECM-8.0, online-trained filters consist of 4 kinds of filter taps: spatial taps, reconstruction-before-DBF based taps, residual based taps and fixed-filter-output based taps, as shown in Figure 1. The spatial taps (i.e., tap #0 ~ #19) located in a cross shape are followed by 3 reconstruction-before-DBF based taps (i.e., tap #26, #27, #30), 2 residual-based taps (i.e., tap #31 ~ #32), and the offline-filtered taps (i.e., tap #20 ~ #25, #28, #29), resulting in 33 taps in total.

Figure 1 The Currently Used Filter Shape in ALF.

Proposed Method

The proposed method introduces an additional fixed filter for ALF. Only one additional fixed filter with a shape of diamond 7x7 is introduced, the filter parameters are stored at both encoder and decoder. There is no classification for the newly added fixed filter.

An online filter of the proposed method is shown in Figure 2, where spatial taps (i.e., tap #0 ~ #19), reconstruction-before-DBF-based taps (i.e., tap #26, #27, #36), residual-based taps (i.e., #37 ~ #38) and fixed-filter-output-based taps (i.e., tap #20 ~ #25, #34, #35) are kept the same as the ECM-8.0, and several extended taps (i.e., tap #28 ~ #33, #39) are introduced into luma online-trained filters. The reconstruction before DBF is fed into the additional fixed filter to produce the filter outputs, then these filter outputs are used as input for newly extended taps.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without any filter shape switching.

Figure 2 The Newly Introduced Filter Shape for ALF.

Simulation Results

The proposed method was implemented on top of the ECM-8.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-8.0 are shown in TABLE 1.

TABLE 1 Performance of EE2-4.1

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | 0.01% |  |  | -0.09% | 0.01% | -0.04% |  |  |  |  |  |  |  |
| Class A2 | -0.07% | 0.03% | 0.03% |  |  | -0.12% | 0.01% | 0.05% |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.02% | 0.02% |  |  | -0.03% | 0.02% | 0.01% |  |  |  |  |  |  |  |
| Class C | -0.08% | 0.02% | 0.02% |  |  | -0.08% | 0.08% | 0.05% |  |  | -0.12% | 0.09% | -0.02% |  |  |
| Class E | -0.07% | -0.01% | -0.01% |  |  |  |  |  |  |  | -0.18% | 0.61% | 0.03% |  |  |
| Overall | -0.07% | 0.01% | 0.01% |  |  | -0.07% | 0.03% | 0.02% |  |  |  |  |  |  |  |
| Class D | -0.03% | 0.04% | 0.05% |  |  | -0.10% | 0.32% | 0.06% |  |  | -0.16% | -0.29% | -0.21% |  |  |
| Class F | -0.10% | 0.03% | 0.03% |  |  | -0.17% | -0.11% | 0.06% |  |  | 0.00% | -0.15% | 1.14% |  |  |

Conclusions

In this contribution, an additional fixed filter for ALF is proposed and several extended taps are introduced into online-trained filters. The proposed method can achieve promising coding performance with negligible encoding time and decoding time increase. It is recommended to adopt the proposed method into the next version of ECM.

Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).